********************************************************************

* **************************************************************** *

* *                                                              * *

* *                    UPDATE  NOTE                              * *

* *                                                              * *

* *                    OCT.  1 1988                              * *

* *                                                              * *

* *    Expanded Memory Manager version 4.0 EMS40.SYS released    * *

* *                                                              * *

* *                                                              * *

* **************************************************************** *

********************************************************************

The previously released Expanded Memory Manager, formerly named

INITAT.SYS, has been modified and renamed to EMS32.SYS, and   a

new manager, EMS40.SYS, is released.

The EMS32.SYS file on your diskette is a device driver conform-

ing to EMS Version 3.2, and the EMS40.sys file is a device dri-

ver conforming to EMS Version 4.0. You may install  either  the

EMS32.SYS  or  the  EMS40.SYS driver, depending on the applica-

tions programs you intend to run.

EMS40.sys  uses  the  same  optional  parameters as EMS32.SYS ,

except that it has two more optional  command  switches, /S and

/H.

Descriptions  and  instructions  on use of the new driver are

given below.

===============================================================

How to use EMS40.SYS

To use EMS40.SYS, the following line must be in the CONFIG.SYS

file.

device=EMS40.SYS {/B=xxxx} {/P=yyy{;yyy}} {/H=hh} {/S=ss}

Description:

{ } indicates an optional term and  is not part of the input.

/B=xxxx

xxxx is the base segment address of a 64-Kbyte page frame  in

hexadecimal. Following are the valid page frame base segments

you  may  specify: C000,  C400, C800, CC00 or  D000.

Specify Only one page segment address, even if two  or   more

MEMO-2000A cards are installed. If you don't specify the base

segment, the software will try to install  the  EMS memory at

C000H.

When selecting a page frame segment, you must avoid any possi-

ble memory conflict with other expansion boards that uses the

memory address space C0000H to DFFFFH.  The  most common con-

flicts occur with  some network cards  and  enhanced  graphic

adapters (EGA), and to a lesser extent, some hard disk  drive

controllers.

/P=yyy{;yyy}{;yyy}{;yyy}

yyy  is the I/O port address for the MEMO-2000A.  Each  MEMO-

2000A installed  must be set at a different  I/O port address,

and you can specify a maximum of four ports.   You can choose

from 16 different ports in the range 208H to 2F8H, with a 10H

interval between successive available ports.

The  valid I/O  ports  range from 208H, 218H,  to 2F8H.

If you do not specify the /P parameter, the EMS40.SYS will by

default check I/O  ports 208H-209H, 218H-219H, 258H-259H  and

268H-269H. The EMS40.SYS checks a port by writing  data to it.

If  another  expansion  card  is  using  a port that is being

checked, data in the other card will be corrupted. We suggest

that you  always specify the  /P  parameter. Only the port(s)

specified will be checked, so there is   no  chance   data in

other cards can be damaged.

/H=hh

hh is the maximun number of the active handles in hexadecimal.

The default value is 64, the same value as you specify /H=40.

/S=ss

SS  is the maximun internal stack size  that  reserved to save

the  hardware context of active handles.  The default value is

64,  the same value you would get by specifying /S=40. SS will

be adjusted downwards to the same value as hh  if  you specify

ss larger than hh.