DRIVER INFORMATION

Driver Name:    NT40TLI.ZIP

Driver Date: 2/28/97

Driver Version: 4.01

Operating System:    Windows NT 4.0

Tseng Graphics chip support:

ET6000

ET4000/w32p

ET4000/w32i

ET4000/w32

ET4000

Driver    File List:

DISK1

ET4000.SYS

VERSION

ET6W32.INF

W32.DLL

README.DOC

DRIVER INSTALLATION

1.          Click the Windows NT Start button.

2.          Move the mouse to Settings.

3.          Move the mouse to Control Panel and click.

4.          Double click the Display icon.

5.          In the Display Properties window, click the Settings tab.

6.          Click Display Type.

7.          In the Display Type window, click Change.

8.          In the Change Display window, click Have Disk.

9.          In the Install From Disk window, type the path to the Et6w32.inf file

from this driver set, or choose Browse to locate this file.

10.      In the Change Display window, click Tseng Labs Inc. Compatible

Graphics Adapter and click OK.

11.      In the Third-party Drivers window click Yes.

12.      Restart the computer according to the instructions in the Installing

Driver dialog box.

13.      After system restart, use the Display Properties/Settings tab to

select a desired mode.

DRIVER FEATURES

1.          For ET6000 chips in 4 Mb configurations, the following modes were

added:

1024x768, 16 Million colors to a maximum refresh rate of 60 Hz.

1280x1024,    65k colors to a maximum refresh rate of 60 Hz.

2.          For ET6000 chips, overlay support through Direct Draw was added

for use with players like Active Movie and Direct Draw compliant

software MPEG players.

3.          The overlay surfaces supported by the ET6000 and this driver are:

RGB8      (not stretched )

RGB555 and RGB565    (not at the same time)

RGB888

YUV422

NOTES/LIMITATIONS

1.          16 color support is only available if there is not sufficient video memory

to support 256 colors in high resolution modes. 16 color modes will not

be available except for boards with limited memory.

2.          16 million color modes are only available if memory is configured in linear

mode. The ET6000 is the only chip that this NT 4.0 driver configures in

linear mode, therefore the only chip that will have 16 million

color modes available. The ET4000, ET4000w32, ET4000w32i, ET4000w32p

do not have 16 million color modes available.

3.          The driver does not support a high color mode of 65k colors, only 32k colors

for the ET4000, ET4000w32, ET4000w32i, and ET4000w32p. The mode table is

erroneously reporting 65k colors for these chips. This driver does support

65k colors for the ET6000.

UNINSTALL INSTRUCTIONS

1.          Click the Windows NT Start button.

2.          Move the mouse to Settings.

3.          Move the mouse to Control Panel and click.

4.          Double click the Display icon.

5.          In the Display Properties window, click the Settings tab.

6.          Click Display Type.

7.          In the Display Type window, click Change.

8.	Scroll down the Manufacturers window and click Tseng Labs.

9.	In the Display window highlight a selection for your    chip type.

10.	Click  OK.

11.	Insert the Windows NT 4.0 CD and your original Microsoft drivers will install.

12.	Exit the Display Properties window and reboot the computer.