UNIT My_TPU;

{ A TPU unit of procedures I repeatedly use                                  }

INTERFACE

USES CRT, DOS, GRAPH;

TYPE Set_Of_Char = Set Of CHAR;

Digit_Type  = ARRAY[0..10,0..10] OF STRING[15];

Line_Info_Type =

RECORD

X1, Y1, X2, Y2 : INTEGER;

END; { line info type }

VAR  OK_Set                  : Set_Of_Char;

Digit                   : Digit_Type;

X, Y, Color, Xdir, Ydir : INTEGER;

Lines                    : Line_Info_Type;

PROCEDURE Cursor_Off;

{ Hide the cursor.  Use BIOS interrupt $10, Function 1 to hide the curser   }

PROCEDURE Cursor_ON;

{ Determine the active vidio card.  This is not a trival task               }

PROCEDURE Clear_Kbd_Buffer;

{ Clear contents of keyboard buffer.                                        }

PROCEDURE Get_Character( OK_Set : Set_Of_Char; VAR Ch : CHAR);

{ Read a character from the keyboard.  Print the character's ASCII code at  }

{ the cursor.  Return the ASCII code to program via variable Ch             }

PROCEDURE Select_Printer_Port(VAR Lpt_Num, Lpt_Port_Address : WORD);

{ check for valid port selection  ( 1, 2, or 3 )                            }

{ check that selected printer port is actually installed on machine         }

{ return port address from BIOS variable segment, ( Lpt_Port_Address )      }

PROCEDURE Find_Lpt1(VAR Lpt1_Base_Address : INTEGER);

{ find first printer port, LPT1 }

FUNCTION Find_LPTx(Num : INTEGER) : WORD;

{ return port address of printer port 1, 2, or 3, selected by variable Num. }

PROCEDURE Init_Printer_Port(Lpt_Num, Lpt_Port_Address : WORD);

{ Initilize selected printer port;                                          }

{ Tell ROM-BIOS service routine $17, function $01, to initilize the         }

{ selected printer port, Lpt_Num.                                           }

PROCEDURE Generate_Characters( Digit : Digit_Type );

{ generate large numbers from extended ASCII characters }

IMPLEMENTATION

VAR         Regs : REGISTERS;

PROCEDURE Clear_Kbd_Buffer;

{ Clear contents of keyboard buffer.                                       }

{ From PC Intern, by Michael Tischer, Abacus Press, 1992, page 462         }

BEGIN

INLINE ( $FA );                  { CLI: disable hardware interrupts          }

MemW[$40:$1A] := MemW[$40:$1C];  { No more characters in Buffer              }

INLINE ( $1B );                  { STI: enable hardware interrupts           }

END;

PROCEDURE Get_Character( OK_Set : Set_Of_Char; VAR Ch : CHAR);

{ Read a character from the keyboard.  Print the character's ASCII code at   }

{ the cursor.  Return the ASCII code to program via variable Ch              }

BEGIN

REPEAT

REPEAT UNTIL KEYPRESSED;

Ch := ReadKey;

IF ORD(Ch) = 0 THEN { it's an extended character code, between 128 and 255 }

BEGIN

Ch := ReadKey;

Ch := CHAR(ORD(Ch) + 127);

END;

IF (Ch IN OK_Set) AND (Ch IN [CHAR(32)..CHAR(255)]) THEN WRITE(Ch);

IF (Ch IN OK_Set) THEN BEGIN END ELSE WRITE(CHAR(7));

UNTIL Ch IN OK_Set;

END; { get character }

PROCEDURE Select_Printer_Port(VAR Lpt_Num, Lpt_Port_Address : WORD);

VAR Ch : CHAR;

{ check for valid port selection  ( 1, 2, or 3 )                           }

{ check that selected printer port is actually installed on machine        }

{ return port address from BIOS variable segment, ( Lpt_Port_Address )     }

{   $0408 contains port address of LPT1                                    }

{   $040A contains port address of LPT2                                    }

{   $040C contains port address of LPT3                                    }

{ The Programmer's PC Sourcebook, MicroSoft Press, 1991, Page 7-75,        }

{ Compute's Mapping the IBM PC, Compute Books, 1985, page 234              }

BEGIN

REPEAT

ClrScr; Ch := ' ';   Lpt_Port_Address := 0;

GOTOXY(1,10); WRITE('Which printer port do you want to use, 1, 2, or 3? ');

OK_Set := ['1'..'3'];

Get_Character( OK_Set, Ch);

Lpt_Num := ORD(Ch) - 48;

Lpt_Port_Address := MemW[ 0: $0400 + 6 + ( Lpt_Num * 2 ) ];

IF Lpt_Port_Address = 0 THEN

BEGIN

WRITELN;

WRITELN('Port selected is not installed on this machine.');

WRITELN; WRITELN('PRESS ANY KEY TO CONTINUE:');

REPEAT UNTIL KEYPRESSED;     Ch := Readkey;

END; { else }

UNTIL Lpt_Port_Address <> 0;

END; { select printer port }

PROCEDURE Find_Lpt1(VAR Lpt1_Base_Address : INTEGER);

CONST Base_Address = 1024;      Offset = 8;

{ use 8 for LPT1, use 10 to find LPT2, and 12 to find LPT3 }

BEGIN

Lpt1_Base_Address := MemW[0: Base_Address + Offset];

END;

FUNCTION Find_LPTx(Num : INTEGER) : WORD;

{

return port address of printer port, 1, 2,  or 3, selected by variable Num.

From:

Compute's Mapping The IBM PC & PCjr, Russ Cavies, Compute Books, 1985, P 234

The Programmer's PC Source Book, 1991, Thom Hogan, Microsoft Press, Page 7-75

}

CONST Base_Address = 1024;  { base address for IBM printer ports }

VAR Offset : INTEGER;

BEGIN

Offset := (2 * Num) + 6; { 8 = LPT1, 10 = LPT2, and 12 = LPT3 }

Find_LPTX := MemW[0: Base_Address + Offset];

END; { find lpt1 }

PROCEDURE Init_Printer_Port(Lpt_Num, Lpt_Port_Address : WORD);

{ Initilize selected printer port;                                           }

{ Tell ROM-BIOS service routine $17, function $01, to initilize the selected }

{   printer port, Lpt_Num.                                                   }

{ Referances:                                                                           }

{ PC Interrupts, Addison-Wesley, 1991, page 3-31, &  Programmer's Reference  }

{ Manual for IBM Personal Computers, Dow Jones Pub, 1986 Pub. Page 678       }

BEGIN

WITH Regs DO  { from Mastering Turbo Pascal 6, (c) 1991, Tom Swan, Page 889  }

BEGIN

AH := $01;     { function number }

DX := Lpt_Num; { printer port number }

END; { with }

INTR($17, Regs);

END; { init printer port }

PROCEDURE Cursor_Off;

{ Hide the cursor.  Actually, sets cursor off the screen                     }

{ Uses BIOS interrupt $10, Function 1 to hide the curser                     }

{ Restores the normal default cursor setings.                                }

{ Uses BIOS interrupt $10, Function 1 to hide the curser                     }

{ Turbo Pascal 6, Comp. Ref; Borland/Osborne Pub; Stephen. O'Brien, Page 416 }

{ PC Interrupts, Addison Wesley Pub. 1991, by Ralf Brown; Page 5-12          }

{ Programmer's Ref Man for IBM Per. Comp. Dow Jones Irwin Pub, 1986; P.522   }

{ Programming the IBM User Interface; Addison-Wesley 1989; Ben Ezzell; P.223 }

BEGIN

WITH Regs DO

BEGIN

ah := $01;  { set cursor shape           }

ch := $20;  { hide cursor, starting row  }

cl := $02;  { hide cursor, ending row    }

END; { with }

INTR($10, Regs);

END; { cursor off }

PROCEDURE Cursor_ON;

{ Find out if a graphics card is being used.  Use apropreate procedure to    }

{ turn on the cursor. Hercules and MDA video cards are different from CGA    }

{ Turbo Pascal 6 System Programming, Abacus 1991, by Michal Tischer, Pg 414  }

TYPE Video_Cards = (MDA, CGA, HERC, EGA, VGA, Unknown );

VioSaveBuf = ARRAY [$0000..$3FFF] OF BYTE;

VSPtr = ^VioSaveBuf;

VAR                          Video_Card : Video_Cards;

Vio_Ptr : VSPtr;  { pointer to video RAM }

IN_Graphc : BOOLEAN;

Active_Mode : String[10];

Video_Mode, Scr_Lines : BYTE;

Vidio_Mode : BYTE;

Graph_Driver, Graph_Mode : INTEGER;

PROCEDURE Graphics_Cursor_ON;

{ Uses BIOS interrupt $10, Function 1 to restore the normal cursor           }

{ Restores the normal default cursor setings to any color graphics card      }

{ Programming the IBM User Interface; Addison-Wesley, 1989; P.223            }

BEGIN

WITH Regs DO

BEGIN

ah := $01;

ch := $06;

cl := $07;

END; { with }

INTR($10, Regs);

END; { graphics cursor on }

PROCEDURE Mono_Cursor_ON;

{ Uses BIOS interrupt $10, Function 1 to restore the normal cursor. Restores }

{ the normal default cursor to monochrome and Hercules graphics cards.       }

{ Programming the IBM User Interface; Addison-Wesley, 1989; P.223            }

BEGIN

WITH Regs DO

BEGIN

ah := $01;

ch := $09;

cl := $0A;

END; { with }

INTR($10, Regs);

END; { mono cursor on }

PROCEDURE Get_Video_Card;

{ Determine the active vidio card.  This is not a trival task               }

{ Turbo Pascal 6 System Programming, Abacus 1991, Michael Tischer,Pg 410-15 }

Const VGA_Vidio_Tab : ARRAY[0..12] OF Video_Cards =

(Unknown, MDA, CGA, Unknown, EGA, EGA,

Unknown, VGA, VGA, Unknown, CGA, CGA, CGA );

Mono_Adr_Reg = $3b4;                   { monochrome card port address }

Mono_Status = $3ba;           { monochrome card's status port address }

VAR   Regs : Registers;

i : INTEGER;

Status : BYTE;                             { MDA-/ Hercules status port }

BEGIN

Video_Card := Unknown;

Regs.AH := $0F;                          { Function to determine vidio mode }

INTR($10, Regs);

Video_Mode := Regs.AL and $7f;                           { store vidio mode }

Regs.AX :=$1a00;                        { Function to determine if only VGA }

INTR($10,Regs);

IF (Regs.AL = $1a) THEN                            { is function available? }

BEGIN

IF (Regs.BL <> $1a) THEN          { yes, VGA, code of current card in  BL }

Video_Card := VGA_Vidio_Tab[Regs.BL]         { yes, get code from table }

END

ELSE                                              { not VGA, is it an EGA ? }

BEGIN

Regs.AH := $12;                                       { call function $12 }

Regs.BL := $10;                                       { sub function $10  }

INTR($10, Regs);                                      { call video BIOS   }

If (Regs.BL <> $10) THEN Video_Card := EGA;           { EGA  is installed }

END; { else }

IF (Video_Card = Unknown) THEN                 { vidio card  still unknown? }

IF  (MemW[$0040:$0063] = Mono_Adr_Reg ) THEN          { monochrome card ? }

BEGIN                                    { yes, must be MDA or Hercules }

{ if card is Hercules, then bit 7 in CRT status register will change,  }

{ otherwise, the card is a monochrome display                          }

Vio_Ptr := PTR($B000, $0000 );

Status := PORT[Mono_Status] AND $80;             { read CRT status port }

i := 0;

WHILE ( PORT[Mono_Status] AND $80 = Status) AND (i < 32767) DO INC(I);

IF ( I = 32767 ) THEN Video_Card := MDA ELSE Video_Card := Herc;

END { if mem }

ELSE Video_Card := CGA;

END;

PROCEDURE Error_Message;

BEGIN

ClrScr;

WRITELN;

WRITELN('Unable to restore your cursor. ');

WRITELN('Your type of vidio display is "UNKNOWN" to this software');

WRITELN; WRITE('PRESS ANY KEY TO CONTINUE: ');READLN;

END; { case }

BEGIN { cursor on }

Get_Video_Card;

CASE Video_Card OF

MDA, HERC : Mono_Cursor_ON;

CGA, EGA, VGA : Graphics_Cursor_ON;

UNKNOWN : Error_Message;

END; { case }

END; { cursor on }

PROCEDURE Generate_Characters( Digit : Digit_Type );

{ generate large numbers from extended ASCII characters }

BEGIN

DIGIT[0,1] := (' ÜÛÛÛÛÛÜ ');

DIGIT[0,2] := ('Ûß     ßÛ');

DIGIT[0,3] := ('Û       Û');

DIGIT[0,4] := ('Û       Û');

DIGIT[0,5] := ('Û       Û');

DIGIT[0,6] := ('Û       Û');

DIGIT[0,7] := ('Û       Û');

DIGIT[0,8] := ('ÛÜ     ÜÛ');

DIGIT[0,9] := (' ßÛÛÛÛÛß ');

DIGIT[1,1] := ('        Û');

DIGIT[1,2] := ('        Û');

DIGIT[1,3] := ('        Û');

DIGIT[1,4] := ('        Û');

DIGIT[1,5] := ('        Û');

DIGIT[1,6] := ('        Û');

DIGIT[1,7] := ('        Û');

DIGIT[1,8] := ('        Û');

DIGIT[1,9] := ('        Û');

DIGIT[2,1] := (' ÜÛÛÛÛÛÜ ');

DIGIT[2,2] := ('Ûß     ßÛ');

DIGIT[2,3] := ('        Û');

DIGIT[2,4] := ('       ÜÛ');

DIGIT[2,5] := (' ÜÛÛÛÛÛß ');

DIGIT[2,6] := ('Ûß       ');

DIGIT[2,7] := ('Û        ');

DIGIT[2,8] := ('Û        ');

DIGIT[2,9] := ('ÛÛÛÛÛÛÛÛÛ');

DIGIT[3,1] := (' ÜÛÛÛÛÛÜ ');

DIGIT[3,2] := ('Ûß     ßÛ');

DIGIT[3,3] := ('        Û');

DIGIT[3,4] := ('       ÜÛ');

DIGIT[3,5] := ('  ÛÛÛÛÛÛ ');

DIGIT[3,6] := ('       ßÛ');

DIGIT[3,7] := ('        Û');

DIGIT[3,8] := ('ÛÜ     ÜÛ');

DIGIT[3,9] := (' ßÛÛÛÛÛß ');

DIGIT[4,1] := ('Û       Û');

DIGIT[4,2] := ('Û       Û');

DIGIT[4,3] := ('Û       Û');

DIGIT[4,4] := ('Û       Û');

DIGIT[4,5] := ('ÛÛÛÛÛÛÛÛÛ');

DIGIT[4,6] := ('        Û');

DIGIT[4,7] := ('        Û');

DIGIT[4,8] := ('        Û');

DIGIT[4,9] := ('        Û');

write('5');

DIGIT[5,1] := ('ÛÛÛÛÛÛÛÛÛ');

DIGIT[5,2] := ('Û        ');

DIGIT[5,3] := ('Û        ');

DIGIT[5,4] := ('ÛÛÛÛÛÛÛÜ ');

DIGIT[5,5] := ('       ßÛ');

DIGIT[5,6] := ('        Û');

DIGIT[5,7] := ('        Û');

DIGIT[5,8] := ('ÛÜ     ÜÛ');

DIGIT[5,9] := (' ßÛÛÛÛÛß ');

DIGIT[6,1] := (' ÜÛ      ');

DIGIT[6,2] := ('Ûß       ');

DIGIT[6,3] := ('Û        ');

DIGIT[6,4] := ('ÛÜÛÛÛÛÛÜ ');

DIGIT[6,5] := ('Ûß     ßÛ');

DIGIT[6,6] := ('Û       Û');

DIGIT[6,7] := ('Û       Û');

DIGIT[6,8] := ('ÛÜ     ÜÛ');

DIGIT[6,9] := (' ßÛÛÛÛÛß ');

DIGIT[7,1] := ('ÛÛÛÛÛÛÛÛÛ');

DIGIT[7,2] := ('        Û');

DIGIT[7,3] := ('        Û');

DIGIT[7,4] := ('        Û');

DIGIT[7,5] := ('        Û');

DIGIT[7,6] := ('        Û');

DIGIT[7,7] := ('        Û');

DIGIT[7,8] := ('        Û');

DIGIT[7,9] := ('        Û');

DIGIT[8,1] := (' ÜÛÛÛÛÛÜ ');

DIGIT[8,2] := ('Ûß     ßÛ');

DIGIT[8,3] := ('Û       Û');

DIGIT[8,4] := ('ÛÜ     ÜÛ');

DIGIT[8,5] := (' ÛÛÛÛÛÛÛ ');

DIGIT[8,6] := ('Ûß     ßÛ');

DIGIT[8,7] := ('Û       Û');

DIGIT[8,8] := ('ÛÜ     ÜÛ');

DIGIT[8,9] := (' ßÛÛÛÛÛß ');

DIGIT[9,1] := (' ÜÛÛÛÛÛÜ ');

DIGIT[9,2] := ('Ûß     ßÛ');

DIGIT[9,3] := ('Û       Û');

DIGIT[9,4] := ('ÛÜ     ÜÛ');

DIGIT[9,5] := (' ßÛÛÛÛÛÛÛ');

DIGIT[9,6] := ('        Û');

DIGIT[9,7] := ('        Û');

DIGIT[9,8] := ('        Û');

DIGIT[9,9] := ('        Û');

END; { generate characters }

END. { my TPU }