** .OBJ FILE FORMAT DOCUMENTATION

.OBJ FILE IS MADE AS FOLLOWING FORMAT.

ALL DATA IS ASCII

<ADDR.><4 SPACE><OP-CODE><SP><OP-CODE>....<OP-CODE><CR LF>

<ADDR.><4 SPACE><OP-CODE>.....

<ADDR.>=<NNNN><SPACE><NNNN>

<NNNN>=16 BIT NUMBER WRITEN BY ASCII

IF <ADDR.> IS NOT MULTIPLE OF 16,

THEN <OP-CODE> STARTS IN THE MIDDLE OF THE LINE.

EX.

0000 0134               65DE 4100 ....

THE NON OPCODE AREA ARE ALL <SPACE>.

IF AX68K MET THE PSEUDO-OP (ORG OR DEFS),

THEN OBJ FILE LINE IS MAKE CR/LF INSTANTENEOUSLY.

AND THE NEW LINE CONTINUE THE OBJ DATA.

THE SKIPED AREA BY THE PSEUDO-OP ARE NOT DEFINED.

IF MET PSEUDO-OP (EVEN),AND IF PC VALUE WAS ODD,

THEN 1 BYTE 00H IS INSERTED.

THE TERMINATOR OF THE .OBJ FILE IS 1AH (EOF)

LIKE AS CP/M EDITOR.

THIS EOF MARK ARE INSERTED TO THE AREA (AT LEAST 1 BYTE)

AFTER  THE LAST LINE CR/LF.

THEREFORE THE OBJ FILE PARMITS TO BE EDITED BY CP/M EDITOR.

##############################################################################################