Zmodem            Technical Reference Manual          Page 1

Zmodem

Zmodem   has  a built in communications manager  and  doesn't

need  any  type of fossil, in fact you should make  sure  you

turn  the  fossil  OFF if running it on a  BBS  that  uses  a

fossil.

Zmodem requires a number of command line switches:

-B, -P, {-R or -F}

-@<filename>, Can be a text file listing the files to  Send.

One  filename  per line, unlimited  number  of

lines. This is to used in place of -F.

-B<Baud Rate>

-P<Port Number>

-L<Serial Lock Rate>

-R Receive files

-UDrive:path,  directs that all files  received  should  be

stored on this drive and in this  directory.

This  switch  is primaryly  for  NON-OSIRIS,

NON-QT  systems. Osiris takes care  of  this

internally  and  this switch should  NOT  be

used.

Example: -UC:\uploads

-UD:\files\temp

-UD:

Do NOT end the path with a backslash "\"

Illegal -U settings would be C:\

C:\uploads\

D:\files\temp\

-F <Filenames>

-K Keep interrupted transfers (Do not use if running on a BBS)

-Nx, with x being the node number in use. For use on Osiris

CBIS  systems only.  Zmodem will show the name  of  the

caller as well as where he's from if this switch is used.

-B,  supported baud rates 300 - 115,200 Baud.

NOTE:  If  no -B switch is given, Zmodem  will  use  the

current  baud  rate the serial port is set to.  This  is

primaryly  a debugging aid. If you leave off the -B  and

you are running a LOCKED serial port (high speed  modem)

your  estimated  transfer  times,  cps,  and  efficiency

ratings will be off.

-S,  Handskaking Slow. Should ONLY be used if you are running

a highspeed modem WITHOUT a 16550 Uart.

-P,  Supported  Com  Ports, COM 1, COM  2, COM 3, and COM 4.

-L,  Supported Locked Baud Rates, 300 - 115200 Baud.

-F,  As  many as DOS allows. Full drive, path,  and  filename

supported.

Zmodem            Technical Reference Manual          Page 2

NOTE:   -R  -@ or -F  MUST be  the   last

parameter on the command line. As soon as

Zmodem encounters either one of these two

switches, it will begin to either Receive

or Send.

Sample Osiris Command line (protocol control file)

Send : -N*N -B*B -P*P -@*U

RCV  : -N*N -B*B -P*P -R

QT 2000 (protocol control file definitions)

Send : -B*B -P*P -F *F

RCV  : -K -B*B -P*P -R

Items to watch out for

Zmodem  has it's own built in commucation driver.  Having  an

ACTIVE   fossil  may  cause  system  lockups,  sessions   not

starting,  and  other possiblities. Make sure  you  have  the

fossil turned off if your running one.

Zmodem            Technical Reference Manual         Page  3

Zmodem  may be freely used by Osiris Owners.  If you do NOT

own Osiris, Zmodem must be purchased.

The price is 15 dollars (US Currency).

Mail to :

Micro TECH Systems

555 North Spring, #39

Cape Girardeau Mo. 63701

Revision History

ver 8.10

Added -S, Handshaking Slow. This causes RTS to DROP during

disk writes. This should only be used *IF* you are running a

high speed modem WITHOUT a 16550 Uart. Try running without the

-S first, if you have a fast computer and you are not running

a multi-tasker or on a Network, you may be able to slide by.

The CORRECT solution is to install a 16550 Uart, priced at

20.00's. If you're computer has the uart soldered to the

mother board you will need to get a second serial card and

install the 16550 on it. ver 8.00

Added  the  ability to leave off the -B  switch  setting  for

debugging purposes.

ver 7.00

Drastic  Speed  up in program speed. You may not  notice  any

increase in transfer speed unless you happen to be running  a

slower  computer attempting to push a highspeed modem  locked

at 38400. Or running a multi-tasker such as Desqview.

ver 6.00

Command line switches and their respective values no longer

have a space between the switch and the value.

Old Method : -N *N -B *B -P *P -F *F

New Method : -N*N -B*B -P*P -F *F

Scrolling Batch effeciency window added.

ver 4.00

New display screen like the one in Tmodem.

Only  Zmodem drivers  that support this feature will supply

batch information. Zmodem.exe and DSZ are the only two that

I know of  at this moment  that do. If  the sender  doesn't

support batch information, the display will remain blank.

Added EFFx.LOG for Osiris Systems.

Zmodem            Technical Reference Manual         Page  4

ver 3.00

Allowing  Zmodem to be used on non-Osiris systems.

If  OSIRIS=  is not set, Zmodem will assume an alien  BBS  or

Terminal  program  and  switch  to  NON-COMPORT.DAT   status.

Meaning  it'll  used the standard com  port  definitions  for

COM1,  2, 3, and 4. Baring in mind that 3 and 4 have no  real

standard.