#

File: sharp.info,  Node: Top,  Prev: (schneide.info),  Next: (sony.info),  Up: (remotes.info)

******************************************************************************

SHARP

******************************************************************************

Version:                3.2

Date:                   10 feb 94

* Menu:

* Technical info::

* TV::

* Stereo::

#

File: sharp.info,  Node: Technical info,  Prev: Top,  Next: TV, Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:            17 bits

Carrier:                32kHz

Transmits code + code XOR #7FE0h

header: None

one:    275us pulse + 1900us space

zero:   275us pulse + 775us space

Space between transmissions:    43ms

#

File: sharp.info,  Node: TV, Prev: Technical info,  Next: Stereo,  Up: Top

******************************************************************************

TV

******************************************************************************

GROUP:  Not needed

Code    Function

----    --------

#2141   0

#2021   1

#2041   2

#2061   3

#2081   4

#20A1   5

#20C1   6

#20E1   7

#2101   8

#2121   9

#2221   Ch+

#2241   Ch-

#22C1   Power

#25E1   10+

#2601   20+

#2621   30+

#2261   TV/video

#22E1   Mute

#

File: sharp.info,  Node: Stereo, Prev: TV,  Up: Top

******************************************************************************

STEREO

******************************************************************************

GROUP   Not needed

Code    Function

----    --------

#2474   Prev

#2454   Play

#2434   Next

#24F4   Stop/Disc skip

#22D2   Power

#2252   Preset -

#2232   Preset +

#30B2   Tuning -

#3092   Tuning +

#3052   Auto scan

#2652   FM

#2632   Mono/Stereo

#2692   AM