���������������������������������������������ͻ

� ASERCTL.COM - 100/200LX Serial Port Control �

���������������������������������������������ͼ

Version 2.1

**********************************************

* Previous Version Users please note the new *

* installation instructions.                 *

**********************************************

What is ASERCTL?

This utility is a HP 100/200LX TSR that will allow control over

the serial port power once it is installed.  To turn power on

press [ALT][+] at any time.  To turn port power off, press

[ALT][-] at any time.  ASERCTL takes 352 bytes memory.

Why do I need it?

The 100/200LX is supplied with a small utility called SERCTL.COM

to control the serial port from DOS.  Unfortunately, the port

setting in SETUP over-rides the action of this program.  Also

System Manager does not turn off the port when it is done with

it when exiting cc:Mail or Datacomm.  By properly managing power

to the port, you can extend your battery life up to 20%.

System Manager does have a 15 minute timeout on the power

to the serial port if it is unused. This feature is not

disconnected by using ASERCTL.

INSTALLATION:

1.  From Filer highlight ASERCTL.COM and press [Enter].  You

should get the following message:

ASERCTL.COM has been successfully initialized.

You can now add it to your AUTOEXEC.BAT and reboot.

If you instead get the following message:

ERROR: Interrupt 1C has been hooked by a TSR.

Launch System Manager without loading TSRs

and run ASERCTL.COM from Filer again.

temporarily remove any TSRs from your AUTOEXEC.BAT file,

reboot and run ASERCTL.COM from Filer again. 100BUDDY is

a TSR that can cause this error.

Once you get a success message you can restore your TSRs.

2. This utility should be loaded prior to System Manager.

Place it in your AUTOEXEC.BAT file before the '100' or

'$SYSMGR' command. If you do not have one on A: drive or

C: drive copy D:\CONFIG.SYS and D:\AUTOEXEC.BAT to

either A:\ or C:\.

3. Reboot your 100/200LX by pressing [CTRL]+[ALT]+[DEL] and

confirm that you see the ASERCTL sign on message.

4. From FILER or AppManager run ASERCTL one more time

and you will now have complete control over the port.

NOTE:

The Port control option in SETUP will now properly set

the port power when shelling to DOS. This means that it

will set it once either ON or OFF. You maintain control

with [ALT][-] to turn it off and [ALT][+] to turn it on.

When attempting to link with F6 in FILER you will need

to enable the port with [ALT][+].

ASERCTL.COM is a freeware utility. Not to be commercially

distributed without written permission from the author.

(c) Copyright 1993-94 Mark V. Scardina

All Rights Reserved