--------------------------------------------------------------------------------------------------------------------------------

OPTi  82c924 Windows NT 4.0 Setup Instructions

Sound Expression Boards

--------------------------------------------------------------------------------------------------------------------------------

These drivers add the following support for Windows NT 4.0:

Wave playback/capture

Midi playback

Mic recording

Volume control

Driver installation & configuration

IDE CD-ROM support

NOTE: This driver is ONLY for the sound portion of the Sound Expression boards! The modem will still need a PnP BIOS to configure the port in order for it to be used with Windows NT 4.0

Installation Instructions:

1. Power down the system and insert the Sound Expression board in an available 16 bit slot.

2. Power up the computer

3. Go into Control Panel then Double Click on the Multimedia icon.

4. Click on the "Devices" tab.

5. From the list presented, choose "Unlisted or Updated Driver" and click OK.

6. Insert the driver disk and change the path in the dialog that comes up to A:\ (Or B:\ depending on your floppy drive letter).

7. Next Click OK one the "Add Unlisted or Updated Driver" dialog that comes up.

Note: The only device listed WILL be the "OPTi  82c928, 82c929, 82c924, 82c925"

8. The "Opti 82c92x Audio Driver" setup window comes up. It is recommended that you keep all the defaults. The configuration program, which is similar tho CSACSET that shipped with the SE1440, can be accessed thru the Control Panel applet at any time.

OPTi 82c82x Audio Driver  setup:

This is the program used to setup the 82c924 sound card. The following options are available:

I/O Address:		DMA Channel:		Interrupt:

530   Default		0			7

604			1  Default		9

E80			3			10

F40						11   Default

Advanced Settings button -  Use this button to set the DMA Buffer size as well as Single DMA Mode.

( Use Single DMA mode and 16 DMA buffer size are the default settins )

CD-ROM Interface Settings button - Use this button to enable the CD-ROM interface on the Sound Expression. ( Disabled is the default setting )