NOISY DRIVES

(or Don't You Growl at me!)

1.) INTRODUCTION:

One of the many annoying features of the IBM PC and

certain compatibles is the noise generated by the floppy disk

drives.  I have found almost invariably that the fault is not

in the drive but in some brain damage associated with the

original design of the system.  Apparently when Big Blue

first designed the PC, they were anticipating using the

"worst case" floppy disk drives.  These 5.25" drives were to

be so poorly designed (i.e. cheap) that they had to use a 9

millisecond track to track step rate.  Thus, and from then

on, all disk drives, no matter how good, were to be driven at

this slow rate.  But then, who ever accused the Big Blue

Mother of using good disk drives?

Unfortunately, (or fortunately as you view it) most

decent drives are designed to run at a stepping rate of at

least 6 millisceonds (track to track).  When these drives are

run at lower than their rated stepping rate, they often sound

like thrashing machines.  Resonance sets up and the system

will almost walk off the table.

Unfortunately, the cloners in their thirst for 100%

compatibility stuck with the 9 ms. step rate.  Thus, a lot of

the "clones" also sound like thrashing machines.

Most (if not all) of the floppy drives manufactured

today are designed to operate at a 6 ms. step rate.  Some of

the full height drives, such as the MPI 52A, while rated at

5 ms will operate dependably (and very quietly) at 3 ms.  All

of the half height drives that I am aware of will operate at

6 ms.

Having a low threshold of pain as far as noise is

concerned, I decided to do something about it.  The program

STEPRATE is the result.

2.) Usage:

==============================

=        STEPRATE [r]        =

= (where r is 3, 6, 9 or 12) =

==============================

3.) Determining the Proper Step Rate:

a.) Finding out your default steprate:

How can you determine which step rate is proper for

your drive?  Simple!  First boot up your system.

Format a blank diskette and place it in Drive B.  (If

you only have one physical drive, wait till Junior asks

you to insert diskette in Drive B.)

If STEPRATE is run without any parameters, it will

display the present step rate.  Therefore, remove your

D.O.S. diskette from Drive a and replace it with your

JR POWER PACK II diskette.  Make sure you have a write

protect tab on it!  At the A> type the following to

know at what speed your drives normally run:

STEPRATE

(then press the <Enter> key)

Remember the number displayed by STEPRATE.

b.) Setting the steprate at 12:

Set the step rate at 12 ms by typing the following

after the A>:

STEPRATE 12

(then press the <Enter> key)

c.) Checking the noise level:

Next insert a clean diskette (the blank, formated

one) into drive B (or drive A in the case of a hard

disk or single drive system), and fill it up with JR

POWER PACK II files via a COPY *.* command.  Do this by

typing:

COPY A:*.* B:

(then press the <Enter> key)

You should be able to hear the drive grinding away.

d.) Erasing the files from the Drive B diskette:

Erase all the files from the "clean" diskette

in Drive B by typing:

DELETE B:*.*

(then press the <Enter> key)

(Answer Y when D.O.S. asks if it is OK the erase all

files.)

e.) Setting the streprate to 9ms:

Now you need to test the sound of running your disk

drive at 9ms.  Therefore, at the A> type:

STEPRATE 9

(then press the <Enter> key)

Repeat DELETE and then the COPY procedures to the

"empty" diskette.  The drive noise should be different

(but perhaps not less).

e.) Setting the steprate to 6 ms:

Next, set the step rate to 6 milliseconds by

typing:

STEPRATE 6

and perform the noise test using the DELETE and the

COPY commands again.

f.) Setting the steprate to 3 ms:

Finally, set the step rate to 3 milliseconds by

typing:

STEPRATE 3

and test again.

4.) Determining the best steprate:

If the specified step rate is too fast for the

particular drive, you will probably get SEEK errors or BAD

SECTOR error messages during the COPYing process.  In this

case, use the next higher step rate (i.e. larger number).

Once the correct step rate is found, include the command on

a separate line in your AUTOEXEC.BAT file (ie.: STEPRATE 6).

[You will need to have a copy of STEPRATE.EXE on your startup

D.O.S. diskette.]

May peace and tranquility be yours!

Don L. Finley

310 Willow St.

Mt. Carmel, TN 37642

(615) 357-3355 (after 7 pm)