SK-UPPS FTP Packet Driver

Installation

July 1993

Software Version v2.05 Rev. A

UPPSPD.COM is a FTP packet driver for SK-UPPS. It follows the

PC/TCP 1.09 (basic + extended) Packet Driver Specification of FTP.

Before starting UPPSPD you have to load the Data Link Interface (UPPSDLI.COM,

UPPSJET.COM, UPPSISDN.COM, UPPSTR.COM, UPPSFE.COM, UPPSFI.COM or UPPSFM.COM).

Get the command line parameters of UPPSPD by typing UPPSPD -? .

After loading UPPSPD one of the the TCP/IP kernels ETHDRV.EXE, IEEEDRV.EXE or

TOKDRV.EXE of FTP and/or corresponding programs (Telnet emulation, LANWatch,...)

can be loaded.

A special version of the packet driver (UPPSLW.COM) was written for

Token Ring and FDDI. The following enhanced possibilities can be

set by command line parameters:

-n  receives also non-LLC packets

-s  cuts the length of received packets to max. 2048 bytes (larger

packets are ignored by LANWatch).

-t  emulates Token-Ring on FDDI when receiving packets, because FDDI

is not yet supported directly by LANWatch. The FC of an FDDI packet

will be transformed by the following rules:

5xh -> 10h 40h

41h -> 10h 41h

4Fh -> 10h 4Fh

C2h -> 10h 12h