CD-ROM Drive

User’s Guide

Rev. 06/25/97

Micro Solutions, Inc., provides this manual as is, without warranty of any kind, either express or implied. Micro Solutions, Inc., shall not be liable for errors contained herein or for incidental or consequential damages in connection with the furnishing, performance, or use of this material.

FEDERAL COMMUNICATIONS COMMISSION

RADIO FREQUENCY INTERFERENCE STATEMENT

Note: This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses and can radiate radio frequency energy and if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by using one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

MODIFICATIONS

Changes or modifications not expressly approved by Micro Solutions could void the user's authority to operate this equipment.

SHIELDED CABLES

Shielded cables must be used with this equipment to maintain compliance with FCC regulations.

backpack is a registered trademark of Micro Solutions, Inc.

Windows and Windows 95 are registered trademarks of Microsoft Corp.

Micro Solutions, Inc.

132 West Lincoln Highway	Technical Support BBS: 815.756.9100

DeKalb, Illinois 60115	Technical Support FAX: 815.756.4986

Phone: 815.756.3411	Automated Fax Response: 815.754.4600

FAX: 815.756.2928	WebSite: http://www.micro-solutions.com

Table of Contents

1. Introduction

1.1 System Requirements

2. Installation

2.1 Drive Installation

2.2 Software Installation

3. Using BACKPACK

3.1 BACKPACK’s Drive Letter

3.2 Playing Audio CDs

3.3 BACKPACK Sound

3.4 Uninstalling BACKPACK

4. Configuring BACKPACK

4.1 Changing the BACKPACK Drive Letter

4.2 BACKPACK Device Driver Parameters

4.3 The NONSTOP Option

4.4 Changing the Internal Drive ID

5. Troubleshooting

5.1 General Troubleshooting Tips

5.2 Error Messages and Specific Problems

Appendix A CD-ROM Tutorial

A.1 Program/Data CDs

A.2 Audio CDs

Appendix B Technical Support

Appendix C Microsoft CD-ROM Extensions

C.1 Microsoft CD-ROM Extensions Options

C.2 Common Microsoft CD-ROM Extensions Error Messages

Appendix D Installation Under OS/2

Appendix E Hardware Warranty

Appendix F Software License Agreement

# Introduction

Congratulations on your decision to purchase backpack, the easy-to-install CD-ROM drive for PCs, XTs, ATs, PS/1s, PS/2s, laptops, notebooks and compatibles. This manual provides information regarding the installation and use of backpack..

## System Requirements

Check the system requirements listed here to make sure you have everything needed for proper operation of backpack:

- IBM PC, XT, AT, PS/1, PS/2, laptop, notebook, or compatible computer with:

- 100% IBM compatible parallel printer port.

- 128K memory.

- DOS version 3.1 or above or

- Windows 3.x, Windows 95 or Windows NT.

Note: backpack is Enhanced Parallel Port (EPP) Aware. If the parallel port is equipped with EPP and the computer BIOS has support for EPP, backpack will detect this and use the feature to improve its performance.

# Installation

The backpack drive and software must be installed before you can use the backpack drive. The step-by-step instructions in Sections 2.1 and 2.2 will help you perform the installation.

## Drive Installation

Installing the backpack drive is a straightforward process. It consists of plugging the drive into your computer’s parallel printer port. The instructions in this section will guide you step by step through the installation procedure. Perform the following steps to install the backpack drive:

- Locate the parallel printer port connector on your computer. If you have more than one parallel printer port, it doesn't matter which one you use for backpack.

- If you have a printer connected to the parallel printer port, disconnect it and attach the printer cable to the connector labeled “Printer” on the backpack drive.

- Connect the cable supplied with the backpack to the connector labeled “Computer” on the back of the backpack drive.

- Connect the backpack cable to the printer port on the computer.

- Plug the backpack power unit into a wall outlet and attach the power cable to the power connector on the drive. backpack must be powered on to operate.

## Software Installation

The SETUP program on your backpack SETUP disk 1 will install the software for you automatically. This procedure assumes your diskette drive letter is A. If you use a different drive letter, substitute it in the procedure. Use the following procedure to run SETUP:

Windows Installation Procedure (Win, 95, and NT):

- Place your backpack SETUP disk 1 into drive A.

- Click on Start, Run  or  File, Run and type

- A:SETUP.

- Click OK.

You will be prompted if SETUP needs you to insert any other disks.

The installation will be complete after you restart your system.

DOS Installation Procedure:

- Start your computer as you normally would.

- Place your backpack SETUP disk 1 into drive A.

- Run the SETUP program with the following command at the DOS prompt

A:\setup

The installation will be complete after you restart your system.

NOTE: The backpack OS/2 Installation Procedure is included in Appendix D of this User’s Guide.

The backpack diskette includes a file called README.TXT which contains information gathered since this guide was written. You can view or print this text file using NOTEPAD under Windows. Under DOS, you can use the DOS EDIT utility to view or print README.TXT.

# Using BACKPACK

backpack is used just like any other disk drive. It has its own drive letter. You can access it from your programs by referring to the drive letter. The only restriction is that you can't write to the CD-ROM drive. If you have never used a CD-ROM drive before, a brief tutorial is included in Appendix A of this User’s Guide.

## BACKPACK’s Drive Letter

Just as your existing disk drives are referred to by letters (A:, B:, etc.), backpack also has a drive letter associated with it. The letter to be used is assigned by DOS or Windows and is determined by which letters are already in use on your system. backpack will get the next available letter in alphabetical order. In a typical hard disk system where the floppy drives are A: and B: and the hard disk is C:, backpack will be drive D:.

If you are using Windows 95 or NT:

The CD-ROM drive will appear as an icon in Files Manager, or My Computer and Windows Explorer

If you are using MS-DOS or Windows 3.x:

When your computer is powered up, the Microsoft CD-ROM extensions program will display a message on the screen to tell you the backpack letter. The message will look like this:

Drive D: = Driver BPCDDRV$ unit 0

In this case, you will refer to the backpack CD-ROM drive as drive D: when you are accessing it through any software packages.

You can check the backpack drive letter at any time by one of two methods:

A)	Insert the backpack SETUP disk 1 in drive A and type:

A:bpdrives or

B)	From the hard drive in DOS, type:

C:\bpcdrom\bpdrives

Additional backpack CD-ROM statistics, including the parallel port mode that has been detected and used by backpack can be obtained by using the /x switch:

C:\bpcdrom\bpdrives /x

## Playing Audio CDs

To play audio CDs on the backpack CD-ROM drive, you can use Windows Media Player or any suitable CD-Audio player software.

Playing An Audio CD Under Windows 95 or NT4:

Make sure that you have already installed Multimedia support for Windows (under Control Panel choose Add/Remove Programs, Windows Setup).

Choose:  Start, Programs, Accessories, Multimedia, CD Player.

Playing An Audio CD Under Windows 3.x or NT 3.51

To play an audio CD under Windows, perform the following steps:

- Double click on the MEDIA PLAYER icon in the ACCESSORIES window. A window similar to a CD player front panel will appear.

- Select DEVICE, then CD AUDIO, to inform the media player to use the CD-ROM drive.

- Use the buttons on the media player to start the CD.

Before using audio CDs with Windows 3.x, you may need to inform Windows that you have a CD-ROM drive with audio capability attached.

- Insert an audio CD in the backpack drive.

- Open CONTROL PANEL in the MAIN Program Group.

- Double click on the DRIVERS icon in the CONTROL PANEL window.

- A list of installed drivers will appear. If the list contains [MCI] CD Audio, the driver has already been installed and you should click on the CANCEL button and skip the next steps. If the driver is not installed already, proceed with the following steps.

- Click on the ADD button. A list of drivers will then appear.

- Click on [MCI] CD Audio and then click on the OK button. Windows may prompt you to insert one of your Windows installation diskettes at this point. If it does, insert the diskette and follow the instructions on the screen.

- Exit Windows; then restart Windows to load the new driver.

Playing An Audio CD Under DOS:

To play an audio CD under DOS, invoke your DOS audio player software from the DOS prompt. You can also use the TRKPLAY.EXE utility under DOS. TRKPLAY.EXE is supplied on your backpack SETUP disk 1 or 2 in the \UTILITY directory.

## BACKPACK Sound

This section only applies if your backpack is equipped with the 16-bit sound card option.

backpack Sound refers to the special capabilities added by the built-in 16-bit sound card.

NOTE: BACKPACK Sound is available with Windows 3.x and Windows 95 only.

### The Audio Rack

The Audio Rack allows you to take advantage of backpack’s audio capabilities with the convenience of all the controls being in one place.  You can play audio CDs, wave files (in .WAV and .AUD formats), and MIDI files.  With the multi-source mixer, you can blend these sources with line-in and microphone sources in anyway you choose.  Then you can record these creations as wave files and edit them with the Audio Recorder.

The AudioRack has five main parts.

- The Command Center customizes the appearance of the AudioRack.

- The Digital Audio Player plays and records files in the .WAV format.

- With the Compact Disc Player, you can play audio CDs on the backpack CD-ROM drive.

- MIDI files can be played with the MIDI Player.

- Finally the Mixer controls the volume and recording level of these various devices.

NOTE: For more information about the Audio Applications choose Help in the Audio Rack window.

### The BACKPACK Sound Setup Screen

A setup screen is available for customizing some backpack driver settings.  Normally these settings should not need changing.  Follow the instructions below to access the setup screen.

Windows 95

- Click on Start, then click on Settings, then Click on Control Panel to open the Control panel folder.

- Double-click on the Multimedia icon to open the Multimedia Properties screen.

- Click on the Advanced tab to display the multimedia devices.

- Double-click on Audio Devices.  The backpack Sound device should appear.

- Double-click on BACKPACK Sound to display the Sound Properties screen.

- Click on the Settings button. The backpack Sound Setup screen should appear.

Windows 3.x

- Double-click on the MAIN folder.

- Double-click on the Control Panel icon.

- Double-click on the Drivers icon.

- Click on the Driver for BACKPACK Sound line, then click on the SETUP button. The backpack Sound Setup screen should appear.

### Audio Connections

The backpack sound option provides a volume control and several audio jacks.

- PHONES  is a stereo output for a set of earphones or computer speakers. This is an amplified output and the output level is controlled by the VOLUME thumbwheel knob (this may be labeled SPK on some models).

- LINE OUT  is a stereo output that can be connected to an amplifier input.

- MIC  is a monaural input that is normally used for a microphone.

- LINE IN  is a stereo input that could be connected to many different audio sources (this may be labeled AUX on some models).

Note:  The backpack sound option has a small internal speaker which can be used when the external speakers are not connected.  The audio volume and clarity of the small internal speaker is lower than some external speakers.

## Uninstalling BACKPACK

Because backpack is portable you may wish to install it on a system only temporarily.

If you use Windows 95 or Windows NT version 4

Choose Start, Settings, Control Panel, Add/Remove Programs. Highlight the BACKPACK CD-ROM entry. Click the Add/Remove button.

If you use Windows3 .x:

- Choose File, Run and enter SYSEDIT.

- Click on the tab labeled CONFIG.SYS. Locate and delete the lines that refer to the backpack driver BPCDDRV.SYS. The line will be similar to:

device=\bpcdrom\bpcddrv.sys /D:BPCDDRV$

- Click on the Tab labeled AUTOEXEC.BAT. Locate and delete the line similar to:

if exist bpcddrv$ \bpcdrom\mscdex /d:bpcddrv$

NOTE: If /d: appears more than once in this line, delete just /d:bpcddrv$ and leave the rest of the line as is.

- If your backpack has the Sound Card option click the tab labeled SYSTEM.INI. Locate and delete these lines:

- device=vbacpac.vxd     (found in the [386Enh] section)

- WAVE=bpsound.drv     (found in the [Drivers] section)

- MIDI=bpsound.drv

- MIXER=bpsound.drv

- Close SYSEDIT and save your changes.

- Open File Manager. Locate the folder where the backpack files were installed, usually C:\BPCDROM. Delete this folder.

- If your system has the Sound card option locate and delete the folder where the audio files were installed, usually C:\BPAUDIO. In Program Manager delete the BACKPACK Sound Program Group.

- If you use MS-DOS:

- You can use the DOS EDIT command to open CONFIG.SYS and AUTOEXEC.BAT, found in the root of the C:\ drive. Edit each file as described above. You may then delete the files and directory where backpack was installed, described above.

# Configuring BACKPACK

## Changing the BACKPACK Drive Letter

You can change the drive letter assigned to backpack to suit your preference or to avoid conflicts with other devices.

If you use Windows 95 or NT 4:

In Control Panel, Choose System and click on the Device Manager tab.

Double click on CD-ROM, then double click on BACKPACK CD-ROM. Click on the Settings tab.

Select a Start drive letter and End drive letter that does not conflict with any other drives on the system, for example M.

If you use MS-DOS or Windows 3.x:

Make sure the CONFIG.SYS file has a LASTDRIVE statement of LASTDRIVE=x where x is a letter greater than the desired drive letter.

Edit AUTOEXEC.BAT and add or change /L option for the Microsoft CD-ROM Extensions. For details see Appendix C.

## BACKPACK Device Driver Parameters

By default, the backpack device driver will test the computer’s parallel port upon boot-up and automatically determine which configuration adjustments to make. In the event that backpack does not function properly you can add the options described below under BACKPACK Options.

If you use Windows 95 or Windows NT

Open Control Panel with Start, Settings and double-click on the backpack icon.

Drop-down lists and check boxes control various options.

Default settings are:

T1, T2 and T8 = AUTO

MAX = EPP

Test For Chipset  Checked

Auto-Insert Notification  Checked.

These options are described below.

If you use DOS or Windows 3.x

The options described below can be used at the very end of the backpack CD-ROM’s device driver line in the CONFIG.SYS file.

device=\bpcdrom\bpcddrv.sys /d:bpcddrv$

This is the default syntax for the backpack CONFIG.SYS line. This should allow backpack to operate properly on almost all computers. The /d:bpcddrv$ parameter specifies the name of the backpack CD-ROM device driver in memory and must not be altered. This parameter does not assign the D: drive designation to backpack. See Section 4.1 of this User’s Guide for modification of the backpack drive letter.

Options can be combined on the same line, if necessary. For example:

device=\bpcdrom\bpcddrv.sys /d:bpcddrv$ NOEPP T1=10

would cause the backpack device driver software to bypass the backpack Enhanced Parallel Port usage and add 10 more timing delays to the data transfer signals from the backpack to the computer’s parallel port.

Always reboot the computer after saving changes to the CONFIG.SYS file.

BACKPACK Options

NONSTOP	This option is detailed in Section 4.3 of this User’s Guide, and does not apply to Windows 95 or Windows NT.

NOEPP	This option turns off backpack’s testing and usage of Enhanced Parallel Ports (EPP). If the parallel port chip in your computer exhibits EPP properties, but is not fully EPP compatible, then this parameter may be necessary to allow backpack CD-ROM to function on that computer’s parallel port. Be sure to turn off the computer’s power and the backpack CD-ROM drive’s power after saving this particular modification to the CONFIG.SYS. This will allow the backpack I/O chips and the computer’s parallel port chipset to be reset to a non-EPP mode.

NOEPP mode can be selected from a drop-down list under Windows 95 or NT in backpack Controls. Set MAX to either Bidirectional or Unidirectional.

UNIDIR	This option turns off backpack testing and usage of bidirectional parallel ports. If the parallel port chip in your computer cannot properly transfer data in bidirectional mode, it may be necessary to force the backpack into unidirectional operation on that computer’s parallel port.

Unidirectional mode can be selected from the a drop-down list under Windows 95 or NT in backpack Controls. Set MAX to Unidirectional.

T1=xx	Values of 1 to 50 are typical for this parameter. This option will place additional signal speed delays on the backpack CD-ROM when sending data into the computer’s parallel port. If the computer’s parallel port is not capable of transferring data at the speed calculated by the backpack device driver during boot, this addition of timing delays may be necessary. The higher the value, the more timing delays are added.

T2=xx	Values of 1 to 50 are typical for this parameter. This option will place additional signal speed delays when receiving data from the computer’s parallel port.

T8=x	Values of 1, 2 or 3 are valid for this parameter. This option adds additional EPP signal speed delays on the backpack CD-ROM when sending data on an Enhanced Parallel Port. If the computer has an EPP port, but the port is not capable of transferring data at the optimum EPP speed, additional signal delays may be necessary.

Auto-Insert Notification

This is a feature of Windows 95 and Windows NT 4 that allows Windows to automatically launch an application on CD or begin playing an Audio CD when it is inserted.

Auto-Insert Notification can cause problems with some printers attached to backpack.

Test for Chipset

This setting is controlled by a checkbox in Windows 95 and NT. When the backpack device driver loads it attempts to determine the type of parallel port in the computer. These tests could cause some computers to lock up. In such a case re-start the computer without the backpack connected and clear this box.

## The NONSTOP Option

This Section only applies if you are using MS-DOS or Windows 3.x

When the DOS system loads the backpack software driver, it will scan the parallel printer ports looking for backpack drives. If it doesn't find at least one backpack CD-ROM drive, it will issue an error message and wait for you to press the ESC key to acknowledge the message or the R key to retry. This could become annoying if the backpack drive isn't always connected to the computer.

The NONSTOP software option can be used to suppress the error message that is displayed if no backpack drive is found when the system starts. Perform the following steps to specify the NONSTOP option:

- Using Windows Notepad or DOS EDIT, bring up the CONFIG.SYS file from your hard drive for editing.

- Locate the line that references BPCDDRV.SYS and add NONSTOP to the end of it, as shown below:

device=\bpcdrom\bpcddrv.sys  nonstop

Be sure to type a space before NONSTOP.

- Save the modified CONFIG.SYS file on your hard drive.

- Restart the computer by holding down CTRL, ALT, and DEL.

Note: If backpack is connected and powered on, but still reports the error “A backpack CD-ROM Drive was not found...” during boot, refer to Section 5, Troubleshooting, of this User’s Guide. Do not install the NONSTOP option since it will only suppress the error and will not correct the conflict.

## Changing the Internal Drive ID

This section is relevant only if you are connecting more than one BACKPACK to your computer.

Each backpack drive is assigned an internal drive ID when it is manufactured. The ID is a number between 0 and 99 and initially is the last two digits of the unit's serial number. When more than one backpack is connected to a computer, the backpack driver software assigns DOS drive letters in ascending drive ID order. The lower the drive ID number, the lower the DOS drive letter.

For example, assume that you have a computer with two floppy drives (A: and B:) and a hard drive (C:). Assume also that you are connecting the following two backpack CD-ROM drives:

Serial number xxxxxx35

Serial number xxxxxx17

Drive letter D: will be assigned to the drive with serial number 17 and drive letter E: will be assigned to the drive with serial number 35.

There are two situations in which you may need to alter a backpack drive ID number:

- If you are connecting two backpack drives to one computer and they happen to have the same last two digits in the serial number. In this case the backpack software will locate only one of the drives and assign it a drive letter. The other drive will not be recognized. Assigning a new drive ID to one of the drives will allow both drives to be recognized.

Note: All backpack tape drives have ID numbers 00 or 01.

- If you are connecting two backpack CD-ROM drives to one computer and you would like to reverse the drive letters. Since drive letters are assigned according to drive ID numbers, the order of the drives can be reversed by assigning new drive ID numbers.

Drive ID numbers are used only for establishing the order of backpack drive letters; they serve no other purpose. Perform the following steps to change a drive ID number:

- Connect the backpack drive whose ID number you will be altering to a parallel printer port. Disconnect all other backpack drives from the computer.

- Unplug the backpack power connector for a few seconds, then connect it again.

- Place your backpack SETUP disk 1 into drive A.

- Run the SETID program with the following command:

A:\setid

The program will ask you to select a new drive ID.

- After you exit the program, remove the backpack SETUP disk 1 from drive A:.

- Unplug the backpack power connector for a few seconds, then connect it again

- Attach all the backpack units to the computer.

- Restart the computer by holding down CTRL, ALT, and DEL.

# Troubleshooting

backpack should not affect the operation of your existing computer software and hardware. If there seems to be a problem using the computer or backpack after installation, read the following problem descriptions to see if they match the problems you are experiencing. Review the README.TXT file included on the software diskette (see Section 2.2) and see if your problem is discussed there. If you can't resolve the problem, review Appendix B  Technical Support.

## General Troubleshooting Tips

- Plug backpack directly into the computer using the supplied cable. Extension cables and other devices between backpack and the computer can cause problems. If you use a Software Security Key (often called a dongle) it will work plugged into the backpack PRINTER connector.

- The backpack CD-ROM Drive is supported in DOS, Windows 3.x, Windows 95, NT and OS/2 environments only. No support is available for any other operating systems

- backpack software is continually being improved. Check with Micro Solutions’ WebSite or Micro Solutions’ BBS for newer software versions. See Appendix B.

- Often read errors or lockups can be solved with selecting the backpack parameters NOEPP and/or T1=20. See Section 4.2.

- Specific printer compatibility information is available and updated regularly on Micro Solutions’ WebSite. See Appendix B.

## Error Messages and Specific Problems

When you are trying to access the drive, an “Invalid drive specification” message appears on the screen.

This can occur if you are not using the correct drive letter to access backpack. Be sure the letter you use is the one indicated on the screen when the computer starts. Review Section 3.1 for information on drive letters.

This can also occur if you have not run SETUP to install the backpack software or if you have not restarted the computer after running SETUP. Review Section 2.2 for information on software installation.

If you use MS-DOS or Windows 3.x this can also occur if there are not enough drive letters available to DOS. If drive letters D: and E: are already assigned to other devices in your computer, you will need to allow DOS to use additional drive letters. Add or change the line: LASTDRIVE=M to the beginning of your CONFIG.SYS file on your boot drive.

The message “A backpack CD-ROM drive was not found” appears when the computer starts.

This will occur if the power to the backpack drive is not on when the computer is started.

This can also occur if the backpack CD-ROM drive’s device driver software has difficulty programming an Enhanced Parallel Port into high-speed mode. Review the NOEPP option in Section 4.2.

This can also occur if a hardware conflict exists. Make sure you don't have two printer ports set to the same port address, and make sure you have a completely IBM compatible printer port.

The backpack appears to operate too slowly. Multimedia programs seem to “stutter” or skip frames

This will occur if the backpack is connected to a parallel port that is not Enhanced (EPP compatible). If your computer’s parallel port is a unidirectional or bidirectional parallel port, backpack will not be able to operate at its optimum speed.

After you install the backpack, the printer doesn't work properly.

Try turning off Auto-Insert Notification in BACKPACK Controls. See Section 4.2.

This can also occur if the power to the backpack is not on. For reliable operation, make sure the backpack is powered on.

After you have connected two backpacks, only one of them is assigned a drive letter. Either drive works properly if connected by itself.

This can occur if two or more backpacks are set to the same internal drive ID number. When more than one backpack is installed on the same computer, they must all have distinct internal drive ID numbers. Drive ID numbers are assigned at the time of manufacture but can be changed with the SETID program. Refer to Section 4.4 for information on changing ID numbers.

The message “BPCDDRV.SYS Error: Bad or missing argument in CONFIG.SYS” appears when the computer starts.

In the CONFIG.SYS line where BPCDDRV.SYS is loaded, a parameter was specified but the argument was either missing or invalid. The invalid argument or the position of the missing argument will be displayed, and you will have to press the ESC key to continue. Refer to Section 4.2 for a list of valid parameters.

The message “BPCDDRV.SYS Error: Unrecognized parameter in CONFIG.SYS” appears when the computer starts

In the CONFIG.SYS line where BPCDDRV.SYS is loaded, an invalid parameter was specified. The invalid parameter will be displayed, and you will have to press the ESC key to continue. Refer to Section 4.2 for a list of valid parameters.

#### CD-ROM Tutorial

##### Program/Data CDs

A CD-ROM drive works much like a floppy diskette drive or hard drive, except that you can’t write (save data) to it. Your CD-ROM drive is a Read Only device.

Once you have placed a CD-ROM disc into the CD-ROM drive, follow the installation or setup procedure that is detailed in the instructions supplied with each CD-ROM disc. Some CD-ROM discs contain an installation program which must be run before the actual programs on that CD-ROM disc will execute.

Your CD-ROM drive will appear under Windows File Manager program or under My Computer in Windows 95 and NT 4 once the CD-ROM drive has been properly installed and the computer has been re-booted.

There is no need for your CD-ROM drive to set up any additional program icons under Windows, since the CD-ROM drive cannot perform any program functions until a CD-ROM disc has been placed in it.

##### Audio CDs

Music CDs which you can play on a home stereo CD player or on a portable CD player are called CD-AUDIO discs. The type of music on these discs is different than the audio found on most Multimedia CDs. You need a CD-AUDIO program on your computer, such as Media Player in Windows, to play CD-AUDIO Music CDs. You may also need to attach speakers or headphones to the front of your CD-ROM drive to hear that CD-AUDIO sound.

Most Multimedia CDs require a computer Sound Board which can convert the multimedia data into sound. Check the requirements listed on the Multimedia CD-ROM disc package to find out if a Sound Board is required for audio.

#### Technical Support

Most questions about backpack and its operation are answered in this guide. To solve most problems:

- Check the solutions and procedures in this User's Guide.

- Check the README.TXT files on the backpack installation diskette.

- Visit Micro Solutions on-line at our WebSite at:

- http://www.micro-solutions.com.

- Contact Micro Solutions' Automated Fax Response at 815.754.4600. Automated Fax Response is available 24-hours daily. A complete catalog of faxes and information is available. If you are calling from outside the United States prefix the digits 011 when you are asked to enter your country code and FAX number.

- Contact Micro Solutions' Bulletin Board Service (BBS) at 815.756.9100 for troubleshooting software, bulletins and driver updates.

If your problem remains unsolved, contact Micro Solutions Technical Support Department at 815.754.4500. Technical Support is available during normal business hours, Monday through Friday, Central Time. Before calling, be sure to have the following information ready:

- The version numbers of your backpack software and your operating system (DOS, Windows 3.x, Windows 95 or NT).

- The name and model of the computer, and the eight-digit serial number found on the bottom of your backpack drive.

- The exact wording of any error message(s) from the backpack program, DOS, Windows or any other application producing the error message.

- The exact model of printer installed, and the revision of printer driver software, if available

If possible, be at your computer when calling.

#### Microsoft CD-ROM Extensions

This Section only applies if you are using MS-DOS or Windows 3.x

Microsoft CD-ROM extensions (MSCDEX.EXE) is an executable program for MS-DOS (version 3.1 or higher) that works in conjunction with the backpack device driver (BPCDDRV.SYS) to allow your computer to access CD-ROM discs as if they were DOS formatted disks. This program is written by Microsoft and is specifically designed to work with Microsoft MS-DOS. Computers with anything other than MS-DOS operating systems may experience unpredictable results unless the operating system manufacturer provides an alternative to the MSCDEX program.

The SETUP program automatically modifies the AUTOEXEC.BAT file to run the MSCDEX program so your computer can access the backpack CD-ROM drive. However, some parameters can be changed, so this section explains the function of each parameter. Use a text editor such as EDLIN or EDIT to modify the appropriate line in the AUTOEXEC.BAT file.

Microsoft CD-ROM Extensions Options

Syntax

MSCDEX  /D:BPCDDRV$ [/M: <value>] [/E] [/V] [/S] [/L:<letter>]

/D:BPCDDRV$

The /D option specifies the name of the backpack CD-ROM device driver and must not be altered.

/L:<letter>

The /L option can be used for software that requires that the CD-ROM drive be identified by a particular drive letter. The following would cause the CD-ROM drive to be assigned drive letter M:.

MSCDEX  /D:BPCDDRV$  /L:M

/M

The /M option determines how many sector buffers MSCDEX will allocate when it installs itself. The larger this value is, the more sector cache entries are available and the less MSCDEX will have to read directly from the CD-ROM drive. The larger this value is, the better the performance will be.

If the /M option is not specified, the default value is 6.

/E

The /E option instructs MSCDEX to use expanded memory for some of its buffers. Note that you must have expanded memory available for this option to work. If no expanded memory is available, the following error message will appear:

Expanded memory not present or not usable.

/V

The /V option instructs MSCDEX to display a summary of memory usage.

/S

The /S option tells MSCDEX to patch MS-DOS to allow the sharing of CD-ROM drives on MS-NET based servers.

Common Microsoft CD-ROM Extensions Error Messages

CDR101: Not ready error reading drive x:

No Disk in drive or The device is not ready

This message is displayed by MSCDEX whenever any type of error occurs on the CD-ROM drive. Since the message is generic, there may be many causes for its appearance. Check for the following conditions.

Make sure the backpack CD-ROM drive is still connected to the computer’s parallel port and still has the original power supply attached. Substituting a power supply of a lower voltage than the one originally packaged with your backpack can produce this error.

Make sure there is a CD-ROM loaded in the drive.

If you just loaded a CD-ROM in the drive, wait a few seconds before accessing it. The drive automatically reads the CD-ROM table of contents when you insert a disc, and this takes a few seconds. During this time the drive will not respond.

Check that the CD-ROM is clean. If necessary, clean it according to the instructions provided with the disc.

This error can also occur if a caching utility, such as Microsoft’s SMARTDRV.EXE program, fails to provide proper caching for the
backpack CD-ROM. To disable the caching of a backpack CD-ROM that is designated as E: drive, add the following to the SMARTDRV line in the AUTOEXEC.BAT file:

c:\windows\smartdrv.exe E-

This is the backpack drive letter, followed by a hyphen.

Reboot the computer after saving the above change to the AUTOEXEC.BAT file.

Not enough drive letters available

MSCDEX cannot allocate a drive letter for the CD-ROM drive. Increase the number of available drive letters, using the LASTDRIVE command in your CONFIG.SYS file. If your CONFIG.SYS file contains a LASTDRIVE command, change the drive letter specified to a higher letter. If your CONFIG.SYS file does not have a LASTDRIVE command, add the line LASTDRIVE=H to the beginning of your CONFIG.SYS file on your boot drive.

#### Installation Under OS/2

Some backpack models include OS/2 software in a directory named \OS2 on one of the SETUP diskettes. You can also download OS/2 CD-ROM drivers from Micro Solutions’ BBS or WebSite. See Appendix B.

When installing backpack under OS/2, the original OS/2 installation diskettes will also be required.

- Open the OS/2 System folder.

- Open the System Setup folder.

- Choose Device Install.

- Follow the instructions displayed. When prompted, insert the backpack CD-ROM Installation diskette. The backpack OS/2 files are in the \OS2 directory of the diskette. Make sure that you specify the complete path: A:\OS2 when you are prompted to specify the installation drive and path.

If there was not already a CD-ROM installed on the system, run Selective Install to set up the OS/2 CD-ROM support:

- Open the OS/2 System folder, then System Setup.

- Open Selective Install.

- Select the CD-ROM Device Support check box.

- Choose OK, then scroll down the CD-ROM device list and select Other.

- Select OK to go from the System Configuration screen to the Setup and Installation window.

- Select Install and follow the instructions displayed.

One or more of the OS/2 installation diskettes will be prompted for. The backpack CD-ROM will be available after restarting the system.

Note: If the backpack is not available after restarting the system, make sure the following lines appear in the CONFIG.SYS file and the files referred to exist in the \OS2 directory of the hard drive:

ifs=\os2\cdfs.ifs /q

device=\os2\mdos\vcdrom.sys

device=\os2\os2cdrom.dmd /q

basedev=bpcdos2.add

When troubleshooting parallel port compatibility problems under OS/2, the NOEPP and UNIDIR parameters detailed in Section 4.2 of this User’s Guide can also be used on the BPCDOS2.ADD driver line. Additional information can be found in the README.TXT file in the \OS2 directory of the backpack SETUP diskette 1 or 2.

#### Hardware Warranty

1 Year Limited Warranty

Micro Solutions, Inc. (MSI), warrants backpack to be free from hardware defects in workmanship and material under normal use for a period of one (1) year from the date of purchase by the original consumer purchaser. During this warranty period, MSI will repair or replace, at its option, any component parts that in its opinion prove to be defective. This warranty does not extend and shall not apply to products that have been subjected to misuse, neglect, accident, or improper installation.

THIS LIMITED WARRANTY AND REMEDY ARE EXCLUSIVE AND EXPRESSLY IN LIEU OF ALL OTHER WARRANTIES EXPRESSED OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE. IN NO EVENT SHALL MICRO SOLUTIONS, INC., BE LIABLE FOR LOST PROFITS, LOSS OF GOODWILL, OR ANY OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. Some states do not allow the exclusion or limitation of incidental or consequential damages, so the above limitation or exclusion may not apply to you.

This warranty gives you specific legal rights, and you may also have other rights, based upon the point of sale.

If your drive is defective, you should return it to the point of purchase for service. If you are unable to do so, contact MSI at one of the phone numbers below to receive the address of an authorized distributor in your country who can perform warranty service. If you live in the United States you may return your drive directly to MSI. If you live outside the United States you may return your drive directly to MSI, but freight cost and duties may be excessive on single item shipments. A Return Material Authorization (RMA) number must be obtained from MSI prior to the return of defective material. Contact MSI with the serial number of your drive by telephone at USA 815.756.3411, Ext. 325, or FAX 815.756.4986 for an RMA number. MSI is not responsible for material returned without the RMA number clearly printed on the outside of the shipping container. Products to be returned to MSI must be returned, shipping and insurance prepaid, by the original purchaser to the address below.

Micro Solutions, Inc.

Attn: RMA# ____________________

300 East Harvestore Drive

DeKalb, Illinois 60115

#### Software License Agreement

Terms and Conditions

Micro Solutions, Inc. (MSI), licenses the enclosed CD-ROM software (“the software”) for your personal use. You assume all responsibility for the operation and results. MSI may from time to time revise this software. In doing so, it shall incur no obligation to furnish such revisions to you. Furthermore, MSI may elect to charge an update fee for such revisions.

Software Warranty Information

MSI warrants that the diskettes containing the software are free from defects and will replace a defective diskette within thirty (30) days from the date of purchase by the original consumer.

MSI makes no warranties, either express or implied, with respect to this software, its quality, performance, merchantability, or fitness for any particular purpose. The entire risk as to the performance of this software is with you.

IN NO EVENT WILL MICRO SOLUTIONS, INC., BE LIABLE TO YOU FOR ANY DAMAGES, INCLUDING ANY LOST PROFITS, LOST SAVINGS, OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OR INABILITY TO USE THIS SOFTWARE.

This agreement shall be interpreted in accordance with the laws of the State of Illinois, USA