From: Ya`akov Miles <multi%dac.triumf.cdn%ubc.csnet at RELAY.CS.NET>

To:   Info-IBMPC at MIT-MC

Re:   BIOS Musings

You may be interested in a history of where this BIOS came from, and

how it arrived in its present form.  A heavily patched,

partially-functionally BIOS (with no copyright statement, or other

visible indication of origin) was supplied with my IBM-PC/xt

compatible 10 mHz motherboard.  In order to get my motherboard to

function correctly, in other words, to work with the parity interrupt

enabled and to operate with the NEC "V20", it was necessary to

disassemble and thoroughly go thru this "anonymous" bios, which was

hinted as supplied by Taiwan, while limping along on a name brand

bios, as supplied on my previous motherboard by a different vendor.

In the course of this disassembly, aided by comparison with the

published IBM-PC/xt listings, it became apparent that the

synchronization on horizontal retrace in the video INT 10h service was

the root cause of the failure to operate with the NEC "V20", and that

correcting it to correspond with logic (as in IBM's bios) caused the

glitch to disappear.  I am unable to account as to why several name

brand BIOS brands (excluding IBM's) had similar glitches - maybe they

they were produced from similar source code, although this seems

unlikely.  In any case, as evidenced by DEBUG, some of these

name-brand BIOS were full of machine-level patches - did the vendor

ever bother to reassemble and optimize the source code.  The code that

I examined was full of recursive INT(errupt) instructions, which did

not to contribute to screaming fast BIOS.  Therefore, the assembly

code was rearranged so as to eliminate some of the unnecessary CALL,

JMP, and especially INT instructions, as the optimization proceeded

with the later releases.  The bios is (c)Anonymous, because there was

no indication of the original authors...

ps: While playing around with my 10 mHz motherboard, I encountered an

unusual program called HELPME.COM, which ran at a higher pitch

than normal.  Since this program behaved normally on other (8 mHz)

turbo motherboards, my curiousity was aroused.  This eventually

led me to discover that the 10 mHz motherboard was refreshed in

hardware by channel 1 of the 8253 timer ic, and that this channel

appeared to be counting down from an unusually fast oscillator.

Maybe this could cause problems with other programs...