DATE: 3/12/93

SUBJ: changes to the source and additional notes with documentation

corrections.

Several changes have been made to the developers tool kit since the

release date.  These changes as well as some additional notes are

documented below.

The type of the variable GP in SetMsndIRQ and ResetMsndIRQ has been

changed to WORD.  This was done for complience with the Microsoft C7.00

compilers _dos_getvect and _dos_setvect functions.

The function SetMsndIRQ in MCONTROL.H had an error when setting an IRQ

value on the first PIC.  The following change was made to the source:

if( nIRQValue > 7 ) {

wGP = nIRQValue + 0x68;

outp( PIC2 + 1 , ~( 1 << (nIRQValue - 8) & inp( PIC2 + 1 ) ) );

}

else {

wGP = nIRQValue + 8;

//    outp( PIC1 + 1 , ~( 1 <<  nIRQValue & inp( PIC1 + 1 ) ) );

//    this old line was removed and replaced with the one below

outp( PIC1 + 1 , ~( 1 <<  nIRQValue ) & inp( PIC1 + 1 ) );

}

The example programs now contain two makefiles.  The original Microsoft

makefiles remain unchanged.  The second makefile is for people using

the Borland C/C++ ver3.1 compiler.  These files have the extension

.MAK.

There are now two header files for the examples.  MCONTROL.H and

PCONTROL.H.  MCONTROL.H remains the same except for the addition of

some functional prototypes.  This file should only be used to include

when compiling MCONTROL.C.  PCONTROL.H is now used as the include in

the example applications.  It is basically the same as MCONTROL.H, only

it defines all the global variables to be external.

The function GetDelayTime used to be an assembly routine.  This

function has been rewritten in C to provide compatability with the

Borland compiler.

The DSP code .ASM file has been changed so that the data now resides in

the DATA segment rather than the CODE segment.  This was done for

complience with the Borland 3.1 compiler.  The source is provided for

this OBJ in case you want to put the code in the code segment to

discard it after uploading.

All the sources have been compiled and tested using the Borland C/C++

ver3.1 compiler.  If you are using an earlier version of the Borland

compiler, you may need to modify the source code for succesful

compilation.

Record example:

The initialization code is wrong for the recording.  The wTail values

should be changed using the macro PCTODSP_OFFSET.  In the current state

the tail is pointing past the end of the queue.

// old version **** BAD ****

DARQ->wTail = DARQ_STRUCT_SIZE;

SendDSPCommand( HDEX_RECORD_START );

DARQ->wTail = 2 * DARQ_STRUCT_SIZE;

SendDSPCommand( HDEX_RECORD_START );

// new version **** GOOD ****

DARQ->wTail = 2 * PCTODSP_OFFSET( DARQ_STRUCT_SIZE );

SendDSPCommand( HDEX_RECORD_START );

There was also a problem with the response to the DSP message for the

completion of a record bank.  The repairs are shown below.

case HIMT_RECORD_DONE:

bLastBank = LOBYTE( wMessage );

// old line **** BAD ****

// wTemp = DARQ->wTail + DARQ_STRUCT_SIZE;

// new line **** GOOD ****

wTemp = DARQ->wTail + ( DARQ_STRUCT_SIZE / 2 );

if( wTemp > DARQ->wSize ) {

wTemp = 0;

}

while( wTemp == DARQ->wHead );

DARQ->wTail = wTemp;

SMA variables:

wCurrPlayBytes and wCurrRecBytes decrement with time.  They show the

current bytes remaining in the currently posted buffer.

Two new variables in the SMA struct allow control of the record

monotoring volume.  These variables replace wUser_1 and wUser_2.  There

new names are:

old:

WORD wUser_1;

WORD wUser_2;

new:

WORD wCurrInVolLeft;    // range 0x0000 - 0x7FFF

WORD wCurrInVolRight;   // range 0x0000 - 0x7FFF

1/19/93

The initialization of the MultiSound is incorrect in the toolkit.

Listed below is the correct procedure:

1. Initialize the board info registers

This involves writing to the following registers to insure that the

board is properly setup.

HP_MEMM

HP_WAIT

HP_BITM

2. InitializeSMA

3. ResetDSP

4. UploadDSPcode

5. Poll *( pMEM.p ) for the dsp to set it to 0.  When this happens

the DSP is ready for normal operation.

1/20/93

When moving the permenent code, the mem copy function should only copy

wDspPermSize bytes, not 0x8000 as was done originally.  This prevents

the uploader from overwriting the initialized values in the SMA.  The

change is shown below.

BYTE UploadDSPCode( void )

{

int nInker;

pbDSPperm = &bDspPermStart;

pbDSPinit = &bDspInitStart;

for( nInker = 0; nInker < 3*wDspInitSize ; nInker += 3) {

if( ( SendHostWord( *(pbDSPinit+nInker), *(pbDSPinit+nInker+1), *(pbDSPinit+nInker+2)))==0) {

return(0);

}

}

outp( ( wBASEIO+ HP_BLKS ), HPBLKSEL_0);

outp( ( wBASEIO+ HP_MEMM ), bMem );

_fmemcpy( pMEM.p , pbDSPperm , wDspPermSize );   // New method

//   _fmemcpy( pMEM.p , pbDSPperm , BUFFSIZE );   Old method

return(1);

}

Also the following lines were added to the initialize SMA function.

They initialize all the DSP RAM to zero.

outp( ( wBASEIO + HP_BLKS ) , HPBLKSEL_0 );

for( wInker = 0 ; wInker < 0x8000 ; wInker++ ) {

*( pMEM.p + wInker ) = 0x00;

}

outp( ( wBASEIO + HP_BLKS ) , HPBLKSEL_1 );

for( wInker = 0 ; wInker < 0x8000 ; wInker++ ) {

*( pMEM.p + wInker ) = 0x00;

}

outp( ( wBASEIO + HP_BLKS ) , HPBLKSEL_0 );

The variables wCurrPlayBytes and wCurrRecordBytes are used to track the

current location within a buffer of audio data.  They start at the size

of the buffer in bytes, and decrease towards zero as the task is

completed.  When the value reaches zero, the current buffer is

complete.

The value of the defined sysex delay in PCONTROL.H and MCONTROL.H

should be increased to 40 hex as shown below.  Using the old delay

caused irregular proteus operation.  The new line should be as follows.

#define TIME_PRO_SYSEX       0x0040

ERRORS in the MultiSound Technical Reference documentation:

pg. 10

The HPMEMM register values are improperly listed.  The correct values

are shown below:

HPMEM_NONE	0	Off / Not Mapped			(PU)

HPMEM_B000	1	B000H-B7FFH

HPMEM_C800	2	C800H-CFFFH

HPMEM_D000  3   D000H-D7FFH

HPMEM_NU	4	not used

HPMEM_D800  5   D800H-DFFFH

HPMEM_E000  6   E000H-E7FFH

HPMEM_E800  7   E800H-EFFFH

pg. 16

The sequence of events to initialize the card is not correct in

Chapter 2.  The correct sequence was discussed previously in this

document.

pg. 23

The table showing who owns which element of each queue contains an

error under the DARQ heading.  The table should read as shown below:

DAPQ -  Digital audio play job queue

Host owns wTail         DSP owns wHead

DARQ -  Digital audio record job queue

Host owns wTail         DSP owns wHead

MODQ -  MIDI out data job queue

Host owns wTail         DSP owns wHead

MIDQ -  MIDI in data job queue

DSP owns wTail          Host owns wHead

DSPQ -  DSP to host messages job queue

DSP owns wTail          Host owns wHead

pg. 31

Under the section Playing Audio it is not neccessary to check for

repeated bank empty messages.  This fact is reflected in the new demo

sources.

pg. 43

The wUser_1 and wUser_2 elements of the SMA_CommonData structure should

be changed to wCurrInVol for the left and right channels respectively.

These values are used to control the monitoring level heard while

recording.

pg. 43

Under wCurrInputTagBits the input value table uses FS as a reference

level.  This stands for Full Sample.  It is the highest quantization

value for the converters.  These bits can be used to track how much a

signal is clipping by.

pg. 56

The SMA memory map contains an error.  The following loacations should

be changed as shown below:

MODQ Buffer             0x6C60 - 0x745F

MIDQ Buffer             0x7460 - 0x785F

DSPQ Buffer             0x7860 - 0x7DFF

pg. 57

The record input monitoring variables should be added to the SMA after

the play values.  The new section should look as shown below:

wCurrPlayBytes              0x7F40

wCurrRecordBytes            0x7F42

wCurrPlayVolLeft            0x7F44

wCurrPlayVolRight           0x7F46

wCurrInVolLeft              0x7F48

wCurrInVolRight             0x7F4A