ATI_16M.GRN

Driver for DJGPP with LIBGRX

Version 0.31	(C) 1994 Bernhard Schwall

This is a driver for the ATI Graphics Ultra Plus/Pro to use with the GNU

DJGPP compiler for DOS. To use it set your GO32 environment variable to

ATI_16M.GRN, e.g.

SET GO32=ansi nodpmi d:\djgpp\drivers\ati_16m.grn gw 800 gh 600 nc 256

There are now two compiled versions of the driver included:

ati_16m.grn   for use with your own program

ati_16md.grn  for use with disp180

(For differences see below)

MODES:

------

The following graphicmodes are supported by the driver:

X-Res	Y-Res	number of colors	color setting for Initialisation

320	200	   16			   16

640	200	   16			   16

640	350	   16			   16

640	480	   16			   16

800	600	   16			   16

1024	768	   16			   16

320	200	  256			  256

640	400	  256			  256

640	480	  256			  256

800	600	  256			  256

1024	768	  256			  256

640	480	32768			32768

800	600	32768			32768

1024	768	32768			32768

640	480	65536			0xc000+16	(=49168)

800	600	65536			0xc000+16	(=49168)

1024	768	65536			0xc000+16	(=49168)

640	480	16.7 mio.		0xc000+24	(=49176)

800	600	16.7 mio.		0xc000+24	(=49176)

The last three modes are only supported by cards with 2 MB-Ram (I think).

Because I don't use any routine of the LIBGRX graphics-library exept the

switching to text and graphics-mode I don't know if any routine of this

package will run with the high- and truecolor modes. But you can access the

video-memory direct. The 16 and 256 color modes work as with the normal

ATI.GRD driver.

MEMORY-ALIGNMENT:

-----------------

The memory is mapped to two different places in the virtual memory-space. If

you can use memory-aperutre you can access the memory directly at the mapping-

address. This address can be set direct in the driver (see below). E.g. if

you have set the aperture to MB 9 you can access the memory at address

0xe0900000

The other way to access the memory is to use the swapping-mechanism of the

LIBGRX grapfics library (or GO32). You have to use a GO32 version 1.11 or

above (tested only with 1.11.maint3). There the memory is mapped to 0xd1000000

for read and write.

So just set a 'char' pointer to this address and access the memory direct.

(see below)

If you use a mode which requires only 1MB you can also use the 'old' memory-

window at 0xd0000000. This will run with GO32 version 1.10a and above (tested

with 1.10a and vmpeg)

In modes with 15 or 16 bit (32768 and 65536 colors) each line is twice the

horizontal resolution in byte (e.g. for 640x480: 1280 byte). Two byte

represent the color-tripple as followd:

15 bit:	-rrrrrgggggbbbbb	bit 0-4 blue, 5-9 green, 10-14 red, 15 unused

16 bit:	rrrrrggggggbbbbb	bit 0-4 blue, 5-10 green, 11-15 red

In truecolor-modes each line is 3 times the horizontal resolution (e.g. for

800x600: 2400 byte/line).

The colors are represented by 3 bytes in BGR-order. So if you write to memory

first write the blue, then the green and at last the red value to the memory.

(see example)

PROGRAMMING:

------------

To switch to the graphics-mode use the command

GrSetMode(GR_width_height_color_graphics,800,600,0xc000+24);

Set the last three parameters to the values you wish (see table above for

parameters).

To switch back to the textmode use the command

GrSetMode(GR_default_text);

If your program crashes while a high- or truecolor-mode is set you must reset

your computer because the DOS-command 'mode co80' can't set the card back to

the VGA mode.

Because LIBGRX was designed to run with a maximum of 32768 colors (15 bit)

you can't pass the values for 16 and 24 bit via the GO32 setting. For

switching into 16 or 24 bit modes you must set the values directly into the

GrSetMode(...) command.

Here is a short example how to access the video memory:

#include <grx.h>

void main(){

char *memory;

long x,y;

/* switch to mode 800x600x16.7mio */

GrSetMode(GR_width_height_color_graphics,800,600,0xc000+24);

for (y=0; y<600; y++){

memory = (char *)0xd1000000+(3l*800l*y);

/*

if you can use memory-aperture use: (here for aperture at MB 9)

memory = (char *)0xe0900000+(3l*800l*y);

*/

for (x=0; x<800; x++){

*memory++ = y % 256;	/* blue */

*memory++ = x % 256;	/* green */

*memory++ = (x+y) % 256;	/* red */

}

}

GrSetMode(GR_default_text);

}

CONFIGURING THE DRIVER:

-----------------------

There is only one point you can change in the driver. This is the address for

the memory-aperture. In line 263 you can enable or disable the aperture by

setting AL to 0x02 (enable) or 0x00 (disable). If you enable it you can set

the address in line 265 in AH (from 0-127, on ISA only 0-12). I don't know

what will happen if you set the address into your normal RAM and access it

via 0xd1000000. But I'm shure if you try to access your videomemory through

e.g. 0xe0100000 your system will crash.

The default settings are:

memory-aperture disabled but set to address MB 9 as example so you can use it

with all boards.

After changing the values you must recompile the driver with TASM (or MASM,

I think), then link it and make it an obj, e.g.

TASM ATI_16M.ASM

TLINK			(ignore the warning 'NO STACK')

EXE2BIN ATI_16M.EXE ATI_16M.GRN

Now the driver should run on your system.

The driver is configured to be used with your own program. If you want to use

it with other programs written for 16 or 24 bit modes perhaps you have to

change the driver, 15 bit modes should run with the standard settings.

E.g. if you want to use the driver with disp180 with 16 or 24 bit modes you

must comment out line 275 and compile the driver. But this drivers wouln't run

with your programs (with LIBGRX 1.03) in 16 and 24 bit modes. If you try to

switch in those modes you will get the error-message 'bad color plane # in

driver' and the program will stop immediately.

The reason for this problem is that LIBGRX was designed for a maximum of 15

colorplanes and it will reject all modes with more than 15 planes. For disp180

LIBGRX was changed (I think) to accept this modes and it uses the # of planes

value returned by the driver to validate the correctness of the videomode.

So you have to support two driver if you want to use disp180 with truecolor

and also want to write your own programs for the drivers.

Another possibility is to change the sourcecode of LIBGRX to support 16 and

24 bit modes. For LIBGRX 1.03 you have to change the file 'setmode.c' and

porhaps some other files to make the driver run with it. Caution: if you do so

notice that the file 'grdriver.inc' shipped with LIBGRX is different to the

file used by disp180 (and by the driver).

ABOUT THE DRIVER:

-----------------

The driver is released without any warranty. It was originally made only for

my own use but I thought it would be usefull for others too. Feel free to send

your comments, hints, changes or reports about this driver. If you have any

problem or question please contact me so I will try to help you.

The driver was tested with disp180 and seems to work in all resolutions.

With a patch it will also work with vmpeg 1.1 but then it will no longer run

with disp180 because vmpeg expects the number of byte per line as horizontal

resolution.

Because 24 bit display with vmpeg is slower then 8 bit display I removed the

patch so you can use the driver with disp180.

Caution: because the programming of 15, 16 and 24 bit modes seems not to be

standardized you should always use the go32 version shipped with the program

you want to use.

AUTHOR:

-------

Bernhard Schwall

Broichstr.56

D-53227 Bonn

e-mail: schwall@athene.informatik.uni-bonn.de