USING JAWS WITH TERMINAL EMULATORS

1/90:

IBM 3270 Adaptor Card

If you have the IBM 3270 Adaptor card, such as found in the PS2

computers, JAWS version 1.xx will not work properly with the IBM

software.  IBM has a nasty habit, perhaps policy, of

circumventing the interrupt "hooks" built into the operating

system.  However, the Avatar PA100G software will work fine with

the IBM hardware.  This solution is being used by several JAWS

users.  The price of the "Entry Level" Avatar software is

currently $100 plus shipping and handling, and it can be ordered

through Henter-Joyce.

Pe-Cox from CXI

This 3270 emulator works fine with JAWS, it is available in

Micro-Channel and PC-AT versions.

The Pe-Cox 1 software with the CXI coax board is compatible with

Irma file transfer software (FTS), according to CXI.

"PC789" by Memorex-Telex, a 3278 emulator, works fine with Jaws,

12/6/89,  from Peter Kantasani.  Load Jaws first, then PC789.  It

may not work if the PC789 is loaded first.

THE AVATAR PA100G, 1/89

When using JAWS and the Synphonix 200 synthesizer or the VoTalker

with the Avatar PA100g for IBM 3270 emulation you must load the

PA100g first, then Sonix and TTS and then JAWS.

You should have a batch file that does the following:

PA3270/h

sonix/i=#

tts

JAWS S200

The "/h" after the PA3270 says load into memory, exit to DOS, and

stay resident.

The "#" in the Sonix refers to the IRQ interrupt line being used

(2, 3, or 7 usually).

If you have a demo JAWS change that last line to "JAWSDEMO S200".

You should also have "device=hotkey.sys" in your config.sys file.

The file named "hotkey.sys" from the Avatar disk should be in

your root directory, or someplace where config.sys can find it.

When all is done correctly a double shift (press both shift keys

at the same time) will invoke the Avatar emulator session.

Pressing double shift again will re-invoke DOS.  This is the hot

key, it can bounce you between a main frame session and a DOS

application session.

If JAWS is used with a different synthesizer it can usually be

loaded first, then the emulator can be run.  This has the

advantage of leaving the Avatar utilities available to the blind

user.  These include the key board map editor, the environment

editor, and macro editor.  If the emulator is loaded first, as it

must be with the Symphonix or VoTalker, the Avatar utilities are

not available.

The Avatar will not display more than 80 characters per line.  If

you need to  have more than 80 characters on the screen it will

scroll horizontally.

Idacom 3278 emulator:

Version 2.0 and 3.x

Your version may not accept alt-insert or alt-delete, so we must

use other keys to perform the insert and delete functions.

Execute the program named "CFIG" which comes on the Idacom disk.

This program allows the user to re-configure the keyboard.

Change the insert and delete functions to a different key

combination, such as alt-I and alt-L.

The Wang WLOC (Wang Local Office Connection), AT model, works

fine with JAWS.  However the XT model may not work with JAWS.

WLOC is a plug-in board and software similar to the Avatar 3270

emulator.  It connects to the VS100 mainframe via a coaxial

connector and a screw-on connector from the same coaxial cable.

(Actually 2 cables bound together).

The VS100 runs Wang Office, which has E-Mail, Word Processing,

Bulletin board, and user applications.  The Word processor puts

up a graphic 250 character whenever the user presses the space

bar.  This appears as a little dot between the words, later

versions of JAWS can handle this fine.

IBM SYSTEM 3X EMULATORS:

A product named Access 36 from Mux Labs has been tested and works

fine with JAWS.  This is a 5250 terminal emulator, consists of a

plug-in card and software, connects to the System 34-36-38 via a

twin axial cable just like a terminal.

The "Smart Aleck" from DCA is another System 3x terminal package

that will work fine with JAWS, but only if you can get version

1.5 or earlier.  The version 1.7 does not work.

The Mux Labs 5250 emulator works fine with Jaws.  Call us for

more info and prices on this product.  We have the manual on disk

also.  This emulator works on IBM System 36-38 and the AS400.