LPT I/O V1.5

by John Augustine

LPT (parallel port) input/output (digital data acquisition, etc.) was

written in Assembly for MS Dos systems, uses very little memory, and requires

no "installation".

This program is NOT for those who know little or nothing about the parallel

port.  Some applications for using LPT will probably require some type of

interface, the simplest of which can be an isolation or load resistor such as

a 10k resistor to allow you to view the patterns on a scope (example).  A

little knowledge about electronics would be very helpful.

PLEASE NOTE:

This is NOT a "free" program. See "CONCLUSION" near the end of this document

for details on making a small "donation" and becoming a "donor".

TIP:

If you have a second (older) PC, use it with this program to experiment.

LPT USES:

There are many possible uses for LPT which I have not thought about at this

time.  LPT allows you to capture (Record) into memory, digital data (TTL level

(0 or +5V) input) via pin 12 (parallel port), normally used for out of paper

detection (printer).  Afterwards, you can view it in binary format (two modes)

or save it to a file as reference, or for "playback" whenever you need to

simulate a digital source.  The digital data can be transmitted ("playback")

via all the data pins (pins 2-9) in the parallel port, in order to simulate

the digital data source.  Note: Only one of those data pins is normally

needed, unless you have a special use which requires more than one line/pin

(up to 8).

With the built in binary editor, you can create your own digital data with

specific patterns to transmit out of the parallel port, or modify an existing

pattern/s that you captured earlier.

You can simulate a digital data sequence for testing purposes, etc.

Another simple example for using LPT is to Record the patterns of a remote

control using a simple inexpensive IR receiver which you can buy at Radio

Shack.

You can use LPT to check your parallel port for defects (example if a

data pin is "stuck" at one level).

GETTING STARTED:

Simply type (without quotes) "LPT" and press Enter

OR

If you want to view a digital data file or simulate a digital data source

(from data saved to a file):

Type (without quotes) "LPT FILENAME" and press Enter.  "FILENAME" represents

the name of the data file.  Doing this presets the filename for Loading.

VIEWING/EDITING DIGITAL DATA:

You can view (and edit) data in two binary modes.  This allows you to see

the bits sequence (digital data) which starts at the top (left) row (moving to

right).  One mode displays in "1" and "0" format.  If you press the "S"

(switch) key, another format is displayed, which is better suited for seeing

patterns.  To change a bit/s, simply moving the blinking (cursor) to where you

want to start editing.  Press (or hold down) the "1" or "0" key to overwrite

the existing bit/s.  Pressing the "U" or "D" key moves the binary reference

bar/scale up and down so you can pin point bit/s.  Other commands allow you to

move throughout the data buffer.  Note: If you did not record or load a data

file, what you see is "residue" from a previous application, etc.

LPT NOTES:

An option allows you to "clear" (fill with binary 1s or 0s) the entire data

buffer.

Within the "Record, Playback..." function, is the option to "Set Delay...".

The effect of this delay will vary with PCs and their CPU speed.  The delay

setting determines how long to wait before taking a sample (bit) in "Record"

mode or how long to wait before transmitting the next bit during "playback".

Note: This delay number should be the same for Playback as it was for

recording, in order to simulate the original digital data.  On the other hand,

you can slow down or speed up the digital data if you prefer.

The length in time, digital data can be recorded, depends on the buffer

size, which will vary some from PC to PC, the CPU speed, and the delay

setting.  Note: Only "conventional" memory is used for storage due to time

balancing in the Record or Playback procedures.  Note: Pressing Esc during

recording or playback will abort that function.

SQUARE WAVE GENERATOR:

An adjustable square wave generator is one of the features in LPT.  You can

use a scope to view it, but I suggest you put a 10K resistor between one of

the data pins (2 - 9) and ground.  Put your scope (or other) across that

resistor.

"TEST PIN 12 (INPUT) FOR ACTIVITY":

This was a feature I added for certain tests I was making.  It will count

the number of transitions from one state to the other that occurs within

1/18th of a second.

PEEK OR POKE:

This function allows you to "peek" or "poke" the Data and Status ports for

the parallel port.  This is useful for testing purposes.

CONCLUSION:

This is NOT a "free" program.  A one time donation is expected.  You choose

the amount ($5.00 minimum).  For your donation, you will become a "donor".

Your name, etc. will be added to the "donors" list.  As a "donor" (one time

donation), you can select (ask for) one or more programs to receive FREE such

as "Tree Restore", etc.  See (use "VIEW-DOC") "OTH-PGMS.DOC" for information

about other programs.  Please include your e-mail address with your donation,

and the name/s of the other program/s you want (if any).  I will tell you in

the email message about the attachment (programs.zip) so you know it is safe

to open.  As a "donor", you will be entitled to receive all improved and new

programs (at your request) developed in the future (sent to you by e-mail).

As a "donor", you may request (from time to time) the latest list of programs

from which you can select to receive FREE.  If you know another shareware

author with a better offer, please let me know.

The donation will motivate me to develop more programs and make improvements

in existing programs (add new features).

Comments or complaints are always welcomed via email.  If you use "snail

mail" and expect a reply, please include a S.A.S.E.

If you are overseas, the local bank charges a $10.00 service fee for checks

outside of U.S. I recommend that you send U.S. cash instead.

Send $5.00 (or more) check, money order, or cash (put in foil for security)

to:

John Augustine N3AOF

3129 Earl St.

Laureldale, Pa 19605

E-Mail:  jaugust@bellatlantic.net