**************************************************************************

RS232

Version  1.05

Ring  Development

Maple Grove,  Minnesota

ringx004@tc.umn.edu

**************************************************************************

****                      This software is SHAREWARE                  ****

**************************************************************************

The files created during extraction are as follows :

FILE                        SIZE

--------------------------------

RS232.    DOC              35230

RS232.    EXE              16020

RS232A.   EXE               7920

XT.       EXE               3720

TY.       EXE              14620

TYBIG.    EXE               9240

ADDLINES. EXE               6525

READ.     ME                3240

ORDER.    FRM               1200

**************************************************************************

TABLE  OF  CONTENTS

-------------------

1    Introduction

1.1  Program Installation

2    RS-232 Basics

3    RS232 Program Usage

3.1  Cabling to PC

3.2  Theory of Operation

3.3  User Interface

3.4  Command Line Arguments

3.5  If you have a real slow PC and/or fast baud rate - Overrun Error

4    Utility Program Usage

4.1  RS232A

4.2  XT

4.3  TY

4.4  TYBIG

4.5  ADDLINES

5    Closing

**************************************************************************

1    INTRODUCTION

The RS232 program here should enable you to debug most of your

RS-232 problems much faster than before. With the program, you

can actually see the data transmitted in both directions on a real

time basis. The program also supports the monitoring of all hardware

lines for those nasty hardware handshaking problems.

5 utility programs are also provided to aid in wading through the

data generated. These utilities are described in Section 4.

There are no hardware changes needed in the PC, the program does

all the monitoring necessary through the 2 COM ports. The program

can be used if your PC has only 1 COM port, though the program

will give the most beneficial information if you have 2. Nowadays,

a second COM port can be added to any PC for a minimum investment.

The hardware needed to use this program is a simple homemade cable

that you create. The cable you need can be probably be built in an

hour from spare parts lying around. Section 3.1 discusses the cable

that most likely will be the one you need.

1.1  PROGRAM INSTALLATION

Installing the RS232 program and the associated utilities is quite

simple. It is simply a matter of copying the .EXE files to any

directory where you want to run the program(s). A more versatile

directory would be a directory that is already in the 'PATH' of

the PC. This enables operation of the program(s) from any place

on the disk. A typical directory in the PATH is \DOS.

To copy the program(s) to the \DOS directory :

1) COPY *.EXE \DOS        (copy files to \DOS)

To copy the program(s) to a new directory (ex \RS232) :

1) MD \RS232              (make RS232 directory)

3) COPY *.EXE \RS232      (copy files to \RS232)

**************************************************************************

2    RS-232 BASICS

RS-232 is a communication standard that has existed for many

years. It's main purpose is to transfer data from one device

to another. For instance, a tester to a host computer so the

computer can store the test results on disk.

The standard RS-232 connector has 25 pins, though a lot of

PC's have a 9 pin connector. The pin assignments for each

signal is as follows (for 25 and 9 pin connectors) :

TABLE  I

SIGNAL                      PIN # (25)   PIN # (9)     IN/OUT

-------------------------------------------------------------

Ground                          1           N/A          --

Transmit Data                   2            3           Out

Receive Data                    3            2           In

Request to Send     (RTS)       4            7           Out

Clear to Send       (CTS)       5            8           In

Data Set Ready      (DSR)       6            6           In

Signal Ground                   7            5           --

Data Carrier Detect (DCD)       8            1           In

Data Terminal Ready (DTR)      20            4           Out

Ring Indicator      (RI)       22            9           In

Note that besides receive/transmit data, there are 2 output

lines and 4 input lines. These are used for modem control and

hardware handshaking, which will be discussed later.

The simplest communication hookup has 3 wires; transmit data,

receive data, and signal ground. The transmit and receive lines

need to be crossed (whether in the connecting cable or internal

to the either device). See Figure 1. This is so the transmit

line of device 1 is tied to the receive line of device 2, and

vice versa.

This configuration only supports software handshaking. Suppose

device 1 transmits a block of data to device 2. Once device 2 has

received the data, it transmits back to device 1. Device 2 can

transmit a character meaning "OK, send the next block of data" or

transmit a character meaning "I don't like the data, send that last

block of data again". The actual protocol used is determined by

the software running on both devices.

DEVICE 1 (25 Pin)                         DEVICE 2 (25 Pin)

PIN  2----------------\  /--------------------PIN  2     Transmit

\/

/\

PIN  3----------------/  \--------------------PIN  3     Receive

PIN  7----------------------------------------PIN  7     Signal GND

(FIGURE 1)

Another configuration uses 5 wires and this is used primarily

for a hardware handshaking protocol. It uses the 3 wires in

Figure 1 and also the 2 wires shown in Figure 2.

DEVICE 1 (25 Pin)                         DEVICE 2 (25 Pin)

PIN  4----------------\  /--------------------PIN  4     RTS

\/

/\

PIN  5----------------/  \--------------------PIN  5     CTS

(FIGURE 2)

From Table I above, you see that pin 4 (RTS) is an output signal

and pin 5 (CTS) is an input signal. No data is transferred through

these pins. They are just control lines using a specific voltage

level for the high state and another voltage level for the low

state. The software running in device 1 can program pin 4 to be

high or low and then device 2 can read this value as high/low

and act accordingly. Here is a typical example. Suppose device 1

is a large mainframe and device 2 is a tester of some sort. Now

assume that there are many testers (say 10) and all are hooked

up to our mainframe (one tester on 10 different mainframe COM

ports), our central data gathering point. It would be difficult

for our mainframe to gather data on all 10 testers simultaneously,

so we implement a hardware handshaking protocol. Our protocol says:

"A tester can not transmit it's test results until it sees the CTS

line go high". If the CTS line is low, our tester will not transmit

it's test results. So now, all our mainframe software has to do is:

1)  Set the RTS line on the port for tester 1 high, this sets

the CTS line high at tester 1.

2)  Tester 1 sees that CTS has gone high, then it starts its

transmission of the test results.

3)  After completion of the test results, set the RTS line low

for tester 1. This will inhibit tester 1 from further

transmissions.

4)  Now set the RTS line for tester 2, and repeat the above

steps for all 10 testers.

Pins 20 and 6 may also be used as pins 4 and 5 were above.

Another possible use for the input pins is to connect an

input pin to a +5V signal in the remote device (for example,

a power supply voltage). This is convenient if the remote

device is not always powered on. Your program in essence can

monitor the power supply of the remote device and know when

it is powered up and safe to 'talk' to.

Pins 8 (DCD) and 22 (RI) are normally used in communicating

with a modem. Data Carrier Detect is set high by the modem

when it is talking to another modem. Ring Indicator is set high

by the modem when there is an incoming call.

**************************************************************************

3    RS232 PROGRAM USAGE

3.1  CABLING TO PC

The cabling to the PC can be as simple or complex as the need

dictates. Refer to section 3.2 for more details. If your system

only has a 3 wire protocol, the cabling will be very simple.

The top 7 wires shown are all optional (for a 3 wire software

handshaking protocol).  The cable will be wired as follows

(all pin numbers assume a 25 pin connector, use TABLE I above

for 9 pin connector conversions):

NOTE : If you are debugging a hardware handshaking problem,

then obviously some of the 'optional' wires shown below

are actually necessary.

25 Pin MALE    ***STRAIGHT THROUGH CABLE***   25 Pin FEMALE

=======  OPTIONAL WIRES ========

CHASSIS GND

PIN  1----------------------------------------PIN  1

RTS

PIN  4----------------------------------------PIN  4

CTS

PIN  5----------------------------------------PIN  5

DSR

PIN  6----------------------------------------PIN  6

DCD

PIN  8----------------------------------------PIN  8

DTR

PIN 20----------------------------------------PIN  20

RI

PIN 22----------------------------------------PIN  22

=======  REQUIRED WIRES ========

XMIT

PIN  2---+------------------------------------PIN  2

|

TO                |                      REC                        TO

DEVICE     PIN  3-----------+----------------------------PIN  3     DEVICE

1                 |       |                                         2

|       |              SIGNAL GND

PIN  7-------+--------------------------------PIN  7

|   |   |

|   |   |

________|   |   |_________

|    ________|_________    |

|   |                  |   |

|PIN|                  |PIN|

3   7                  7   3

COM 1 of PC            COM 2 of PC

(FIGURE 3)

So with this cable, PC COM 1 will monitor the transmit line of

device 1 (receive line of device 2) and PC COM 2 will monitor

the receive line of device 1 (transmit line of device 2). The

cabling scheme just inserts a straight through cable installed

between the 2 devices under test and then tap off the desired

signals and run them into the PC. Note that both signal lines

are wired to the PC COM ports pin 3 (receive line). This is

because the PC is never an active participant in the communi-

cation. It is only there to monitor the communication.

The computer needs 2 COM ports so it can monitor both halves of

the communication. (This program can be used with only 1 COM

port in the PC, see section 3.2 below).

Now, it should be easy to extend the above cabling to include

the desired hardware lines as needed. If you need to monitor

the CTS line of device 1, just add another wire from device 1,

pin 5 (CTS) to PC COM 1, pin 5, and so on.

3.2  THEORY OF OPERATION

The theory behind this program is quite straight forward. The

purpose is to monitor the RS232 communication between 2 devices.

This is accomplished by inserting a cable in the line and

tapping off the desired signals and connecting them to the PC.

See section 3.1. The PC only 'listens', it never transmits any

data. It only monitors what is coming across the cable, in both

directions. For optimum benefit a PC with 2 COM ports is needed,

though you can use a PC with only 1 COM port. Also, each device

has 4 hardware lines and all 4 lines for both devices can be

monitored if your cable has the lines hooked up to the PC. If your

PC only has 1 COM port, you can only monitor one half of the

conversation. However, if you have 2 PC's and they each have 1 COM

port, you can use 1 PC to monitor the transmit line and the other

PC to monitor the receive line, though you will lose the timing

information. The timing of the communication can be quite useful.

See section 4.5 below. This is a portion of an actual Kermit file

transfer between 2 computers. PC COM 1 was listening to the computer

that was initiating the Kermit session and PC COM 2 was listening

to the computer that was being put in server mode. The computer on

COM 1 was invoking Kermit with the command ">SYSLIB2>KERMIT SV".

As you can see in the actual transmission timing :

Computer 1 : ">S"

Computer 2 : echos a 7F

Computer 1 : "YS"

Computer 2 : echos a ">"

Computer 1 : "L"

Computer 2 : echos a 7F

Computer 1 : "I"

Computer 2 : echos a "S"

Computer 1 : "B"

Computer 2 : echos a 7F

Computer 1 : "2"

Computer 2 : echos a "Y", etc.

You can see that the second computer is echoing back the command

it is receiving, with a few stray 7F characters embedding in

the data stream. The format of the data in section 4.5 is

exactly what will be logged to disk on a write data command.

The raw hex values will be on the left side and the ASCII

equivalents will be on the right side. If a data character is

a non ASCII character, it will be displayed as a period. They

will appear on the PC's display this way also.

3.3  USER INTERFACE

To start the program, just enter RS232. The optional command line

arguments are discussed in section 3.4 and can also be seen by

entering 'RS232 -H'.

If the first thing displayed is 'Bad COM ports specified', the

likely cause is that your PC has only 1 COM port. The program

assumes that you are using 2 COM ports. You will need to use

the '-1' command line argument to use the program with this PC.

See section 3.2.

The display is split up into 2 areas, the top 20 lines of the

screen and the bottom 5. The top 20 lines is where the captured

data is displayed. They are shown in alternating colors of blue

and light blue for a color monitor or normal video and reverse

video for a monochrome monitor. See below for more discussion of

these top 20 lines.

The bottom 5 lines of the display is split up into 4 smaller areas.

The two sections on the left show available function keys for

performing specific tasks. The 8 function keys supported are :

F1  : Help    (Displays help screen)

F4  : Goto    (Go directly to a specific position in the read buffer)

F7  : Srch    (Search the read buffer for specific data)

F10 : EXIT    (Exit RS232 program, must confirm exit with 'Y')

Alt-F1  : Gather Data                       (Monitor ports for data)

Alt-F4  : Gather Data and Clear Read Buffer (Monitor ports for data)

Alt-F7  : Write the read buffer to disk.

Alt-F10 : Write the read buffer to disk and then Exit.

The next section on the left shows the present configuration of the

COM ports. The default setup is 9600 baud, 8 bit characters, no

parity and 1 stop bit. These can changed with the command line

arguments discussed in section 3.4.

The last section on the bottom displays the total read buffer

available in bytes, the number of bytes used, and the percentage.

Initially, the total read buffer is typically 450 - 500K in size

(on a 640K machine) with 0 bytes used and 0 %. After the program

loads into memory, it allocates all remaining conventional memory

as a read buffer. This behavior can be changed by using command line

argument '-K' discussed in section 3.4 below.

FUNCTION KEYS :

F1  -  HELP      Displays short help screen

F4  -  GOTO      This will let you jump around the read buffer

very easily. For example, if you have 350K used

in your read buffer, it is very slow to 'PG DOWN'

from beginning to end. After pressing F4, you

will get prompted for an offset. Enter the desired

offset from the beginning of the buffer, example :

215400. The number you enter will be rounded down

to the nearest multiple of 16.

F7  -  SRCH      This will let you search the read buffer for

specific data. You can search for ASCII text or

a hex pattern. An example of an ASCII text search

would be 'KERMIT FILE TRANSFER'. An example of a hex

pattern search would be '0120204A0D0A'. The hex

search above would search the read buffer for the

6 data bytes 01, 20, 20, 4A, 0D, 0A. NOTE! : The

search takes place from the present display position

to the end of the read buffer. After a successful

search, pressing 'ALT N' will find the next occur-

rence of the search, from the new display position

to the end of the read buffer.

Pressing 'ALT A' will prompt directly for an ASCII

text search and pressing 'ALT H' will prompt

directly for a hex pattern search.

The search does not distinguish between COM 1 and

COM 2, the data is treated as one long buffer.

For a text search, the search is case sensitive.

'FILE' is different than 'file'.

For a hex pattern search, the program only accepts

the characters 0-9, A-F (and also a-f). All other

characters are not allowed and the program will

tell you that there is an illegal character in the

string.

F10 -  EXIT      Press this to exit RS232. You will be asked 'Are

you sure ? (Y/N)'.  Press 'Y' to confirm the exit.

Any key other than 'Y' is treated as 'N'.

ALT-F1  -  GATHER DATA

This puts the PC in the mode to actually capture

data. If there is already data in the read buffer,

any new data is appended to the end of the present

data. Pressing 'ESC' will discontinue 'gather data'

mode. If the blinking 'PRESS ESC' is annoying,

pressing any other key will turn off the blinking.

In this mode, the bottom left portion of the

display has changed to indicate the status of both

COM ports.  The bottom left portion of the screen

now looks something like :

--------------------------------------------------

1  CTS : L   PAR : -     2  CTS : L   PAR : -

DSR : L   FRM : -        DSR : L   FRM : -

DCD : L                  DCD : L

RI : L                   RI : L

This is showing the status of the 4 hardware lines

CTS, DSR, DCD, and RI for both COM ports. If the

line is low, you will see a normal video 'L'. If

the line is high, you will see a reverse video 'H'.

This is also showing whether a parity (PAR) or

framing (FRM) error has happened on either port.

If no error has occurred, you will see a normal

video '-'. If a parity/framing error has occurred,

you will see a reverse video block in place of the

'-' and also a 'latched' solid block 1 space to the

right of the normal display.

The hardware lines and the parity/framing errors

are updated continuously.

ALT-F4  -  GATHER DATA AND CLEAR READ BUFFER

This is the same as ALT-F1, except that if there is

any data in the read buffer, the old data in the

read buffer is discarded.

ALT-F7  -  WRITE BUFFER TO DISK

This will write the present read buffer to disk.

The default file name is 'RS232.LOG'. If 'RS232.LOG'

already exists on disk, then the old file contents

will be lost and overwritten with the new data.

The default file name can be changed by using the

'-F' command line argument or by pressing the

'ALT F' key. Pressing 'ALT F' will prompt you for

a new filename. A full drive:\pathname\filename

can be used, if desired.

ALT-F10 -  WRITE BUFFER TO DISK AND EXIT

After writing the read buffer to disk, RS232 will

terminate.

OTHER 'ALT' KEYS :

ALT 'A'          Prompt directly for an ASCII text search.

ALT 'F'          Change the default filename for logging data.

ALT 'H'          Prompt directly for a hex pattern search.

ALT 'L'          Toggle between upper/lower case for hex values

in the display.

ALT 'N'          Search for the next occurence of a previously

successful search.

As mentioned above, the top 20 lines of the screen is where the

captured data is displayed. At most, 160 bytes of the read buffer

can be shown at a given time. The screen is broken down into 10

sets of 2 lines each. These 2 lines (top for COM 1, next for COM 2)

will show 16 bytes of the read buffer. The data will be on the top

line if it came in COM 1 or the second line if it came in on COM 2.

From the snapshot shown below, COM 1 was receiving 2E, 61, 73, 59,

and 0D (line a). Then COM 2 started to get 7F, 01, 2A, etc up to

23 (line b). COM 2 was still receiving the data 26, 3B, 0D, 00 (line

d). And finally, COM 1 received the data 01, 30, 20, etc (line c).

(a)      2E 61 73 59 0D                                   1  .asY.

(b)                     7F 01 2A 20 53 7E 2A 20 40 2D 23  2       ..* S~* @-#

(c)                  01 30 20 59 7A 2A 20 5C 40 2D 23 26  1      .0 Yz* \@-#&

(d)      26 3B 0D 00                                      2  &;..

Every 2 lines in the display make up real time, left to right. There

can never be 2 characters above each other in the display for COM 1

and COM 2 in the same time period. This is not happening above. Lines

a & b make up 1 time period while lines c & d make up the next time

period.  The example below could never happen.

|| ||    ||

(e)      2E 61 73 59 0D             20 41    43           1  .asY.     A C

(f)                     7F 01 2A 20 53 7E 2A 20 40 2D 23  2       ..* S~* @-#

(g)                  01 30 20 59 7A 2A 20 5C 40 2D 23 26  1      .0 Yz* \@-#&

(h)      26 3B 0D 00                                      2  &;..

When you are gathering data, nothing else can be done until you

press ESC to end the gather data session. The PC is dedicated to

monitoring both COM ports for activity. For example, you cannot

write the read buffer to disk while in gather data mode. The display

is updated continually with new data as it comes in. When ESC is

pressed to end 'gather data', then you can write the buffer to disk,

search for data, etc. When you are not gathering data, any incoming

data to the PC is not collected.

You can scroll through the read buffer by using the HOME, END, PG UP,

PG DOWN, and UP/DOWN arrows. Pressing the HOME key will move you to

the beginning of the read buffer (offset 0). Pressing the END key

will move you to the end of the read buffer. Pressing the PG UP /

PG DOWN keys will move you up/down 160 bytes in the buffer, and the

UP/DOWN arrows will move you up/down 16 bytes in the buffer.

3.4  COMMAND LINE ARGUMENTS

The command line arguments may use the '-' or '/' character,

be upper or lowercase, and must be separated by spaces. The

recognized arguments are:

-1           :   Only use 1 COM port for monitoring data. See

section 3.2 for more details.

-A           :   Mask off the most significant bit of all

incoming data.

-B<number>   :   Where <number> is any legal baud rate. The default

baud rate is 9600. The maximum baud rate supported

is 115200. If an illegal baud rate is entered, the

baud rate used is the closest one supported by the

system (PC).

-C<number>   :   Where <number> can be 5, 6, 7, or 8. This is

the of number of bits per character. The default

is 8.

-D           :   Disable screen update during 'gather data'. Use

this option if you ever see a blinking "OVERRUN

ERROR" message on the screen. (See section 3.5)

-F<text>     :   Where <text> is alternate file name for

writing the read buffer to the disk. The

default file name is 'RS232.LOG'.

-G           :   Go directly into gather data mode. This may be

useful if invoking RS232 from a .BAT file.

-H           :   Display the command line arguments.

-K<number>   :   Where <number> is the number of K-bytes of RAM

NOT to allocate as a read buffer. The default

is to allocate all remaining conventional memory

as a read buffer. This may be undesirable if you

have a TSR that requests some RAM when invoked.

EX : -K64 will leave 64K of RAM free.

-M<2 numbers>:   Where the two numbers are which 2 COM ports to

use. The possible options are 1, 2, 3, or 4. The

default is to use COM 1 and 2.

EX: -M13 to use COM 1 and COM 3.

-P<letter>   :   Where the possible letters are N, E, O, M, S.

This sets the parity to None, Even, Odd, Mark,

or Space. The default is None.

-S<number>   :   Where <number> can be 1 or 2. This is the number

of stop bits. The default is 1.

Example      :   RS232 -b19200 -pe -c7 -fdata.txt

3.5  IF YOU HAVE A REAL SLOW PC AND/OR FAST BAUD RATE - OVERRUN ERROR

When the RS232 program is in 'gather data' mode, you may see

a blinking "OVERRUN ERROR" message. This happens when the PC

can not keep up the with data transfer taking place. The data

transfer shown in section 4.5 was captured at 19200 baud with

no problems on a 6 MHz AT PC. However, if you try to capture

115200 baud on a 4.77 MHz XT, you may have a problem.

Here's a general look at the processing performed:

LOOP FOREVER

scan keyboard                   (looking for ESC key to break out)

for both COM1 and COM2:

update status of 4 hardware lines    (high or low)

update parity and framing errors

check for overrun error

has a character arrived at either port ?

if so

get the character

store character in read buffer

store whether it came in on COM 1 or COM 2

update the screen with the new data received  (***)

end if

end for

END LOOP FOREVER

There are basically four ways to correct the "Overrun Error":

(In order of preference)

1) Disable most if not all TSRs that are running. These steal

precious clock cycles from the main program. SMARTDRV is an

especially bad one. SMARTDRV is commonly used with Windows

to speed up disk reads/writes, but is can take over control

of the CPU for hundreds of milliseconds which is just too long

for practically any baud rate that we are trying to capture.

2) Disable screen updating by using the -D command line option.

See *** above. Updating the screen is actually a rather

time consuming task, relative to the other processing.

3) Slow down the baud rate. Not at all unreasonable. For example,

you could debug a 38400 baud problem at 9600 baud and probably

be safe. This logic would only fail if you were debugging long

distance communications.

4) Buy a faster PC

**************************************************************************

4    UTILITY PROGRAM USAGE

4.1  RS232A.EXE

This program simply displays the current settings (how each

COM port is configured) for all 4 COM ports on a PC.  If a

given port doesn't exist, then 'NONE' is displayed for that port.

To run, just type 'RS232A' and return. A typical output is as

follows:

COM1:   0x03F8   9600 Baud   8 Bit Chars   1 Stop Bits   No Parity

COM2:   0x02F8   9600 Baud   8 Bit Chars   1 Stop Bits   No Parity

COM3:   None

COM4:   None

So this PC has 2 COM ports and both are configured for 9600,8,1,N.

The '0x03F8' is the base address of the port in the computer's

memory. The '0x' is a C programming convention for a hexadecimal

constant, so the base address of COM1 is 3F8 hex and the base

address of COM2 is 2F8 hex. These addresses are read from the

BIOS data area at 40h:0000h.

4.2  XT.EXE

When RS232 logs data to the disk, the data file can be quite

large. A 500K byte read buffer in the program can result in

a 4.3M data file. This size file is usually impractical to

work with, so 'XT' will extract certain portions of the file

(actually, this program can be used on any ASCII file). 'XT'

requires 4 arguments on the command line:

1)  Source file name

2)  Destination file name

3)  Starting line number

4)  Ending line number

Entering less than 4 arguments will result in the following

display output:

Usage  : XT <source-file> <destination-file> <start-line> <end-line>

A '+' as the first char of the destination file will append

Example: XT DATA.TXT TEMP.TXT 1091 1584

Let's assume you have a very large 'RS232.LOG' data file but you

are only interested in lines 1211 through 1600 (See TY, TYBIG).

Use program XT to extract the desired lines as follows:

XT RS232.LOG RS232.TMP 1211 1600 <Return>

The program will then create file 'RS232.TMP' with the desired

lines from the original file. The original file is unchanged. If

the new file name already exists, the old file contents are lost,

unless the destination file has the read-only attribute set.

4.3  TY.EXE

'TY' is a utility to look at any ASCII file on the computer's

display. The usage is TY <filename | wildcards>. To view

RS232.LOG, simply enter 'TY RS232.LOG' <return>. TY *.* will

read the current directory and you choose which file to view.

Press function key F1 for help. In this release, TY has numerous

improvements. Some of the features are search capability, scroll

left/right, and go to any line number instantly. Also displayed

is the current line number and the total number of lines in the

file. This program attempts to load the entire file into memory,

for the fastest possible viewing. TY will also use any extended

memory that is available. If the file size exceeds the available

RAM in the computer, the program will exit with an out of memory

error. If this error occurs, use program TYBIG (see below).

4.4  TYBIG.EXE

This program looks similar to TY, except that it is used for

viewing very large files. This program has to perform disk I/O

for some operations, and therefore is somewhat slower than TY.

Usage is TYBIG <filename>.

4.5  ADDLINES.EXE

This program is used to alter the appearance of the 'RS232.LOG'

data file. Using this program on the data file makes it somewhat

more legible. The original data file is unchanged. Usage is

ADDLINES <Source-file> <Destination-file>. If the destination file

already exists the old file contents will be lost, unless the

destination file has the read-only attribute set.

Example : ADDLINES RS232.LOG RS232.TXT <Return>.

If the original RS232.LOG file looked something like this :

20 3E 53    59 53    4C    49    42    32    3E  1   >S YS L I B 2 >

7F       3E    7F    53    7F    59     2     .  > . S . Y

4B    45    52    4D 49    54    20    53     1   K E R MI T   S

7F    53    7F    4C       7F    49    7F    42  2  . S . L  . I . B

56    0D                                         1  V .

7F    3E 7F 45 7F 49 7F 20 7F 56 7F 0A 0D 7F  2   . >.E.I. .V....

1

4B 65 72 6D 69 74 2D 47 43 4F 53 20 56 33 2E 31  2  Kermit-GCOS V3.1

1

0A 0D 7F 53 65 72 76 65 72 20 6D 6F 64 65 0A 0D  2  ...Server mode..

01 30 20 49 7A 2A 20 5C 40 2D 23 26 31 7E 22 20  1  .0 Iz* \@-#&1~"

2

20 7A 5C 40 0D                                   1   z\@.

7F 01 29 20 59 7E 2A 20 40 2D 23  2       ..) Y~* @-#

01 3F 20 52 3E 75 64 64 3E 69 72 73 3E  1     .? R>udd>irs>

5D 0D 00                                         2  ]..

70 65 74 65 72 73 6F 6E 3E 73 74 65 70 72 6F 74  1  peterson>steprot

2

2E 61 73 59 0D                                   1  .asY.

7F 01 2A 20 53 7E 2A 20 40 2D 23  2       ..* S~* @-#

01 30 20 59 7A 2A 20 5C 40 2D 23 26  1      .0 Yz* \@-#&

26 3B 0D 00                                      2  &;..

then RS232.TXT would look like this :

20 3E 53    59 53    4C    49    42    32    3E  1   >S YS L I B 2 >

7F       3E    7F    53    7F    59     2     .  > . S . Y

--------------------------------------------------------------------

4B    45    52    4D 49    54    20    53     1   K E R MI T   S

7F    53    7F    4C       7F    49    7F    42  2  . S . L  . I . B

--------------------------------------------------------------------

56    0D                                         1  V .

7F    3E 7F 45 7F 49 7F 20 7F 56 7F 0A 0D 7F  2   . >.E.I. .V....

--------------------------------------------------------------------

1

4B 65 72 6D 69 74 2D 47 43 4F 53 20 56 33 2E 31  2  Kermit-GCOS V3.1

--------------------------------------------------------------------

1

0A 0D 7F 53 65 72 76 65 72 20 6D 6F 64 65 0A 0D  2  ...Server mode..

--------------------------------------------------------------------

01 30 20 49 7A 2A 20 5C 40 2D 23 26 31 7E 22 20  1  .0 Iz* \@-#&1~"

2

--------------------------------------------------------------------

20 7A 5C 40 0D                                   1   z\@.

7F 01 29 20 59 7E 2A 20 40 2D 23  2       ..) Y~* @-#

--------------------------------------------------------------------

01 3F 20 52 3E 75 64 64 3E 69 72 73 3E  1     .? R>udd>irs>

5D 0D 00                                         2  ]..

--------------------------------------------------------------------

70 65 74 65 72 73 6F 6E 3E 73 74 65 70 72 6F 74  1  peterson>steprot

2

--------------------------------------------------------------------

2E 61 73 59 0D                                   1  .asY.

7F 01 2A 20 53 7E 2A 20 40 2D 23  2       ..* S~* @-#

--------------------------------------------------------------------

01 30 20 59 7A 2A 20 5C 40 2D 23 26  1      .0 Yz* \@-#&

26 3B 0D 00                                      2  &;..

--------------------------------------------------------------------

**************************************************************************

5    CLOSING

If you have any questions or problems regarding the programs,

feel free to contact :

Michael Ring

Ring Development

10750 108th Ave. N.

Maple Grove, MN  55369

**************************************************************************