File:  CSLPKT.DOC

CSLPKT Ver 0.54    Copyright 1999  -  [totally] ������ Software

Usage:

=====

C:\> CSLPKT [/? ][/d ][/Option Value ][/Option Value ]...

Options [default:min..max]  (any order,  / = DOS SwitchChar)

/b - Baud rate divisor (dec) [-:1..65535]

/c - COM port (AT) [1;1..4]

/d - Drop DTR on termination [no]

/f - FIFO trigger level [4:1,4,8,14]

/h - Hardware handshaking (check CTS) [no]

/i - IP address for RARP [none]

/p - I/O Port base address (hex) [3F8:0..3F8]

/q - IRQ [4:3..15(AT),2..7(XT)]

/s - Speed in bps (dec) [-:2..115200]

/v - Interrupt Vector [60h:60h..80h]

/? - Help [no]

Numbers specified as:  either  0xNN, NNh (hex)  or  NNN (dec).

Copyright, Licence, Warranty Disclaimer:

=======================================

See the FINE PRINT section at the end of this file.

Document Organization:

=====================

This document incorporates, by reference, the file SLIPKT.DOC

which should be consulted for a description of CSLPKT's basic

operation.  The present file will only discuss differences

and/or additional features of CSLPKT not present in SLIPKT.

In other words, read SLIPKT.DOC first.  Almost everything there

applies to CSLPKT too, with exceptions as noted in this file.

Synopsis.                       CSLPKT is ...

CSLIP Vs. SLIP.                 FAQ #0

C/SLIP Vs. PPP.                 FAQ #1

SLIP Servers.                   Toggling

Technical Summary.              Oh, is that right ?

Caveats.                        CSLPKT is *not* ...

Acknowledgement.                Without whom ...

FINE PRINT.                     Read this !

Synopsis:

========

CSLPKT is a DOS TSR which drives a PC serial port using

SLIP framing and Van Jacobson TCP/IP header compression,

and provides an ethernet packet driver interface (FTP

Software, Class 1).  This allows TCP/IP-based application

software, which was designed to be used on an ethernet

connection, to be used with a Compressed SLIP (CSLIP)

server (as described in Appendix B of RFC 1144).

CSLPKT was written from scratch in assembly language.

Its features include:

- Performs VJ TCP/IP header compression to improve interactive

response and throughput on low-speed serial links.

- Low memory overhead, under 8.3K.  This leaves a lot of

memory free for application programs, especially as

compared to PPP drivers.

- RARP emulation to simplify application configuration.

- Detects and uses FIFO buffers on 16550A or equivalent UARTs.

- Supports packet multiplexer PKTMUX' /b option.

CSLIP Vs. SLIP:

==============

In this section, and the next, I discuss two common

questions.  The intent is to give a relatively simple clear

introduction to the main issues (as I see them), rather than to

be thorough.  I know much is being omitted, but hope not too

much is wrong, and that this will be useful to newcomers (as we

all are at some time).

**  What is CSLIP, what is it good for, and how do I best use it?

First, let's review ordinary SLIP (Serial Line Internet

Protocol).  This is a way of "framing" datagrams that are to be

transmitted on a serial line.  Basically, a special byte (a

number between 0 and 255) is chosen to signify the (start and)

end of the datagram.  This byte is called SLIP_END.  You would

like to transmit SLIP_END, then all the bytes of data, and

finally SLIP_END again.  But SLIP_END itself can occur in the

datagram, and if so it must be "escaped".  This is done by

sending an "escape byte" called SLIP_ESC followed by another

called ESC_END, in place of SLIP_END in the data.  SLIP_ESC

must similarly be escaped when it occurs in the data.

The receiver of such a "SLIP frame" undoes the two kinds of

escaping, and knows a datagram is complete when it sees

SLIP_END.  That's about all there is to it.  For further

details see the easily readable RFC 1055.  (RFC's are Internet

Request for Comment documents.  They can be found at

ftp://ds.internic.net/rfc/rfc####.txt

ftp://ftp.cis.ohio-state.edu/pub/rfc/rfc####.txt

where #### is the document number, such as 1055.)

To understand CSLIP (Compressed SLIP), one must consider the

structure of the datagram itself.  It can be viewed as composed

of two parts, a TCP/IP header, and then a segment of user data,

[ TCP/IP header | data segment (if any) ] .

The header occupies at least 40 bytes, and can be as long as

120.  The 40-byte minimum size is actually very common.  It

occurs, e.g., in ftp sessions ("unidirectional bulk data

transfer") and while typing in a telnet session ("interactive

data transfer").

To see the utility of compression, it's helpful to look at

these two examples more closely.  Consider first ftp transfer

of a large file.  If the hardware driver has an MTU of, say,

1006 bytes, this represents the largest datagram that can be

sent.  40 bytes are taken up by the header, leaving 966 for

chunks of the file being transferred.  To maximize efficient

use of the line, you would send 966 data bytes with each

header.  But, note that such large packets will take longer to

transmit than shorter, less efficient, ones.  Turning to

telnet, each keystroke you type may well be sent in its own

datagram -- 40 bytes of header carrying a single data byte!

Not very efficient.

This is where compression, in the form of Van Jacobson TCP/IP

header compression (VJ TCP/IP Comp., the C in CSLIP) comes in.

(This kind of compression shouldn't be confused with the kind

of wholesale compression performed by modern modems (V42.bis,

MNP 5).)  VJ Comp. is a way of preparing datagrams for

transmission by reducing the TCP/IP header size significantly.

Typical 40-byte headers, as above, are reduced to 3 bytes (by a

"differential coding" technique described in RFC 1144).  When

this compressed datagram is received, the original is

reconstructed by decompressing the received header.  This has

two important consequences:

(1) Clearly, line efficiency is increased, relatively more so

for datagrams with smaller amounts of data.  But in itself this

is actually of lesser importance, as compared to its interplay

with the following.

(2) For bulk data transfer, one can amortize the 3-byte header

against a much smaller user data segment and still be very

efficient.  E.g., 256 or even 128 data bytes instead of nearly

1000.  Now, these small datagrams are transmittable in a

fraction of the time (e.g., 1/4 or 1/8) needed for kilobyte

sized ones.  The main point is now that any (tiny 4-byte)

telnet datagrams, that may be being generated at the same time,

don't have to wait as long for their turn to be transmitted

(and echoed back), resulting in good interactive response.

This means that with VJ compression you can see your telnet

keystrokes echo quickly during background data transfers over a

low-speed serial line.  You achieve this by choosing a small

MTU in the application programs (to keep the ftp datagrams

small), and this doesn't sacrifice very much in line

efficiency.  Clearly there is a parameter trade-off involving

MTU, line-speed, efficiency and response time.  In the simple

model described in Sec. 2 of RFC 1144 these parameters are

related by:

t = [ MTU/2 + 2(h+1) ] 10/s

e = 1 - h/MTU

where

t ... average delay time for key echo (seconds),

h ... header size (bytes) [40 for SLIP, 3 for CSLIP],

s ... line speed (bits/second),

e ... line efficiency.

These formulae can be used to select an MTU large enough to

give reasonable efficiency ( say, e > 0.9 ) yet small enough to

give good interactive response (say, t < 0.2 s = 200 ms).

Example: using CSLIP at 14,400 bps (assuming no other

compression) with an MTU of 256 bytes gives

t =  94 ms,     e = 0.988 = 98.8% .

If SLIP is used instead, then

t = 146 ms, e = 0.84 .

I highly recommend reading portions of RFC 1144 for further

details (get the PostScript version if you can print it).

In summary, VJ compression is useful when performing data

transfer and interactive TCP/IP tasks simultaneously.

("Multiplexing" TCP/IP sessions, as can be done in DOS with

programs like KA9Q, or PKTMUX and DESQview.)  For it to be

useful, relatively small MTU's should be used.  It does wonders

for a slow serial line (e.g., over a modem), but would not be

so useful with very fast hardware like ethernet.  Note that the

extra CPU time needed to compress or decompress the headers is

well spent compared to the time taken to transmit 37 extra

bytes on a slow line.

C/SLIP Vs. PPP:

==============

**  What is PPP, how does it differ from C/SLIP, and which should I use?

In the context of the previous section, a first remark is

that PPP (Point-to-Point Protocol) provides an alternate way to

frame datagrams for transmission over a serial line.  Thus one

can optionally use VJ TCP/IP header compression with PPP, just

as with SLIP (although one never speaks of CPPP), to obtain

similar benefits.  But PPP does much more than just this.

PPP is a modern, robust, flexible, standards-based link-level

protocol which supports:

- Multiple network protocols.

In addition to IP, can carry Novell IPX, DECnet

and others.

- Negotiation of link parameters.

Things like frame size and compression, and

control character escaping are negotiated

between the client and server.

- Authentication.

For security purposes.

- Link-level error detection and link quality monitoring.

- Network Control Protocol.

Can negotiate IP addresses and use of VJ compression.

By contrast, SLIP provides none of these features, and is

somewhat of a non-standard.

Whether to use SLIP or PPP ultimately depends on your needs

and circumstances, but the case in favour of PPP is clear from

the above.  (Some unlovely discussions over which is "better"

orbit tightly around the idea that SLIP is faster because it

requires fewer framing bytes and less processing, detach this

from all other relevant issues, and impute it with unrestricted

significance.  While based on grains of technical truth, the

effect is likely unnoticeable in practice and can be overwhelmed

by other implementation factors, making it far from the single

important aspect.)

Having said all this, I believe that in the DOS world a case

can often still be made for preferring SLIP.  This arises from

its simplicity of implementation, which makes SLIP drivers much

smaller than PPP drivers.  It also means that SLIP drivers

generally won't incompletely implement the protocol.

The driver size can be important in DOS, where memory

availability may be a problem.  Current free- and share-ware

PPP drivers range in size from about 43 to 128 Kb resident,

many times larger than SLIPKT and CSLPKT.  Smaller drivers such

as these could benefit:

- DOS users of 386+ PC's who load enough TSRs to fill their

available upper memory.

- Users of DESQview, which typically takes all upper and high

memory, and additionally a part of conventional memory (at

least, if some other TSRs are in use too).

- The XT community, which has no high memory.  Especially here

one must use DOS rather than Windows for Internet access.  This

probably applies to 286's to a fair degree too.

- Users who just like small programs, who have no PPP access, etc.

Furthermore, until recently the free DOS PPP drivers were

plagued by problems like poor performance and instability.  (A

notable exception, KA9Q, is an integrated package and so

doesn't provide a packet driver interface for external

applications to use.)  Recently, however, there is some

excellent news in this area.  The Linux PPP daemon has been

ported to DOS by Antonio Molero.  His work is available at:

http://personal.redestb.es/tonilop/

ftp://ftp.agate.net/users/01935/internet/dosppp06.zip

His packet drivers appear to work very well and will likely

undergo further development to support more features.

For technical information on PPP see RFC's 1332, 1661, 1662

(to start with).  A more friendly introduction is Bob Sutterfield's

"PPP White Paper"

http://www.progressive-systems.com/docs/pppwp.pdf

SLIP Servers:

============

SLIP servers come with a variety of capabilities, which

should be kept in mind when connecting to one (or choosing

one).  All of the following are possible.

- Server does SLIP only.

CSLPKT won't work -- it sends (VJ header-)compressed

packets which the server doesn't understand.

- Server does CSLIP only.

SLIPKT won't work -- server sends compressed packets.

- Server does "adaptive SLIP".

Both SLIPKT and CSLPKT will work.  However, once

CSLIP is used, if the server keeps sending compressed

packets, you can't revert to SLIP in that session.

Modern servers may cope better with such toggling.

Technical Summary:

=================

CSLPKT allocates 16 connection state slots for VJ Comp.,

and allows connection state ID compression.

CSLPKT supports the /b option of PKTMUX ver 1.2i.  This

option is used to increase efficiency by reducing the

amount of packet copying during reception.

Caveats:

=======

-   SLIP framing provides no link-level error detection.

Consequently, when it is used in combination with VJ TCP/IP

header compression (as it is in any CSLIP driver such as

CSLPKT) there is a small (but non-zero) probability of data

corruption due to per-conversation loss-of-sync (RFC 1144, Sec.

4.1).  This may be unacceptable for certain uses (e.g.,

medical, scientific, commercial), while tolerable for others

(e.g., recreational).

Acknowledgement:

===============

The VJ compression routines used by CSLPKT were hand-written

in 8086 assembly, but are patterned after the sample C code

implementation found in Van Jacobson's RFC 1144 and in

cslip-2.7.tar.Z on ftp.ee.lbl.gov.  In case it matters, that C

code bears the following copyright notice:

/*

* Copyright (c) 1989 Regents of the University of California.

* All rights reserved.

*

* Redistribution and use in source and binary forms are

* permitted provided that the above copyright notice and this

* paragraph are duplicated in all such forms and that any

* documentation, advertising materials, and other materials

* related to such distribution and use acknowledge that the

* software was developed by the University of California,

* Berkeley.  The name of the University may not be used to

* endorse or promote products derived from this software

* without specific prior written permission.

* THIS SOFTWARE IS PROVIDED ``AS IS'' AND WITHOUT ANY EXPRESS

* OR IMPLIED WARRANTIES, INCLUDING, WITHOUT LIMITATION, THE

* IMPLIED WARRANTIES OF MERCHANTIBILITY AND FITNESS FOR A

* PARTICULAR PURPOSE.

*/

FINE PRINT:

==========

CSLPKT is Copyright 1999 by Frank Molzahn.

All applicable rights reserved.

CSLPKT is smiley-ware.  If you think this software is

worth using (or maybe you just like the DOC), why not

send the author your smiley?

CSLPKT is part of the CSLPKT05 package.

CSLPKT may be freely distributed provided all the files in

the package are present and unmodified.  This specifically

includes the files

BTSA.TXT

SLIPKT.COM

SLIPKT.DOC

CSLPKT.COM

CSLPKT.DOC

PKT_APPS.TXT

README.TXT

This copyright, licence and warranty disclaimer notice

applies to the entire CSLPKT05 package.  If you are unsure

whether or not your package is intact, download the current

version from Simtel.Net.

CSLPKT IS PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND,

either expressed or implied, including, but not limited

to, the implied warranties of merchantability and fitness

for a particular purpose.  The entire risk as to the

quality and performance of the product is with the user.

This documentation explicitly states that CSLPKT will not

work properly in some circumstances; the user assumes the

cost of all necessary servicing, repair or correction.

In no event will Frank Molzahn be liable for damages,

including any general, special, incidental or consequential

damages arising out of the use of, or inability to use,

CSLPKT (including but not limited to loss of data or data

being rendered inaccurate or losses sustained by the user

or third parties or a failure of the product to operate

with any other programs), even if the author has been

advised of the possibility of such damages.

These copyright, license, and disclaimer notices must be

included with all copies of CSLPKT.

You may use CSLPKT only if you have read, understood and

accepted all of the above conditions.

Please contact me to report bugs or errors in the document

(even minor ones).

[totally] ������ Software

is:

Frank Molzahn                  molzahn@cc.umanitoba.ca [preferred]

Winnipeg, Canada               molzahn@wotan.physics.umanitoba.ca

http://home.cc.umanitoba.ca/~molzahn/software.html