SAALRA v1.00 - Documentation

Copyright (C) 1997

by Robert B. Clark <rclark@iquest.net>

ClarkWehyr Enterprises

This program is SHAREWARE.  Please register!

�������������������������������������������������������������������������

What is SAALRA?

~~~~~~~~~~~~~~~

SAALRA (SpectrAA Linear Regression Analysis) is a DOS utility

designed to extract analytical run information from a Varian SpectrAA

300/400 graphite furnace AA instrument running Zeeman data

acquisition software.  While the software is capable of printing a

report containing all of the standard absorption and concentration

information, it does not calculate the sample (unknown)

concentrations.  This has to be done by hand, resulting in much lower

laboratory productivity than desired.

SAALRA takes the report generated by the acquisition software and

extracts the standard absorption and concentration information.  It

then performs a linear regression on that data and calculates the

concentrations of the unknown samples.  Finally, SAALRA generates a

plain ASCII report file in tabular form listing all of the standard

and sample data.

Optionally, SAALRA may generate a comma-separated value (CSV) file

that may be read by almost any standard database or spreadsheet

application such as Lotus 1-2-3 or Excel.

Installing SAALRA

~~~~~~~~~~~~~~~~~

SAALRA is a DOS utility and must be run from the DOS command prompt

or from a DOS box in Windows 3.x or Windows 95.  The requirements for

SAALRA are quite modest; you need only have

� An IBM/PC compatible 8088/8086 or better

� 512kb conventional RAM

� Floppy drive or hard disk drive

� Optional printer

� IBM/MS/DR/DOS version 3.x or better

To install SAALRA, simply extract (unzip) the distribution archive

into a directory listed in your path.  If you do not know what is in

your path, type SET at the DOS prompt and look for the line beginning

with "PATH=".

Using SAALRA

~~~~~~~~~~~~

Once you've extracted the SAALRA.EXE file, you may use the program.

At the DOS prompt, type

Page 1

SAALRA /?

You will see a short summary of the program's command line switches

and parameters.  Only the command line summary is shown here:

SAALRA [/C] [/F] [/Pa[:c]] [/Ln] report-file[.TXT]

[output-file[.REP]]

Items enclosed within square brackets are optional; items unbracketed

are required.

Either a dash (-) or a forward slash (/) may be used to indicate a

command line option.  The command line options may be listed in any

order.

Following is a detailed explanation of the various command line

parameters:

[/C]

Generate a comma-separated value (CSV) output file.

By default, SAALRA produces tabular report files with a .REP

extension. If you include the /C switch, the output will be in a

comma-separated format having the following structure:

"Standard", "Abs","Conc"

"BLANK     ", abs, conc

"STANDARD 1", abs, conc

...

"STANDARD n", abs, conc

"Sample", "Abs", "Calc Conc"

"Sample 1", abs, conc

"Sample 2", abs, conc

...

"Sample n", abs, conc

[/F]

Force form feeds.

When an output report file is written to disk instead of to the

printer, SAALRA does not paginate the report.  Use /F to force

SAALRA to include form feeds and page breaks when writing a report

to a disk file.

[/Ln]

Set page length.

Sets page length to n lines per page.  This switch is ignored if

Page 2

'output-file' is a disk file.  'n' must be at least 25; smaller

values are changed to 55.

The default is 55 lines per page.

[/Pa[:c]]

Override absorbance and concentration precision.

Specify the precision (number of decimal places) to be used when

displaying absorbance and concentration values.

Absorbance 'a' and concentration 'c' default to 3 and 5 decimal

places, respectively.

[-?|-H]

Prints a help message on the screen.

[report-file[.TXT]]

Report file generated by the Zeeman software.  The default

extension is .TXT.  This parameter is required and may contain

wildcards.

[output-file[.REP]]

Report file generated by SAALRA.  This will either be a tabular

report file with a .REP extension, or a comma-separated value file

with a .CSV extension if /C was specified in the command line.

The base filename defaults from the 'report-file' parameter.  If

'report-file' is RUNA.TXT, by default 'output-file' will be

RUNA.REP.

To send the output directly to a printer, use PRN or LPTn as the

'output-file' filename, where n is the number of the printer port.

As an aside, any valid DOS device (e.g., CON, AUX, NUL, COMn) may

be used for the 'output-file' parameter.

In most cases, you will never need to specify anything other than the

'report-file' and 'output-file' parameters when using SAALRA.

Generating a Report

~~~~~~~~~~~~~~~~~~~

Configure the Varian SpectrAA 300/400 acquisition software to

generate a QC protocol report on disk instead of directing the output

to the printer. For this example, let's call this report file

Page 3

RUN.TXT.

Go to the "External Control" page and change the "Output Channel

Name" from the default "PRINTER:" to RUN.TXT.  You may also specify

the path if desired.

Make sure to press the "Output Setup" soft key to confirm your

changes.

Note that the acquisition software will neither overwrite nor delete

an existing report file.  If such a file exists, the software will

append the current run to it.  Delete any existing RUN.TXT file prior

to initiating a sequence.

After the sequence is complete and you have the RUN.TXT file, simply

run SAALRA to generate the LR report:

C:\ZEEMAN\DATA> SAALRA RUN

On your monitor, you should see

SAALRA v1.00 Copyright (C) 1997 ClarkWehyr Enterprises

All rights reserved.

� Generating tabular report file

� Absorbance precision    : 3 decimal places

� Concentration precision : 5 decimal places

Processing C:\ZEEMAN\DATA\RUN.TXT ...

RUN.TXT      ==> C:\ZEEMAN\DATA\RUN.REP

Thank you for trying SpectrAA LR Analysis v1.00!

The report file will be named RUN.REP and may be viewed and/or edited

by any ASCII text editor.

You may use wildcards to specify a group of TXT files to process all

at one time.  For example, to process all of the TXT files in the

current directory and print the results to the default printer, use

C:\ZEEMAN\DATA> SAALRA *.TXT PRN

If you use a wildcard for the 'report-file' parameter, you must

specify an explicit filename for the 'output-file' parameter.

Included in this archive is an example QC Protocol Report file named

SAMPLE.TXT.  This file was generated from an actual sample run using

the SpectrAA 300/400 Zeeman data acquisition software.  You may use

this file to test the operation of SAALRA:

C:\> SAALRA SAMPLE

The report file generated by SAALRA will be named SAMPLE.REP.

Page 4

Exit Codes

~~~~~~~~~~

SAALRA will generate an exit code of 0 if there were no errors;

otherwise an exit code of 1 will be returned.  For example, SAALRA

will terminate and return errorlevel 1 if you run out of disk space

while writing report files to disk.

It Ain't Free

~~~~~~~~~~~~~

Unlike most of ClarkWehyr Enterprises utilities, SAALRA was designed

for and targeted to business users.  It is therefore released as

shareware, NOT public domain or freeware.

BUSINESSES (defined as "for-profit" ventures) may use a single copy

of SAALRA for up to thirty (30) consecutive days for evaluation

purposes. If after that time period you elect not to register the

product, you must remove it from your computer systems and all backup

media.

SAALRA may be installed and run from a network server's disk volume.

You must purchase one license for each workstation attached to that

server, or you may negotiate a site license for more than twenty (20)

workstations.

STUDENTS AND HOME USERS:  You may use the non-registered version of

the product for as long as you wish, subject to the other

restrictions noted above.  I only ask that you freely distribute the

non-registered SAALRA archive wherever it is welcomed.

Please read the LEGALESE.DOC document for additional restrictions

and/or warranties.

So How Much?

~~~~~~~~~~~~

SAALRA is sold on a per-license basis.  Each license is $49.95 US,

with a 10% discount for quantities of more than 10.  Site licenses

are available if you require more than 20 licenses; contact the

author for details.

Payment may be made by check or money order, drawn on a U.S. bank.

Please make checks payable to "Robert B. Clark."  Orders will be sent

out when the check has cleared the bank; please allow up to five

working days.

Send cash at your own risk.

Upgrade Policy

~~~~~~~~~~~~~~

Your registration fee entitles you to free minor revision upgrades.

Upgrades to the next major revision are discounted for registered

Page 5

users.  A "major" revision is a whole number increment in the version

number; a "minor" revision is a fractional increment.  For example, a

v1.10 release would be a minor upgrade from version 1.00; a v2.01

release would be a major upgrade from version 1.00.

Why Register?

~~~~~~~~~~~~~

When you register SAALRA, you get

o Removal of any shareware "beg" screens;

o Better chance of a response when requesting enhancements;

o Direct E-mail support by the author for one year;

o My undying gratitude.

When you register SAALRA, you will be provided a registration code

and instructions on how to "brand" your copy of the program.  The

registration code will either be mailed to you on a postcard or

e-mailed, at your preference.

Source code is available to registered users.  Contact Robert B.

Clark <rclark@iquest.net> for details.

Contact Information

~~~~~~~~~~~~~~~~~~~

The author may be contacted via any of the following methods:

Robert B. Clark

ClarkWehyr Enterprises

7204 Knobwood Drive Suite E

Indianapolis, IN 46260-3889

USA

Voice: 317-298-7707  8am-9pm EST (GMT-5:00)

BBS: 317-298-5391  8-N-1, 28.8kbps

E-mail: rclark@iquest.net

WWW: http://www.iquest.net/~rclark/ClarkWehyr.html

###

Page 6