Synchronized Voice Server Function Set

Call Engineering

For Visual Basic Client-Server Voice Telephony Operations On Dialogic Voice Boards In Synchronous Mode

All of the information in this document is subject to change without notice. No part of this document may be reproduced

or transmitted in any form or by any means without the express written permission of Call Engineering

© 1998 - 1999 Call Engineering, All rights reserved.

Printed in the United States

Use of this software is restricted by the terms of the license agreement. ( -see svsorder.wri )

Software Copyright © 1998 - 1999, Call Engineering.

Windows™ and Visual Basic™ are trademarks of Microsoft Corporation.

Dialogic™ is a trademark of Dialogic Corporation.		HTTP://WWW.DIALOGIC.COM

SpeechWorks™ is a trademark of SpeechWorks International, Inc.

Contact Information:	Bob Johnson

Call Engineering

1824 Point View Avenue, Youngstown, Ohio 44502

(330) 788-3073, BobJohnson@NEO.RR.COM

Ordering Information:	See File SVSORDER.WRI

This document may be easier to use with your display set to 800 x 600.   Use WordPad.exe and set the page margins to .5" left margin and .5" right margin.

Operating Systems Requirements

The SVSERVER runs on Windows 95/98 and single processor NT4.0 operating systems.

Visual Basic Voice Telephony Software Tool For Dialogic Boards

The SVSERVER is a synchronized voice server which allows Visual Basic and\or any database or application

language which supports the Microsoft DDEML to drive Dialogic professional voice boards.  This download supplies a sample Visual Basic 6.0 client program that access the voice server software.  In addition, this documentation is provided to enhance your understanding of the voice client server interaction and operation in developing voice telephony applications for the SVSERVER.

Client-Server Operation

The client application sends function commands as an ascii string via the Dynamic Data Exchange medium of

Microsoft Windows DDEML to the SVSERVER application which receives the commands and executes them via an

interface to the Dialogic Multithread Function Library.  When the function completes, the SVSERVER returns the Dialogic

function status and any data to the client application.  The design of SVSERVER permits multiple client applications

having different channel assignments to interact with the voice server simultaneously.  For instance, one application may

be running voice mail on channels 1 to 4 while some other  automated telephony application runs using channels 5 to 8.

This methodology may be implemented in Visual Basic or any database or application language that supports the

Microsoft Windows DDEML.  The sample in this documentation is limited to VB Text Controls.  The user should first

review the initiate-response interaction of the client-server operation and then study the sample synchronous applications

SVSAMPLE and SNSAMPLE.  Please note: SVSERVER operates in one thread per channel, synchronous mode.

Simplicity of Design

The SVSERVER  application handles hundreds of functions consisting of ten ez_ ease of use synchronous functions, twenty two ut_ utility functions, several hundred Dialogic functions and three hundred or more Speechworks API functions.  This range makes it both easy to use and a powerful implementation of the Dialogic function set.  Versions are available in from 1 to 128 lines.  Current libraries supported are:

The Voice Library			libdxxmt.lib,

The Standard Run-Time Library		libsrlmt.lib,

The SVServer Convenience & Utility Functions

The Digital Network Library		libdtimt.lib,

The D42 PBX Function Library		libd42mt.lib,

The SCBus SCTools functions		sctools.c,

The SCx160 Bus Adapter		scx functions,

The Dialogic Audio Confrencing		dcblib.lib,

The Modular Station Interface		msilib.lib,

The TextToSpeech API			texttalk.lib,	Requires Dialogic Enablement Disk

The SpeechWorks Host API.		interface,	Requires SpeechWorks License

The SpeechWorks Extended API.	interface,	Requires SpeechWorks License

The Global Call Library			libgc.lib

The Cross Compatability Library and the Fax library functions are in the works.

Documentation for the SpeechWorks Extended API functions is also in process.

Key Features

Extensive Dialogic Function Set

Includes EZ Convenience Functions

Rapid Prototyping, Development, And Deployment

Uses Simple Text Box DDE Linkage In Visual Basic

Supports SpeechWorks ASR and Dialogic Text-To-Speech

Single Process Fast Multithreading Results In Lower Memory Requirements

Over Eight Hundred Functions Make For A Powerful Implementation

Try Before You Buy, One Line Evaluation Version Is Free

Getting Started

Unzip in an empty folder.  Rename the folder SVSERVER and copy to the root directory, C:\SVSERVER.  If you

do not place it in the root directory you will have problems running the sample Visual Basic programs provided.

Evaluation Version of the Software

The evaluation version of the software is provided in both source and executable for the sample application and

is limited to 1 line of operation in the SVSERVER.  A date-limited mult-line evaluation version is available on request.

All evaluation software has a 1 hour at a time run limit.

The Dialogic Configuration Manager and SVSERVER Synchronized Voice Server

The Dialogic Configuration Manager must start the boards before running SVSERVER applications and the SVSERVER application must be running prior to a client application sending Dialogic voice function commands to it.  Please Note: The SVSERVER application should be launched with command line arguments.  There are a number of methodologies available to develop telephony software with SVSERVER as follows:

SVSERVER Applications

1)	A single client application driving a single SVSERVER multithreaded process.

The SVSAMPLE application demonstrates a VB client application designed to run eight lines via a single process

multithreaded SVSERVER.

DIALOGIC Channel 1

DIALOGIC Channel 2

CLIENT APPLICATION		<-DDE->	SVSERVER	<--->	DIALOGIC Channel 3

DIALOGIC Channel 4

---

DIALOGIC Channel n

To run as a one thread per channel synchronous operation use the following command string:

(-See the VB6.0 application SVSample procedure: ShellSVServer)

Result = Shell(Trim$(SVServerFilePath), 1)

To limit the number of lines use "MaxLines nn".  For instance if you have a 32 line version of SVSERVER but

you want to limit the number of active lines to 8 use the term "MaxLines 8".  This prevents the software from creating

unused threads and wasting cpu time.

Result = Shell(Trim$(AppFilePath) & " MaxLines " & Trim$(Str(MAXLines)), 1)

2)	Several client applications with differing line assignments driving a single SVSERVER multithreaded process.

DIALOGIC Channel 1

CLIENT APPLICATION 1	<-DDE->				DIALOGIC Channel 2

CLIENT APPLICATION 2	<-DDE->	SVSERVER	<--->	DIALOGIC Channel 3

CLIENT APPLICATION 3	<-DDE->				DIALOGIC Channel 4

---

DIALOGIC Channel n

Each client application has different channel assignments.  Lines are assigned when the DDE conversation is

established between the client application and the SVSERVER.

Please Note: In order to utilize the SpeechWorks functions: the keyword "SpeechWorks" must be included in the SVServer startup command line.  To utilize the Global Call functions the keywords "Global Call" must be included in the SVServer startup command line.  These functions require initialization processes.

Examples:	Result = Shell(Trim$(AppFilePath) & " Global Call, MaxLines " & Trim$(Str(MAXLines)), 1)

Result = Shell(Trim$(AppFilePath) & " SpeechWorks, MaxLines " & Trim$(Str(MAXLines)), 1)

Client-Server Interaction

Please Note: The terms line and channel may be used interchageably in this document.

Client Application Initiates				SVSERVER Voice Server Responds

1)	Establishes a conversation with the SVSERVER

Text1.LinkMode = 0	'	break connection

Text1.LinkTopic = "SVServer|SVSFunction"

Text1.LinkItem = "Line1"

Text1.LinkMode = 3	'	establish connection

do this once for each line

2)	SVSERVER responds to DDEML accordingly

3)	Sends a function command to the SVSERVER

Text1.Text = "dx_open"	'	open the line

Text1.LinkPoke		'	send to SVServer

4)	SVSERVER  executes the function then advises the 								client application when it has completed the function 								that it has status data to return.

5)	Notification arrives via the text controls Link Notify Event

The text control's link notify procedure then requests

that SVSERVER send the data

Private Sub Text1_LinkNotify

On Error Resume Next

Text1.Text = ""

Text1.LinkRequest	'	request data

End Sub

6)	SVSERVER sends the function status to the client 								application's text control

7)	Status data arrives in the application's text control

LNE=3, FNC=100, STA=4, ERR=0, RTN=2, RSL:

The client application's main loop watches for the

status information to arrive and processes it when it comes.

It then continues sending other commands and receiving status.

The status.  From the client application's point of view this is a simple send-receive, send-receive scenario:

1)	Send the function command to the svserver.

2)	Do whatever ... until notification is received from the svserver.

3)	Send the request that the svserver return the status.

4)	Receive the function status and data from the svserver.

If the text control's link mode is set to automatic then this process is reduced even further to just send-receive.

The link notify event is handled automatically.

1)	Send the function command to the svserver.

2)	Receive the function status and data from the svserver.

Returned Function Status

Example	LNE=3, FNO=100, STA=4, ERR=0, RTN=2, RSL: Resultant Extended Data Returned Here

Line Number			LNE=	1 - 32

Status				STA=	SVSERVER Function Progress Status

STATUS_FUNCTION_NONE		0	SVServer Channel Idle

STATUS_FUNCTION_READY		1	Function Ready To Run

STATUS_FUNCTION_RUNNING		2	Function Started

STATUS_FUNCTION_NOTIFIED		3	Client Application Notified

STATUS_FUNCTION_COMPLETE	4	Function Completion

Return				RTN=	Value Returned From Dialogic Function, -1 or < 0  = Failure

Result				RSL:	Resultant Extended Data From Dialogic Function -see function specifics

Error				ERR=	Errors

Function Not Found			1	Syntax Error In Function Name

Memory Allocation Error			2	Unable To Allocate Memory Space

Index Value Out Of Range		3	Index Value Out Of Range

Parameter Count Error			4	Too Many Or Too Few Parameters Sent

Function Not Implemented		5	Function Not Implemented

Null Pointer				6	Null Pointer Encountered, Bad Pointer

Empty Parameter Value			7	Empty Parameter Value (,,)

Bad Parameter				8	Bad Parameter

Negative Value Of Function No		< 0	Negative Value Specifies Function Number In Error

ATDV_LASTERR			=	ATDV_LASTERR values returned on some convenience functions

Functions			FNO=		Function Identification Number

ag_getctinfo			0

ag_getxmitslot			1

ag_listen			2

ag_unlisten			3

ATD4_BDTYPE			4

ATD4_CHTYPE			5

ATDT_BDMODE		6

ATDT_BDSGBIT		7

ATDT_DNLDVER		8

ATDT_IDLEST			9

ATDT_ROMVER			10

ATDT_STATUS			11

ATDT_TSMODE			12

ATDT_TSSGBIT			13

ATDV_ERRMSGP		14

ATDV_IOPORT			15

ATDV_IRQNUM			16

ATDV_LASTERR		17

ATDV_NAMEP			18

ATDV_SUBDEVS		19

ATDX_ANSRSIZ		20

ATDX_BDNAMEP		21

ATDX_BDTYPE			22

ATDX_BUFDIGS		23

ATDX_CHNAMES		24

ATDX_CHNUM			25

ATDX_CONNTYPE		26

ATDX_CPERROR		27

ATDX_CPTERM			28

ATDX_CRTNID			29

ATDX_DBMASK		30

ATDX_DEVTYPE		31

ATDX_DIGBUFMODE		32

ATDX_DTNFAIL			33

ATDX_EVTCNT			34

ATDX_FRQDUR			35

ATDX_FRQDUR2		36

ATDX_FRQDUR3		37

ATDX_FRQHZ			38

ATDX_FRQHZ2			39

ATDX_FRQHZ3			40

ATDX_FRQOUT			41

ATDX_FWVER			42

ATDX_HOOKST			43

ATDX_LINEST			44

ATDX_LONGLOW		45

ATDX_NUMCHAN		46

ATDX_PHYADDR		47

ATDX_PRODUCT		48

ATDX_RINGCNT		49

ATDX_RXDATABUFSIZE	50

ATDX_SHORTLOW		51

ATDX_SIZEHI			52

ATDX_STATE			53

ATDX_TERMMSK		54

ATDX_TONEID			55

ATDX_TRCOUNT		56

ATDX_XFERBUFSIZE		57

ATFX_BADIOTT			58

ATFX_BADPAGE		59

ATFX_BADSCANLINES		60

ATFX_BSTAT			61

ATFX_CHTYPE			62

ATFX_CODING			63

ATFX_ECM			64

ATFX_ESTAT			65

ATFX_FXVERSION		66

ATFX_LASTIOTT		67

ATFX_PGXFER			68

ATFX_PHDCMD		69

ATFX_PHDRPY			70

ATFX_RESLN			71

ATFX_RTNPAGES		72

ATFX_SCANLINES		73

ATFX_SPEED			74

ATFX_STATE			75

ATFX_TERMMSK		76

ATFX_TFBADTAG		77

ATFX_TFNOTAG		78

ATFX_TFPGBASE		79

ATFX_TRCOUNT		80

ATFX_WIDTH			81

ATMS_DNLDVER		82

ATMS_FEMINFO		83

ATMS_NUMCHAN		84

ATMS_STATINFO		85

ATMS_TSSGBIT		86

dx_addspddig			100

dx_addtone			101

dx_addvoldig			102

dx_adjsv			103

dx_blddt				104

dx_blddtcad			105

dx_bldst				106

dx_bldstcad			107

dx_bldtngen			108

dx_chgdur			109

dx_chgfreq			110

dx_chgrepcnt			111

dx_close			112

dx_close_board			113

dx_clrcap			114

dx_clrdigbuf			115

dx_clrsvcond			116

dx_clrtpt			117

dx_deltones			118

dx_dial				119

dx_distone			120

dx_enbtone			121

dx_filechsize			122

dx_fileclose			123

dx_filecommit			124

dx_filedup			125

dx_filedup2			126

dx_fileerrno			127

dx_filefilelength			128

dx_filefstat			129

dx_fileisatty			130

dx_filelocking			131

dx_fileopen			132

dx_fileopen_osfhandle		133

dx_fileread			134

dx_fileseek			135

dx_filesetmode			136

dx_filesopen			137

dx_filestat			138

dx_filetell			139

dx_fileumask			140

dx_filewrite			141

dx_get_tngencad		142

dx_getctinfo			143

dx_getcursv			144

dx_getdig			145

dx_getevt			146

dx_getfeaturelist			147

dx_getparm			148

dx_getr2mf			149

dx_getsvmt			150

dx_getuio			151

dx_getxmitslot			152

dx_getxmitslotecr		153

dx_gtcallid			154

dx_gtextcallid			155

dx_gtsernum			156

dx_initcallp			157

dx_listen			158

dx_listenecr			159

dx_mreciottdata			160

dx_open			161

dx_open_board			162

dx_play				163

dx_playf			164

dx_playiottdata			165

dx_playtone			166

dx_playtoneEx			167

dx_playvox			168

dx_playwav			169

dx_rec				170

dx_recf				171

dx_reciottdata			172

dx_recvox			173

dx_recwav			174

dx_route			175

dx_routerxtx			176

dx_sendevt			177

dx_set_lasterr			178

dx_setdevuio			179

dx_setdigtyp			180

dx_setevtmsk			181

dx_setgtdamp			182

dx_sethook			183

dx_setiostatus			184

dx_setparm			185

dx_setrings			186

dx_setsvcond			187

dx_setsvmt			188

dx_setuio			189

dx_setuio_direct			190

dx_stopch			191

dx_TSFStatus			192

dx_unlisten			193

dx_unlistenecr			194

dx_wink				195

dx_wtcallid			196

dx_wtring			197

sr_dishdlr			200

sr_enbhdlr			201

sr_getboardcnt			202

sr_getevtdatap			203

sr_getevtdev			204

sr_getevtlen			205

sr_getevttype			206

sr_getparm			207

sr_hold				208

sr_NotifyEvent			209

sr_putevt			210

sr_release			211

sr_setlasterr			212

sr_setparm			213

sr_waitevt			214

sr_waitevtEx			215

ez_dial				300

ez_getdigits			301

ez_hangup			302

ez_hookflash			303

ez_pickup			304

ez_playfile			305

ez_playfiles			306

ez_playfilex			307

ez_playtext			308

ez_recfile			309

ez_recfilex			310

is_GetAllCRN			311

is_internaltest			312

nr_scroute			313

nr_scunroute			314

nx_scroute			315

nx_scunroute			316

ut_allocate			315

ut_deallocate			316

ut_free_setup			317

ut_get_ca_param		318

ut_get_ca_params		319

ut_get_devices			320

ut_get_dv_tpt			321

ut_get_dx_iott			322

ut_get_dx_svcb			323

ut_get_dx_svmt			324

ut_get_dx_xpb			325

ut_load_setup			326

ut_save_setup			327

ut_set_ca_param		328

ut_set_ca_params		329

ut_set_devices			330

ut_set_dv_tpt			331

ut_set_dv_tpt_size		332

ut_set_dx_iott			333

ut_set_dx_iott_size		334

ut_set_dx_svcb			335

ut_set_dx_svmt			336

ut_set_dx_xpb			337

ut_set_timer			338

xz_getdigits			339

xz_playfile			340

xz_playfiles			341

xz_playfilex			342

xz_playtext			343

xz_recfile			344

xz_recfilex			345

dt_close			400

dt_dial				401

dt_getctinfo			402

dt_getevt			403

dt_getevtmsk			404

dt_getparm			405

dt_getxmitslot			406

dt_hookflash			407

dt_listen				408

dt_mtfcn			409

dt_open				410

dt_open_board			411

dt_route				412

dt_rundiag			413

dt_set_lasterr			414

dt_setalrm			415

dt_setevtmsk			416

dt_setidle			417

dt_setiostatus			418

dt_setparm			419

dt_setsigmod			420

dt_settssig			421

dt_settssigsim			422

dt_tstcom			423

dt_tstdat			424

dt_unlisten			425

dt_wt_clear			426

dt_wt_event			427

dt_xmitalrm			428

dt_xmittone			429

dt_xmitwink			430

scx_close			500

scx_clrevtmsk			501

scx_getbrdparm			502

scx_getctinfo			503

scx_getevtmsk			504

scx_getver			505

scx_gtbdattr			506

scx_open			507

scx_setbrdparm			508

scx_setevtmsk			509

scx_tstcom			510

scx_tstdat			512

d42_brdstatus			600

d42_chnstatus			601

d42_closefeaturesession	602

d42_display			603

d42_flags			604

d42_getparm			605

d42_getver			606

d42_gtcallid			607

d42_indicators			608

d42_lcdprompt			609

d42_longbeep			610

d42_openfeaturesession		611

d42_putpbxmsg			612

d42_setparm			613

d42_writetodisplay		614

dcb_addtoconf			700

dcb_close			701

dcb_close_board		702

dcb_delconf			703

dcb_dsprescount		704

dcb_estconf			705

dcb_evtstatus			706

dcb_get_cdt			707

dcb_get_cdt2			708

dcb_get_volume			709

dcb_getatibits			710

dcb_getbrdparm			711

dcb_getcde			712

dcb_getcnflist			713

dcb_getdigitmsk			714

dcb_getevt			715

dcb_gettalkers			716

dcb_monconf			717

dcb_open			718

dcb_open_board		719

dcb_remfromconf		720

dcb_set_cdt			721

dcb_set_cdt2			722

dcb_set_volume			723

dcb_setbrdparm			724

dcb_setcde			725

dcb_setdigitmsk			726

dcb_tstcom			727

dcb_tstdat			728

dcb_unmonconf			729

ms_addtoconf			800

ms_chgxtder			801

ms_close			802

ms_close_board			803

ms_delconf			804

ms_delxtdcon			805

ms_dsprescount			806

ms_estconf			807

ms_estxtdcon			808

ms_genring			809

ms_genringex			810

ms_genziptone			811

ms_get_cdt			812

ms_get_cdt2			813

ms_get_ncb			814

ms_getbrdparm			815

ms_getcb			816

ms_getcde			817

ms_getcnflist			818

ms_getctinfo			819

ms_getevt			820

ms_getevtmsk			821

ms_getxmitslot			822

ms_listen			823

ms_monconf			824

ms_open			825

ms_open_board			826

ms_remfromconf		827

ms_set_cdt			828

ms_set_cdt2			829

ms_set_ncb			830

ms_setbrdparm			831

ms_setcb			832

ms_setcde			833

ms_setevtmsk			834

ms_setstparm			835

ms_setvol			836

ms_stopfn			837

ms_tstcom			838

ms_tstdat			839

ms_unlisten			840

ms_unmonconf			841

gc_AcceptCall			1000

gc_AnswerCall			1001

gc_Attach			1002

gc_CallAck			1003

gc_CallProgress		1004

gc_CCLibIDToName		1005

gc_CCLibNameToID		1006

gc_CCLibStatus			1007

gc_CCLibStatusAll		1008

gc_CRN2LineDev		1009

gc_Close			1010

gc_Detach			1011

gc_DropCall			1012

gc_ErrorValue			1013

gc_ExtensionFunction		1014

gc_Get_CallAck_Blk		1015

gc_GetANI			1016

gc_GetBilling			1017

gc_GetCallInfo			1018

gc_GetCallState			1019

gc_GetCRN			1020

gc_GetDeviceNameInfo		1021

gc_GetDlgerrValue		1022

gc_GetDNIS			1023

gc_GetFrame			1024

gc_GetLineDev			1025

gc_GetLinedevState		1026

gc_GetMetaEvent		1027

gc_GetMetaEventEx		1028

gc_GetNetworkH		1029

gc_GetParm			1030

gc_GetUsrAttr			1031

gc_GetVer			1032

gc_GetVoiceH			1033

gc_HoldCall			1034

gc_HoldCallACK		1035

gc_HoldCallRej			1036

gc_LibInit			1037

gc_LoadDxParm		1038

gc_MakeCall			1039

gc_Open			1040

gc_OpenEx			1041

gc_ReleaseCall			1042

gc_ReqANI			1043

gc_ResetLineDev		1044

gc_ResultMsg			1045

gc_ResultValue			1046

gc_RetrieveCall			1047

gc_RetrieveCallAck		1048

gc_RetrieveCallRej		1049

gc_Set_CallAck_Blk		1050

gc_SetBilling			1051

gc_SetCallingNum		1052

gc_SetChanState		1053

gc_SetDlgerrValue		1054

gc_SetEvtMsk			1055

gc_SetInfoElem			1056

gc_SetParm			1057

gc_SetUsrAttr			1058

gc_SndFrame			1059

gc_SndMsg			1060

gc_Start			1061

gc_StartTrace			1062

gc_Stop			1063

gc_StopTrace			1064

gc_WaitCall			1065

Dialogic Text-To-Speech, Not Implemented In The Evaluation Version

tts_close			1100

tts_geterror			1101

tts_getpitch			1102

tts_getspeechrate		1103

tts_getvoice			1104

tts_getvolume			1105

tts_init				1106

tts_jump				1107

tts_open			1108

tts_pause			1109

tts_phoneme			1110

tts_pitch			1111

tts_reset			1112

tts_resume			1113

tts_setcontext			1114

tts_speakbuffer			1115

tts_speakemail			1116

tts_speakfile			1117

tts_speechrate			1118

tts_stop				1119

tts_tracelevel			1120

tts_voice			1121

tts_volume			1122

Basic SpeechWorks API Functions ver 4.0.1/6.0.0

ALTServiceCreate		1200

ALTServiceDestroy		1201

ALTServiceExecute		1202

ALTServiceInitialize		1203

ALTServiceTerminate		1204

ALTServiceWaitForPrompt	1205

ALTSessionEndCall		1206

ALTSessionNewCall		1207

Extended SpeechWorks API Functions 6.0.0, Not Implemented In The Evaluation Version

ALTdmActivateGrammarRule							1300

ALTdmAddCommand								1301

ALTdmAddCommandWithMappings						1302

ALTdmAddCreditCardPrefix							1303

ALTdmAddCurrencyItem							1304

ALTdmAddDateItemWithMapping						1305

ALTdmAddDTMFCommand							1306

ALTdmAddEntry									1307

ALTdmAddListItem								1308

ALTdmAddListItemWithMappings						1309

ALTdmAddNaturalNumberItem							1310

ALTdmAddPhraseToClass							1311

ALTdmAddPrefix								1312

ALTdmAddPromptConstant							1313

ALTdmAddPromptVariable							1314

ALTdmAddSuffix								1315

ALTdmAddSynonymForNo							1316

ALTdmAddSynonymForYes							1317

ALTdmAddTimeOfDayItemWithMapping						1318

ALTdmAddWordToRuntimeClass							1319

ALTdmAllowAllCommandVocabulary						1320

ALTdmAllowAllListItemVocabulary						1321

ALTdmAllowAllVocabulary							1322

ALTdmCheckForCommand							1323

ALTdmCheckForEntry								1324

ALTdmCheckForListItem								1325

ALTdmCheckForSynonymForNo							1326

ALTdmCheckForSynonymForYes						1327

ALTdmCopyToPrompt								1328

ALTdmCreateModule								1329

ALTdmCreatePrompt								1330

ALTdmCreatePromptByName							1331

ALTdmDestroyModule								1332

ALTdmDestroyPrompt								1333

ALTdmExecute									1334

ALTdmFactoryAddNewService							1335

ALTdmFactoryCreate								1336

ALTdmFactoryDestroy								1337

ALTdmFactoryRemoveService							1338

ALTdmGetActiveLanguage							1339

ALTdmGetAfterEndOfSpeechTimeout						1340

ALTdmGetAllowingBargeIn							1341

ALTdmGetAllowingSameMistakeTwice						1342

ALTdmGetApologizingForMistakes						1343

ALTdmGetBargeInThreshold							1344

ALTdmGetBeepingAfterPrompts							1345

ALTdmGetBeforeBeginOfSpeechTimeout						1346

ALTdmGetBestAnswer								1347

ALTdmGetBestValueForKey							1348

ALTdmGetCannotDifferentiateYesNoPromptN					1349

ALTdmGetCCExpDateRange							1350

ALTdmGetCommandConfirmationPromptN					1351

ALTdmGetConfirmationAfterEndOfSpeechTimeout				1352

ALTdmGetConfirmationBeforeBeginOfSpeechTimeout				1353

ALTdmGetConfirmationMaxRetries						1354

ALTdmGetConfirmationMaxTimeouts						1355

ALTdmGetConfirmationPromptN							1356

ALTdmGetConfirmationTimeoutPromptN						1357

ALTdmGetConsideringRepeatedLowConfidenceHypotheses			1358

ALTdmGetCreditCardSequenceLength						1359

ALTdmGetCreditCardTypeActivation						1360

ALTdmGetCurrencyDisambiguationMode						1361

ALTdmGetCurrencyGranularity							1362

ALTdmGetCurrencyRange							1363

ALTdmGetCurrencyType							1364

ALTdmGetDateDisambiguationMode						1365

ALTdmGetDateRange								1366

ALTdmGetDefaultConfirmation							1367

ALTdmGetDisambiguationBeforeBeginOfSpeechTimeout				1368

ALTdmGetDisambiguationConfirmationPromptN					1369

ALTdmGetDisambiguationMaxRetries						1370

ALTdmGetDisambiguationMaxTimeouts						1371

ALTdmGetDisambiguationNoAnswerApologyN					1372

ALTdmGetDisambiguationPrompt						1373

ALTdmGetDisambiguationRepromptN						1374

ALTdmGetDisambiguationTimeoutPromptN					1375

ALTdmGetDisambiguationWrongAnswerApologyN				1376

ALTdmGetDTMFConfirmationPromptN						1377

ALTdmGetDTMFDateFormat							1378

ALTdmGetDTMFDecimalPoint							1379

ALTdmGetDTMFLength								1380

ALTdmGetDTMFRepromptN							1381

ALTdmGetDTMFTerminator							1382

ALTdmGetDTMFTimeout								1383

ALTdmGetDTMFYesNo								1384

ALTdmGetFailureDetails								1385

ALTdmGetFailurePrompt								1386

ALTdmGetInitialPrompt								1387

ALTdmGetMaxAnswers								1388

ALTdmGetMaxRetries								1389

ALTdmGetMaxTimeouts								1390

ALTdmGetModuleName								1391

ALTdmGetModuleType								1392

ALTdmGetNaturalNumberGranularity						1393

ALTdmGetNaturalNumberRange							1394

ALTdmGetNoAnswerApologyN							1395

ALTdmGetNthAnswer								1396

ALTdmGetNthAnswerConfidenceScore						1397

ALTdmGetNthAnswerInVocabularyScore						1398

ALTdmGetNthAnswerRawScore							1399

ALTdmGetNthAnswerRawText							1400

ALTdmGetNthValueForKeyGivenPath						1401

ALTdmGetNumAlphanums							1402

ALTdmGetNumAnswers								1403

ALTdmGetNumberOfNBestToUseAsBackup					1404

ALTdmGetNumConfirmations							1405

ALTdmGetNumDigits								1406

ALTdmGetNumNBestBackups							1407

ALTdmGetNumNoAnswers							1408

ALTdmGetNumRetries								1409

ALTdmGetNumTimeouts								1410

ALTdmGetNumWrongAnswers							1411

ALTdmGetPhoneNumberFormat							1412

ALTdmGetPlayingFailurePrompt							1413

ALTdmGetPlayingInitialPrompt							1414

ALTdmGetPlayingPercolatingSound						1415

ALTdmGetPlayingSuccessPrompt						1416

ALTdmGetPlayingYesNoDirectivePrompt						1417

ALTdmGetProcessingDTMF							1418

ALTdmGetPromptActiveLanguage						1419

ALTdmGetPromptName								1420

ALTdmGetRepromptN								1421

ALTdmGetRestrictingToEntries							1422

ALTdmGetSentenceCanonicalizingNumbers					1423

ALTdmGetSpellingAfterEndOfSpeechTimeout					1424

ALTdmGetSpellingBeforeBeginOfSpeechTimeout					1425

ALTdmGetSpellingConfirmationPromptN						1426

ALTdmGetSpellingMaxRetries							1427

ALTdmGetSpellingMaxTimeouts							1428

ALTdmGetSpellingNoAnswerApologyN						1429

ALTdmGetSpellingPrompt							1430

ALTdmGetSpellingRepromptN							1431

ALTdmGetSpellingTimeoutPromptN						1432

ALTdmGetSpellingWrongAnswerApologyN					1433

ALTdmGetSuccessPromptN							1434

ALTdmGetTimeOfDayDisambiguationMode					1435

ALTdmGetTimeOfDayGranularity							1436

ALTdmGetTimeOfDayHourOffset							1437

ALTdmGetTimeOfDayRange							1438

ALTdmGetTimeoutPromptN							1439

ALTdmGetUsingDisambiguation							1440

ALTdmGetUsingLookAheadSpelling						1441

ALTdmGetUsingSpellingAsFallback						1442

ALTdmGetWaveform								1443

ALTdmGetWhetherHadHighConfidenceScore					1444

ALTdmGetWhetherMadeSameMistakeTwice					1445

ALTdmGetWhetherUsedDisambiguation						1446

ALTdmGetWhetherUsedDTMFCollection						1447

ALTdmGetWhetherUsedSpelling							1448

ALTdmGetWrongAnswerApologyN						1449

ALTdmGetYesNoDirectivePrompt						1450

ALTdmGetZipCodeFormat							1451

ALTdmInitProc									1452

ALTdmInitSys									1453

ALTdmInitThread								1454

ALTdmPlayPrompt								1455

ALTdmPlayPromptWithData							1456

ALTdmRegisterCallFlowCallback							1457

ALTdmRegisterParseCallbackFunc						1458

ALTdmRegisterParseCallbackUserData						1459

ALTdmRegisterRuntimeSemanticModelCallback					1460

ALTdmRemoveAllDTMFCommands						1461

ALTdmRemoveAllEntries								1462

ALTdmRemoveCommand							1463

ALTdmRemoveDateItem								1464

ALTdmRemoveEntry								1465

ALTdmRemoveListItem								1466

ALTdmRemoveParseCallbackFunc						1467

ALTdmRemoveSynonymForNo							1468

ALTdmRemoveSynonymForYes							1469

ALTdmRemoveTimeOfDayItem							1470

ALTdmResetAllVocabulary							1471           	ALTdmResetClasses								1472

ALTdmResetCommandVocabulary						1473

ALTdmResetGrammar								1474

ALTdmResetListItemVocabulary							1475

ALTdmResetModuleState							1476

ALTdmResetRuntimeClasses							1477

ALTdmSessionCreate								1478

ALTdmSessionDestroy								1479

ALTdmSessionEnableListen							1480

ALTdmSessionEndCall								1481

ALTdmSessionGetAudience							1482

ALTdmSessionGetCallerID							1483

ALTdmSessionGetEventLog							1484

ALTdmSessionGetHaltReason							1485

ALTdmSessionGetHW								1486

ALTdmSessionGetName								1487

ALTdmSessionHalt								1488

ALTdmSessionNewCall								1489

ALTdmSessionNotifyDTMF							1490

ALTdmSessionNotifyHungUp							1491

ALTdmSessionNotifyReadDTMF							1492

ALTdmSessionPlayQueuedPrompts						1493

ALTdmSessionSetApplicationName						1494

ALTdmSessionSetAudience							1495

ALTdmSessionSetCallerID							1496

ALTdmSessionSetHW								1497

ALTdmSessionWaveCaptureResume						1498

ALTdmSessionWaveCaptureSuspend						1499

ALTdmSetActiveLanguage							1500

ALTdmSetAfterEndOfSpeechTimeout						1501

ALTdmSetAllowingBargeIn							1502

ALTdmSetAllowingSameMistakeTwice						1503

ALTdmSetApologizingForMistakes						1504

ALTdmSetBargeInThreshold							1505

ALTdmSetBeepingAfterPrompts							1506

ALTdmSetBeforeBeginOfSpeechTimeout						1507

ALTdmSetCannotDifferentiateYesNoPromptN					1508

ALTdmSetCCExpDateRange							1509

ALTdmSetCommandConfirmation						1510

ALTdmSetCommandConfirmationPromptN					1511

ALTdmSetConfirmationAfterEndOfSpeechTimeout				1512

ALTdmSetConfirmationBeforeBeginOfSpeechTimeout				1513

ALTdmSetConfirmationMaxRetries						1514

ALTdmSetConfirmationMaxTimeouts						1516

ALTdmSetConfirmationPromptN							1517

ALTdmSetConfirmationTimeoutPromptN						1518

ALTdmSetConsideringRepeatedLowConfidenceHypotheses			1519

ALTdmSetCreditCardSequenceLength						1520

ALTdmSetCreditCardTypeActivation						1521

ALTdmSetCurrencyDisambiguationMode						1522

ALTdmSetCurrencyGranularity							1523

ALTdmSetCurrencyRange							1524

ALTdmSetCurrencyType							1525

ALTdmSetDateDisambiguationMode						1526

ALTdmSetDateRange								1527

ALTdmSetDefaultConfirmation							1528

ALTdmSetDisallowed								1529

ALTdmSetDisambiguationBeforeBeginOfSpeechTimeout				1530

ALTdmSetDisambiguationConfirmationPromptN					1531

ALTdmSetDisambiguationMaxRetries						1532

ALTdmSetDisambiguationMaxTimeouts						1533

ALTdmSetDisambiguationNoAnswerApologyN					1534

ALTdmSetDisambiguationPrompt							1535

ALTdmSetDisambiguationRepromptN						1536

ALTdmSetDisambiguationTimeoutPromptN					1537

ALTdmSetDisambiguationWrongAnswerApologyN				1538

ALTdmSetDTMFConfirmationPromptN						1539

ALTdmSetDTMFDateFormat							1540

ALTdmSetDTMFDecimalPoint							1541

ALTdmSetDTMFLength								1542

ALTdmSetDTMFRepromptN							1543

ALTdmSetDTMFTerminator							1544

ALTdmSetDTMFTimeout								1545

ALTdmSetDTMFYesNo								1546

ALTdmSetFailurePrompt								1547

ALTdmSetInitialPrompt								1548

ALTdmSetListItemConfirmation							1549

ALTdmSetMaxAnswers								1550

ALTdmSetMaxRetries								1551

ALTdmSetMaxTimeouts								1552

ALTdmSetModuleID								1553

ALTdmSetNaturalNumberGranularity						1554

ALTdmSetNaturalNumberRange							1555

ALTdmSetNoAnswerApologyN							1556

ALTdmSetNumAlphanums							1557

ALTdmSetNumberOfNBestToUseAsBackup					1558

ALTdmSetNumDigits								1559

ALTdmSetPhoneNumberFormat							1560

ALTdmSetPlayingFailurePrompt							1561

ALTdmSetPlayingInitialPrompt							1562

ALTdmSetPlayingPercolatingSound						1563

ALTdmSetPlayingSuccessPrompt						1564

ALTdmSetPlayingYesNoDirectivePrompt						1565

ALTdmSetProcessingDTMF							1566

ALTdmSetPromptActiveLanguage						1567

ALTdmSetPromptCallbackFunc							1568

ALTdmSetReferenceDate							1569

ALTdmSetRepromptN								1570

ALTdmSetRestrictingToEntries							1571

ALTdmSetSentenceCanonicalizingNumbers					1572

ALTdmSetSentenceConfirmationCallback						1573

ALTdmSetSentenceParsingCallback						1574

ALTdmSetSpellingAfterEndOfSpeechTimeout					1575

ALTdmSetSpellingBeforeBeginOfSpeechTimeout					1576

ALTdmSetSpellingConfirmationPromptN						1577

ALTdmSetSpellingMaxRetries							1578

ALTdmSetSpellingMaxTimeouts							1579

ALTdmSetSpellingNoAnswerApologyN						1580

ALTdmSetSpellingPrompt							1581

ALTdmSetSpellingRepromptN							1582

ALTdmSetSpellingTimeoutPromptN						1583

ALTdmSetSpellingWrongAnswerApologyN					1584

ALTdmSetSuccessPromptN							1585

ALTdmSetTimeOfDayDisambiguationMode					1586

ALTdmSetTimeOfDayGranularity							1587

ALTdmSetTimeOfDayHourOffset							1588

ALTdmSetTimeOfDayRange							1589

ALTdmSetTimeoutPromptN							1590

ALTdmSetUsingDisambiguation							1591

ALTdmSetUsingLookAheadSpelling						1592

ALTdmSetUsingSpellingAsFallback						1593

ALTdmSetWrongAnswerApologyN						1594

ALTdmSetYesNoDirectivePrompt							1595

ALTdmSetZipCodeFormat							1596

ALTdmWaitForPrompt								1597

ALTdmUnregisterCallFlowCallback						1598

ALTdmParseCallbackDeleteKey							1599

ALTdmParseCallbackGetValueForKey						1600

ALTdmParseCallbackSetValueForKey						1601

SVServer Ease of Use Function Definitions

Each of these functions implement a set of Dialogic functions and setup all necessary SVSERVER structures as a means of simplifying the processing of commonly used commands.  Since these functions run a number of Dialogic functions an error in one of them may be indicated by a negative value of that function number being returned in ERR=.

ez_dial:		if on-hook go off-hook and dial a number

Dialogic	none	uses ATDX_HOOKST, dx_sethook, dx_dial, ATDV_LASTERR, 			ATDV_ERRMSGP

SVServer	ez_dial, dialstr, wait, default, nohook, mode

where	dialstrp = ascii dial string

wait = wait time delay for dialtone in milliseconds

default = set capp pointer to NULL uses basic call analysis, otherwise set to 0 if perfect call 			analysis

nohook = zero for analog lines, non-zero for dialing to a digital line (bypasses dx_sethook)

mode = synchronous + call analysis parameter

returns	results of call analysis if used or -1 if failure

e.g.	"ez_dial, 3305551234, 1000, NULL, 1"

-see	Dialogic function dx_dial Help file for parameter values

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

Use ~ tilda character for a pause comma in the dialstring.

Use & ampersand character for hookflash in the dialstring.

ez_getdigits:		initiates the collection of digits

Dialogic	none	uses dx_clrtpt, dx_getdig, ATDX_TERMMSK, ATDV_LASTERR, ATDV_ERRMSGP

SVServer	ez_getdigits, digits, digitmask, initial, silence, maxtime, tone, tptindex, mode

Or	xz_getdigits, tptindex, mode

where	digits = the number of digits to accumulate from 1 to 32

digitmask = specific digit(s)

intitial = initial silence period allowed before digit input to generate termination

silence = silence period allowed after digit input to generate termination

maxtime = time period allowed

tone = tone identifier for user defined tone

tptindex = the index of a tpt structure setup by the ut_set_dv_tpt and ut_save_setup functions

(for xz_getdigits, the DV_TPT structure must be setup prior to use for termination detection)

(for ez_getdigits, the settings in the tpt structure pointed to by tptindex are added to those built 			automatically, otherwise set to -1, tptindex is optional for ez_, and required for xz_ )

mode = must be synchronous

e.g.	"ez_getdigits , 4, 0, 40, 20, 300, 0, -1, 0"

-see	Dialogic function dx_getdig Help file for parameter values

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

ez_hangup:		tests hook state, if off-hook then it hangs up the line

Dialogic	none	uses ATDX_HOOKST, dx_sethook

SVServer	ez_hangup

ez_hookflash:		if off-hook go on-hook for time delay in milliseconds then back off-hook

Dialogic	none	uses ATDX_HOOKST, dx_sethook

SVServer	ez_hookflash, timeonhook

timeonhook = time between hangup and pickup, if no timeonhook then default = 500 ms

e.g.	"ez_hookflash" or "ez_hookflash, 500"

ez_pickup:		tests hook state, if on-hook then it picks up the line

Dialogic	none	uses ATDX_HOOKST, dx_sethook

SVServer	ez_pickup

ez_playfile:		plays a .vox or .wav file

Dialogic none	uses dx_clrtpt, dx_clrdigbuf, dx_playf or dx_playwav, ATDX_TERMMSK, ATDV_LASTERR, 			ATDV_ERRMSGP

SVServer	ez_playfile, fNamepath, digits, digitmask, tone, tptindex, mode

Or	xz_playfile, fNamepath, tptindex, mode

where	fNamepath = file path

digits = number of digits to allow input before generating termination

digitmask = specific digit(s) to allow input before generating termination

tone = tone identifier for user defined tone

tptindex = the index of a tpt structure setup by the ut_set_dv_tpt and ut_save_setup functions

(for xz_playfile, the DV_TPT structure must be setup prior to use for termination detection)

(for ez_playfile, the settings in the tpt structure pointed to by tptindex are added to those built 			automatically, otherwise set to -1, tptindex is optional for ez_, and required for xz_ )

mode = must be synchronous

e.g.	"ez_playfile, " & Trim$(CFGTempData.CFGApplicationText(2)) & ", 1, 0, 0, -1, 0"

-see	Dialogic function dx_playf Help file for parameter values

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

ez_playfiles:		plays a series of up to 32 files, one after the other in the order entered using dx_playiottdata

Dialogic none	uses dx_clrtpt, dx_clrdigbuf, dx_playiottdata, ATDX_TERMMSK, ATDV_LASTERR,

ATDV_ERRMSGP

SVServer	ez_playfiles, digits, digitmask, tone, tptindex, mode, fNamepath1, fNamepath2, ...

Or	xz_playfile, tptindex, mode, fNamepath1, fNamepath2, ... fNamepathn

where	digits = number of digits to allow input before generating termination

digitmask = specific digit(s) to allow input before generating termination

tone = tone identifier for user defined tone

tptindex = the index of a tpt structure setup by the ut_set_dv_tpt and ut_save_setup functions

(for xz_playfiles, the DV_TPT structure must be setup prior to use for termination detection)

(for ez_playfiles, the settings in the tpt structure pointed to by tptindex are added to those built 			automatically, otherwise set to -1, tptindex is optional for ez_, and required for xz_ )

mode = must be synchronous

fNamepath1, fNamepath2, ... fNamepathn = file paths

e.g.	"ez_playfiles, 1, 0, 0, -1, 0, " & Trim$(Filepath1) & ", " & Trim$(Filepath2)& ", " & Trim$(Filepath3)

Note: This command is useful for stringing numbers, dates, time, etc. together for play.

Note: This command opens all of the files first then uses the dx_playiottdata command to play the 			files.  The user must setup the DX_XPB structure.

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

ez_playfilex:		plays a .vox or .wav file, same as ez_playfile except you can specify the sampling rate in the xpb

structure.  Use ut_set_dx_xpb utility function to setup the sampling rate for .vox files.

Dialogic none	uses dx_clrtpt, dx_clrdigbuf, dx_playf or dx_playwav, ATDX_TERMMSK, ATDV_LASTERR,

ATDV_ERRMSGP

SVServer	ez_playfilex, fNamepath, digits, digitmask, tone, tptindex, mode

Or	xz_playfilex, fNamepath, tptindex, mode

where	fNamepath = file path

digits = number of digits to allow input before generating termination

digitmask = specific digit(s) to allow input before generating termination

tone = tone identifier for user defined tone

tptindex = the index of a tpt structure setup by the ut_set_dv_tpt and ut_save_setup functions

(for xz_playfilex, the DV_TPT structure must be setup prior to use for termination detection)

(for ez_playfilex, the settings in the tpt structure pointed to by tptindex are added to those built 			automatically, otherwise set to -1, tptindex is optional for ez_, and required for xz_ )

mode = must be synchronous

e.g.	"ez_playfilex, " & Trim$(CFGTempData.CFGApplicationText(2)) & ", 1, 0, 0, -1, 0"

-see	Dialogic function dx_playf Help file for parameter values

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

You must setup the DX_XPB structure before using this function.

ez_playtext:		plays a text file via text-to-speech

Dialogic none	uses dx_clrtpt, dx_clrdigbuf, dx_playf or tts_speakfile, ATDX_TERMMSK, ATDV_LASTERR, 			ATDV_ERRMSGP

SVServer	ez_playtext, fNamepath, digits, digitmask, tone, tptindex, mode

Or	xz_playtext, fNamepath, tptindex, mode

where	fNamepath = file path

digits = number of digits to allow input before generating termination

digitmask = specific digit(s) to allow input before generating termination

tone = tone identifier for user defined tone

tptindex = the index of a tpt structure setup by the ut_set_dv_tpt and ut_save_setup functions

(for xz_playtext, the DV_TPT structure must be setup prior to use for termination detection)

(for ez_playtext, the settings in the tpt structure pointed to by tptindex are added to those built 			automatically, otherwise set to -1, tptindex is optional for ez_, and required for xz_ )

mode = must be synchronous

e.g.	"ez_playtext, " & Trim$(CFGTempData.CFGApplicationText(2)) & ", 1, 0, 0, -1, 0"

-see	Dialogic function tts_speakfile Help file for parameter values

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

You must have purchased the Text-To-Speech option to use this function.

ez_recfile:		records a .vox or .wav file

Dialogic none	uses dx_clrtpt, dx_clrdigbuf, dx_recf or dx_recwav, ATDX_TERMMSK, ATDV_LASTERR, 			ATDV_ERRMSGP

SVServer	ez_recfile, fNamepath, digits, digitmask, initial, silence, tone, tptindex, mode

Or	xz_recfile, fNamepath, tptindex, mode

where	digits = the number of digits to accumulate before generating termination

digitmask = to allow input before generating termination

intitial = initial silence period allowed before voice to generate termination

silence = silence period allowed after voice to generate termination

tone = tone identifier for user defined tone

tptindex = the index of a tpt structure setup by the ut_set_dv_tpt and ut_save_setup functions

(for xz_recfile, the DV_TPT structure must be setup prior to use for termination detection)

(for ez_recfile, the settings in the tpt structure pointed to by tptindex are added to those built 			automatically, otherwise set to -1, tptindex is optional for ez_, and required for xz_ )

mode = must be synchronous

e.g.	"ez_recfile, " & Trim$(CFGTempData.CFGApplicationText(4)) & ", 1, 0, 40, 20, 0, -1, 1"

-see	Dialogic function dx_recf Help file for parameter values

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

ez_recfilex:		records a .vox or .wav, same as ez_recfile except you can specify the sampling rate in the xpb

structure.  Use ut_set_dx_xpb utility function to setup the sampling rate for .vox files.

Dialogic none	uses dx_clrtpt, dx_clrdigbuf, dx_recf or dx_recwav, ATDX_TERMMSK, ATDV_LASTERR, 			ATDV_ERRMSGP

SVServer	ez_recfilex, fNamepath, digits, digitmask, initial, silence, tone, tptindex, mode

Or	xz_recfilex, fNamepath, tptindex, mode

where	digits = the number of digits to accumulate before generating termination

digitmask = to allow input before generating termination

intitial = initial silence period allowed before voice to generate termination

silence = silence period allowed after voice to generate termination

tone = tone identifier for user defined tone

tptindex = the index of a tpt structure setup by the ut_set_dv_tpt and ut_save_setup functions

(for xz_recfilex, the DV_TPT structure must be setup prior to use for termination detection)

(for ez_recfilex, the settings in the tpt structure pointed to by tptindex are added to those built 			automatically, otherwise set to -1, tptindex is optional for ez_, and required for xz_ )

mode = must be synchronous

e.g.	"ez_recfile, " & Trim$(CFGTempData.CFGApplicationText(4)) & ", 1, 0, 40, 20, 0, -1, 1"

-see	Dialogic function dx_recf Help file for parameter values

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

You must setup the DX_XPB structure before using this function.

SVServer Special Status Function Definitions

stop:			stops all channel io, closes all channels and terminates the SVSERVER

Dialogic		none, uses dx_stopch and dx_close

SVServer	stop

Note: This command may be sent via a Line item or SVSCallback item.

stopch			stops input/output on the specified channel

Dialogic		None

SVServer	stopch, channel

where	channel is the channel number returned when the line was opened

uses dx_stopch in asynch mode

svstype:		returns the svserver type

Dialogic		none

SVServer	svstype

Note: This command must be sent via an SVSCallback item.

returns 	the svserver configuration including the libraries compiled

requires separate text control and separate item SVSCallback for events

Note: This command must be sent via the SVSCallback item to stop the io synchronous

operation blocking the channel.  This command returns nothing.  The i/o function stopped returns 			it's status.

separate additional text control and topic used for "stopch"

initialize		Text2.LinkMode = 0	'	break connection

Text2.LinkTopic = "SVServer|SVSFunction"

Text2.LinkItem = "SVSCallback"

Text2.LinkMode = 3	'	establish connection

then send	Text2.Text = "stopch, " & Trim$(Str$(Channel))

Text2.LinkPoke

This link responds only to the "stop", "svstype" and "stopch" commands.

SVServer Utility Function Definitions

This set of functions are provided as a means of allocating and deallocating space in SVSERVER for the structures required by Dialogic functions and supplying the parameters to fill them.  They also allow the user to examine the current setting of structure values.  The DV_TPT and DX_IOTT structures may be allocated by the user for each line where they are used.  They may be allocated via the ut_set_dv_tpt_size and ut_set_dx_iott_size functions respectfully.  All other structures in SVServer are automatically allocated, one set for each line as shown in the channel table below.

typedef struct {

int			chindex;

int			chdevice;

int			dxbddevice;

int			dtdevice;

int			dtbddevice;

int			fxdevice;

int			fxbddevice;

int			gcdevice;

int			lsdevice;

int			dcdevice;

int			dcbddevice;

int			msdevice;

int			msbddevice;

int			scdevice;

int			ttdevice;

int			chstatus;

int			cherrors;

long			chreturn;

long			chhandle;

char			*chresult;

char			*ddedata;

Args			*args;

int			argc;

int			curvol;

int			curspeed;

int			int_value;

int			function;

int			fhandle[32];

long			timecount;

DWORD		dwThreadId;

DWORD		dwSThreadId;

HANDLE		hThread;

HANDLE		sThread;

PDDETOPICINFO	pTopic;

PDDEITEMINFO	pItem;

CT_DEVINFO		ct_devinfo;

DV_DIGIT		dv_digit;

DV_TPT		*dv_tpt_ptr;	//	if used, must be allocated by  ut_set_dv_tpt_size

DX_CAP		dx_cap, *dx_cap_ptr;

DX_CST		*dx_cst_ptr;

DX_EBLK		dx_eblk;

EV_EBLK		dt_eblk, ms_eblk, dc_eblk;

DX_IOTT		*dx_iott_ptr;	//	if used, must be allocated by ut_set_dx_iott_size

DX_SVMT		dx_svmt;

DX_SVCB		dx_svcb[20];

DX_XPB		dx_xpb;

SC_TSINFO		sc_tsinfo;

TN_GEN		tn_gen;

int			stopmask;

int			dv_tpt_size;

int			dx_iott_size;

long			sc_ts;

char			string[MAXSTRING];

char			*bufptr[MAXBUFFERS];

void			*savptr[MAXBUFFERS];

long			size_savptr[MAXBUFFERS];

int			type_savptr[MAXBUFFERS];

#if DCB_LIB

MS_VOL		dcb_vol;

#endif

#if MSI_LIB

MS_NCB		ms_ncb;

#endif

#if MSI_LIB || DCB_LIB

MS_CDT		ms_cdt[2];

#endif

#if SCX_LIB

tSCX_BRDST		sc_brdst;

#endif

#if GLC_LIB

CRN			crn;

GC_CALLACK_BLK	gc_callack_blk;

GC_MAKECALL_BLK	gc_makecall_blk;

GC_IE_BLK		gc_ie_blk;

GC_L2_BLK		gc_l2_blk;

GC_RATE_U		gc_rate_u;

GC_PARM		gc_parm;

#endif

#if ALT_LIB

THEServiceData	sdm;

ALTserviceData		asd;

ALThwData		hwdata;

int			enabled;

#endif

} ChannelTable;

ChannelTable			chtable[MAXCHAN];

ut_allocate:		dynamically allocates a character memory buffer

Dialogic none	utility function normally implemented in C code

SVServer	ut_allocate, size

where	size = the number of bytes to allocate

Note: These buffer pointer indexes are not the same as those used by ez_save_setup.

returns	char buffer ptr index, refers to 1 of an array of 32 indexes per channel starting with 0

ut_deallocate:		deallocates a character memory buffer setup by ut_allocate

Dialogic	none	utility function normally implemented in C code

SVServer	ut_deallocate, index

where	index = char buffer ptr index returned from ut_allocate

Note: These buffer pointer indexes are not the same as those used by ez_save_setup.

ut_get_ca_param:	returns the setting for the parameter of the channels DX_CAP call analysis structure

Dialogic	 none	utility function normally implemented in C code

SVServer	ut_get_ca_param, param

where	-see ut_set_ca_param for param identification number 0 - 56 also in dxcallp.bas

returns	parameter value

ut_get_ca_params:	returns multiple parameter values of the channels DX_CAP call analysis structure

Dialogic	 none	utility function normally implemented in C code

SVServer	ut_get_ca_params, param1, param2, ... paramN

where	-see ut_set_ca_param for param identification number 0 - 56, also in dxcallp.bas

returns parameter values in (RSL=) in the order of input parameters value1, value2, ... valueN

ut_get_devices:	returns the array of devices which are used with ut_set_devices & sr_waitevtEx

Dialogic	none	utility function normally implemented in C code

SVServer	ut_get_devices

returns	dev, dev, ... dev

ut_get_dv_tpt:		returns the current setting for a channel's DV_TPT structure array element

Dialogic	none	utility function normally implemented in C code

SVServer	ut_get_dv_tpt, index

where	index = DV_TPT structure array index, 1, 2, 3, ... n

returns	in RSL: type, termno, length, flags, data, rfu

-see Dialogic Help file for DV_TPT structure values

ut_get_dx_iott:		returns the current setting for a channel's DX_IOTT structure array element

Dialogic	none	utility function normally implemented in C code

SVServer	ut_get_dx_iott, index

where	index = DX_IOTT structure array index, 1, 2, 3, ... n

returns	in RSL: type, rfu, fhandle, bufno, offset, length

-see Dialogic Help file for DX_IOTT structure values

ut_get_dx_svcb:	returns one of twenty dx_svcb structure entries for the channel

Dialogic	none	utility function normally implemented in C code

SVServer	ut_get_dx_svcb, index

where	index = array index value 0 - 19

returns	type, adjsize, digit, digtype

-see Dialogic Help file for DX_SVCB structure values

ut_get_dx_svmt:	returns the DX_SVMT structure

Dialogic	none	utility function normally implemented in C code

SVServer	ut_get_dx_svmt

returns	dec, dec, dec, dec, dec, dec, dec, dec, dec, dec, org, inc, inc, inc, inc, inc, inc, inc, inc, inc, inc

ut_get_dx_xpb:	returns the DX_XPB structure elements

Dialogic	none	utility function normally implemented in C code

SVServer	ut_get_dx_xpb

returns 	FileFormat, DataFormat, SamplesPerSec, BitsPerSample

-see Dialogic Help file for DX_XPB structure values

ut_free_setup:		releases a memory buffer which were setup by ut_save_setup

Dialogic	none	utility function normally implemented in C code

SVServer	ut_free_setup, index

where	index = buffer ptr index, refers to 1 of an array of 32 indexes per channel starting with 0

Note: These buffer pointer indexes are not the same as those used by ut_allocate

e.g.	"ut_free_setup, 0"

ut_load_setup:		reloads the DV_TPT, DX_IOTT, DX_CAP, DX_SVCB, or TN_GEN structure from a 			memory buffer previously saved using ut_save_setup

Dialogic	none	utility function normally implemented in C code

SVServer	ut_load_setup, index

where	index = memory buffer pointer index returned from ut_save_setup

Note: These buffer pointer indexes are not the same as those used by ut_allocate.

e.g.	"ut_load_setup, 0"

returns	the number of array elements loaded

ut_save_setup:	allocates a memory buffer and saves the setup for the channels current setting of

DV_TPT, DX_IOTT, DX_CAP, DX_SVCB, or TN_GEN structure for reuse via ut_load_setup

Dialogic none	utility function normally implemented in C code

SVServer	ut_save_setup, type		( allocation utility )	( creation utility )

where	type = 	1 - DV_TPT_TYPE	ut_set_dv_tpt_size	ut_set_dv_tpt

2 - DX_IOTT_TYPE	ut_set_dx_iott_size	ut_set_dx_iott

3 - DX_CAP_TYPE

4 - DX_SVCB_TYPE	saves an array of twenty = dx_svcb[20]

5 - TN_GEN_TYPE

e.g.	"ut_save_setup, 1"

returns	the memory buffer ptr index, 1 of an array of 32 indexes per channel starting with 0

Note: You must save this index in order to reload the structure from the buffer.

Note: These structures are initially allocated and created using ( ut_ ) utility functions. The 			saving and reloading of these structures eliminates having to create the structures with each use.

Note: These buffer pointer indexes are not the same as those used by ut_allocate

ut_set_ca_param:	sets the parameter value for the channels DX_CAP call analysis structure

Dialogic	none	utility function normally implemented in C code

SVServer	ut_set_ca_param, param, value

Note: -See  dxcallp.bas for parameters

where	param = 			value =	-see Dialogic Help for call analysis values

0	vs_ca_nbrdna		# of rings before no answer.

1	vs_ca_stdely		Delay after dialing before analysis.

2	vs_ca_cnosig		Duration of no signal time out delay.

3	vs_ca_lcdly		Delay after dial before lc drop connect

4	vs_ca_lcdly1		Delay after lc drop con. before msg.

5	vs_ca_hedge		Edge of answer to send connect message.

6	vs_ca_cnosil		Initial continuous noise timeout delay.

7	vs_ca_lo1tola		acceptable pos. dev of short low sig.

8	vs_ca_lo1tolb		acceptable neg. dev of short low sig.

9	vs_ca_lo2tola		% acceptable pos. dev of long low sig.

10	vs_ca_lo2tolb		% acceptable neg. dev of long low sig.

11	vs_ca_hi1tola		% acceptable pos. dev of high signal.

12	vs_ca_hi1tolb		% acceptable neg. dev of high signal.

13	vs_ca_lo1bmax		Maximum interval for shrt low for busy.

14	vs_ca_lo2bmax		Maximum interval for long low for busy.

15	vs_ca_hi1bmax		Maximum interval for 1st high for busy

16	vs_ca_nsbusy		Num. of highs after nbrdna busy check.

17	vs_ca_logltch		Silence deglitch duration.

18	vs_ca_higltch		Non-silence deglitch duration.

19	vs_ca_lo1rmax		Max. short low  dur. of double ring.

20	vs_ca_lo2rmin		Min. long low  dur. of double ring.

21	vs_ca_intflg		Operator intercept mode.

22	vs_ca_intfltr		Minimum signal to qualify freq. detect.

23	vs_rfu1			reserved for future use

24	vs_rfu2			reserved for future use

25	vs_rfu3			reserved for future use

26	vs_rfu4			reserved for future use

27	vs_ca_hisiz		Used to determine which lowmax to use.

28	vs_ca_alowmax		Max. low before con. if high >hisize.

29	vs_ca_blowmax		Max. low before con. if high <hisize.

30	vs_ca_nbrbeg		Number of rings before analysis begins.

31	vs_ca_hi1ceil		Maximum 2nd high dur. for a retrain.

32	vs_ca_lo1ceil		Maximum 1st low dur. for a retrain.

33	vs_ca_lowerfrq		Lower allowable frequency in hz.

34	vs_ca_upperfrq		Upper allowable frequency in hz.

35	vs_ca_timefrq		Total duration of good signal required.

36	vs_ca_rejctfrq		Allowable % of bad signal.

37	vs_ca_maxan		Maximum duration of answer.

38	vs_ca_ansrdgl		Silence deglitching value for answer.

39	vs_ca_mxtimefrq	max time for 1st freq to remain in bounds

40	vs_ca_lower2frq		lower bound for second frequency

41	vs_ca_upper2frq		upper bound for second frequency

42	vs_ca_time2frq		min time for 2nd freq to remains in bounds

43	vs_ca_mxtime2frq	max time for 2nd freq to remain in bounds

44	vs_ca_lower3frq		lower bound for third frequency

45	vs_ca_upper3frq		upper bound for third frequency

46	vs_ca_time3frq		min time for 3rd freq to remains in bounds

47	vs_ca_mxtime3frq	max time for 3rd freq to remain in bounds

48	vs_ca_dtn_pres		Length of a valid dial tone (def=1sec)

49	vs_ca_dtn_npres	Max time to wait for dial tone (def=3sec)

50	vs_ca_dtn_deboff	The dialtone off debouncer (def=100ms)

51	vs_ca_pamd_failtime	Wait for AMD/PVD after cadence break(default=4sec)

52	vs_ca_pamd_minring	min allowable ring duration (def=1.9sec)

53	vs_ca_pamd_spdval	char Set to 2 selects quick decision (def=1)

54	vs_ca_pamd_qtemp	char The Qualification template to use for PAMD

55	vs_ca_noanswer	time before no answer after 1st ring default=30

56	vs_ca_maxintering	Max inter ring delay before connect (8 sec)

ut_set_ca_params:	sets multiple parameter values for the channels DX_CAP call analysis structure

Dialogic	none	utility function normally implemented in C code

SVServer	ut_set_ca_params, param1, value1, param2, value2 ... paramN, valueN

Note: -See  dxcallp.bas for parameters

ut_set_devices:	sets the array of devices used with sr_waitevtEx

Dialogic	none	utility function normally implemented in C code

SVServer	ut_set_devices, dev, dev, ... dev

where	dev = channel device number returned when the line device is opened

Note: Sending ut_set_devices with no devices clears the devices array.

ut_set_dv_tpt:		sets up the dv_tpt structure array element

Dialogic	none	utility function normally implemented in C code

SVServer	ut_set_dv_tpt, index, type, termno, length, flags, data, rfu

where	where	index = tpt stucture array index value 0, 1, 2, ...

-see Dialogic Help file for DV_TPT structure values

Note: Type must not be a linked list IO_LINK.

ut_set_dv_tpt_size:	allocates space for the dv_tpt structure

Dialogic	none	utility function normally implemented in C code

SVServer	ut_set_dv_tpt_size, size

where	size = the number of dv-tpt structure entries

Note: dx_clrtpt allocates the dv_tpt structure as well

ut_set_dx_iott:		sets up the dx_iott structure array element

Dialogic	none	utility function normally implemented in C code

SVServer	ut_set_dx_iott, index, type, rfu, fhandle, bufno, offset, length

where	where	index = iott stucture array index value 0, 1, 2, ...

bufno = character memory buffer pointer index allocated by ut_allocate if IO_MEM is set in type

fhandle = file handle returned by dx_fileopen if IO_DEV is set in type, otherwise 0

-see Dialogic Help file for DX_IOTT structure values

Note: Type must not be a linked list IO_LINK.

ut_set_dx_iott_size:	allocates space for the dx_iott structure

Dialogic	none	utility function normally implemented in C code

SVServer	ut_set_dx_iott_size, size

where	size = the number of dx_iott structure entries

ut_set_dx_svcb:	sets up one of twenty dx_svcb structure entries for the channel or clears the array

Dialogic	none	utility function normally implemented in C code

SVServer	ut_set_dx_svcb, index, type, adjsize, digit, digtype

or	ut_set_dx_svcb, clearall	clears the entire 20 entry array if clearall is = 1 or nonzero

where	index = array index value 0 - 19

-see Dialogic Help file for DX_SVCB structure values

ut_set_dx_svmt:	sets up the dx_svmt structure

Dialogic	none	utility function normally implemented in C code

SVServer	ut_set_dx_svmt, value1, value2, value3, ... value21

where	-see Dialogic Help file for DX_SVMT structure values

ut_set_dx_xpb:	sets up the dx_xpb structure

Dialogic	none	utility function normally implemented in C code

SVServer	ut_set_dx_xpb, wFileFormat, wDataFormat, nSamplesPerSec, nBitsPerSample

where	-see Dialogic Help file for DX_XPB structure values

ut_set_timer:		sets interal timing functions in the SVServer

Dialogic none	utility function normally implemented in C code

SVServer	ut_set_timer, interval, show

where	interval = the timer interval in milliseconds

show = the timer count SVServer window stays open

default	ut_set_timer, 10, 300

Note: The user should not change these values unless advised by the developer.

Dialogic Analog And Attribute Function API Definitions

ag_getctinfo:		returns information about the analog device.

Dialogic		int ag_getctinfo( int chdev, CT_DEVINFO *ct_devinfop );

SVServer	ag_getctinfo

returns  All of the CT_DEVINFO fields are returned in the SVServer result field (RSL=) delimited by a 			comma in the order they appear in the CT_DEVINFO structure.

ag_getxmitslot:	returns SCbus time slot number of the analog transmit channel.

Dialogic		int ag_getxmitslot( int chdev, SC_TSINFO *sc_tsinfop );

SVServer	ag_getxmitslot

ag_listen:		connects analog receive channel to SCbus time slot.

Dialogic		int ag_listen( int chdev, SC_TSINFO *sc_tsinfop );

SVServer	ag_listen, timeslot#

ag_unlisten:		disconnects analog receive channel from SCbus.

Dialogic		int ag_unlisten( int chdev );

SVServer	ag_unlisten

ATD4_BDTYPE:	Returns the D/42-xx board type of the queried device.

Dialogic		int ATD4_BDTYPE ( int iBoard )

SVServer	ATD4_BDTYPE, ddd

where	ddd = The device handle

ATD4_CHTYPE:	Returns the D/42-xx channel type of the queried device.

Dialogic		int ATD4_CHTYPE ( int iChannel )

SVServer	ATD4_CHTYPE, ddd

where	ddd = The device handle

ATDT_BDMODE:	Returns the current mode of every time slot on the specified Digital Network Interface device.

Dialogic		long ATDT_BDMODE ( int bddev )

SVServer	ATDT_BDMODE

Note: returns a long bitmap in (RTN=)

ATDT_BDSGBIT:	Returns the current state of the transmit and receive , bits for all time slots on the Digital Network 			Interface.

Dialogic		char * ATDT_BDSGBIT( int bddev )

SVServer	ATDT_BDSGBIT

Note: returns state in the (RSL=) field

ATDT_DNLDVER:	Returns the firmware version downloaded to the device.

Dialogic		long ATDT_DNLDVER( int bddev )

SVServer	ATDT_DNLDVER

ATDT_IDLEST:		Returns the current idle state of the Digital Network Interface time slot

Dialogic		long ATDT_IDLEST( int tsdevh )

SVServer	ATDT_IDLEST

ATDT_ROMVER:	Returns the version of the EPROM that is installed on the Digital Network Interface device.

Dialogic		long ATDT_ROMVER( int bddev )

SVServer	ATDT_ROMVER

ATDT_STATUS:	Returns the current status of the Digital Network Interface time slot.

Dialogic		long ATDT_STATUS( int tsdevh )

SVServer	ATDT_STATUS

ATDT_TSMODE:	Returns the current signaling mode of the time slot.

Dialogic		long ATDT_TSMODE( int tsdevh )

SVServer	ATDT_TSMODE

ATDT_TSSGBIT:	 Retrieves the current state of the transmit and receive signaling bits for the time slot.

Dialogic		long ATDT_TSSGBIT( int tsdevh )

SVServer	ATDT_TSSGBIT

ATDV_ERRMSGP:	returns an ASCIIZ error message string

Dialogic	char *ATDV_ERRMSGP( long ddd );

SVServer	ATDV_ERRMSGP, ddd

where	ddd = The device handle from dx_open, dx_open_board, dt_open, dt_open_board etc.

ATDV_IOPORT:	returns the port number

Dialogic	long ATDV_IOPORT( long ddd );

SVServer	ATDV_IOPORT, ddd

where	ddd = The device handle from dx_open, dx_open_board, dt_open, dt_open_board etc.

ATDV_IRQNUM:	returns the interrupt (irq) number

Dialogic	long ATDV_IRQNUM( long ddd );

SVServer	ATDV_IRQNUM, ddd

where	ddd = The device handle from dx_open, dx_open_board, dt_open, dt_open_board etc.

ATDV_LASTERR:	returns a long that indicates the error that occurred

Dialogic	long ATDV_LASTERR( long ddd );

SVServer	ATDV_LASTERR, ddd

where	ddd = The device handle from dx_open, dx_open_board, dt_open, dt_open_board etc.

ATDV_NAMEP:		returns an ASCIIZ string which contains the device name

Dialogic	char *ATDV_NAMEP( long ddd );

SVServer	ATDV_NAMEP, ddd

where	ddd = The device handle from dx_open, dx_open_board, dt_open, dt_open_board etc.

ATDV_SUBDEVS:	returns the number of subdevices for the device

Dialogic	long ATDV_SUBDEVS( long ddd );

SVServer	ATDV_SUBDEVS, ddd

where	ddd = The device handle

ATDX_ANSRSIZ:	returns the duration of the answer

Dialogic		long ATDX_ANSRSIZ( int chdev );

SVServer	ATDX_ANSRSIZ

ATDX_BDNAMEP:	returns the board name

Dialogic		char * ATDX_BDNAMEP( int chdev );

SVServer	ATDX_BDNAMEP

ATDX_BDTYPE:	returns the board type

Dialogic		long ATDX_BDTYPE( int chdev );

SVServer	ATDX_BDTYPE

ATDX_BUFDIGS:	returns the number of uncollected digits

Dialogic		long ATDX_BUFDIGS( int chdev );

SVServer	ATDX_BUFDIGS

ATDX_CHNAMES:	returns an array of channel names

Dialogic		char ** ATDX_CHNAMES( int bddev );

SVServer	ATDX_CHNAMES, index

where	index = the channel number of the channel on the board, e.g. = 0, 1, 2, 3

ATDX_CHNUM:	returns the channel number

Dialogic		long ATDX_CHNUM( int chdev );

SVServer	ATDX_CHNUM

ATDX_CONNTYPE:	returns the connection type

Dialogic		long ATDX_CONNTYPE( int chdev );

SVServer	ATDX_CONNTYPE

ATDX_CPERROR:	returns the error

Dialogic		long ATDX_CPERROR(  int chdev );

SVServer	ATDX_CPERROR

ATDX_CPTERM:	returns the last Call Analysis termination

Dialogic		long ATDX_CPTERM(   int chdev);

SVServer	ATDX_CPTERM

ATDX_CRTNID:	returns the tone identifier

Dialogic		long ATDX_CRTNID( int chdev );

SVServer	ATDX_CRTNID

ATDX_DTNFAIL:	returns the code for the dial tone that failed to appear

Dialogic		long ATDX_DTNFAIL( int chdev );

SVServer	ATDX_DTNFAIL

ATDX_DBMASK:	no documentation

Dialogic		long ATDX_DBMASK( int chdev );

SVServer	ATDX_DBMASK

ATDX_DEVTYPE:	returns the device type

Dialogic		long ATDX_DEVTYPE( int dev );

SVServer	ATDX_DEVTYPE, dev

where	dev = device handle for the board or channel

ATDX_DIGBUFMODE:

Dialogic		long ATDX_DIGBUFMODE( int chdev );

SVServer	ATDX_DIGBUFMODE

ATDX_EVTCNT:

Dialogic		long ATDX_EVTCNT( int chdev );

SVServer	ATDX_EVTCNT

ATDX_FRQDUR:	returns the first frequency duration in 10ms units

Dialogic		long ATDX_FRQDUR( int chdev  );

SVServer	ATDX_FRQDUR

ATDX_FRQDUR2:	returns the second frequency duration in 10ms units

Dialogic		long ATDX_FRQDUR2(  int chdev  );

SVServer	ATDX_FRQDUR2

ATDX_FRQDUR3:	returns the third frequency duration in 10ms units

Dialogic		long ATDX_FRQDUR3( int chdev  );

SVServer	ATDX_FRQDUR3

ATDX_FRQHZ:		returns the frequency of the answered signal in hz

Dialogic		long ATDX_FRQHZ( int chdev  );

SVServer	ATDX_FRQHZ

ATDX_FRQHZ2:	returns the frequency of the second detected in hz

Dialogic		long ATDX_FRQHZ2( int chdev  );

SVServer	ATDX_FRQHZ2

ATDX_FRQHZ3:	returns the frequency of the third detected in hz

Dialogic		long ATDX_FRQHZ3( int chdev  );

SVServer	ATDX_FRQHZ3

ATDX_FRQOUT:	returns the percentage of a single tone frequency

Dialogic		long ATDX_FRQOUT( int v );

SVServer	ATDX_FRQOUT

ATDX_FWVER:	returns the version number of the D/4x firmware

Dialogic		long ATDX_FWVER( int chdev  );

SVServer	ATDX_FWVER

ATDX_HOOKST:	returns the current hook state

Dialogic		long ATDX_HOOKST( int chdev  );

SVServer	ATDX_HOOKST

ATDX_LINEST: 	returns a bitmapped representation of the activity

Dialogic		long ATDX_LINEST( int chdev  );

SVServer	ATDX_LINEST

ATDX_LONGLOW:	returns the duration of the longer silence

Dialogic		long ATDX_LONGLOW( int chdev  );

SVServer	ATDX_LONGLOW

ATDX_NUMCHAN:

Dialogic		long ATDX_NUMCHAN( int bddev  );

SVServer	ATDX_NUMCHAN

ATDX_PHYADDR:	returns the physical address of the board

Dialogic		long ATDX_PHYADDR( int bddev );

SVServer	ATDX_PHYADDR

ATDX_PRODUCT:

Dialogic		long ATDX_PRODUCT( int chdev );

SVServer	ATDX_PRODUCT

ATDX_RINGCNT:	returns the ring count

Dialogic		long ATDX_RINGCNT( int chdev );

SVServer	ATDX_RINGCNT

ATDX_RXDATABUFSIZE:	no documentation

Dialogic		long ATDX_RXDATABUFSIZE(INT SrlDevice );

SVServer	ATDX_RXDATABUFSIZE

ATDX_SHORTLOW:	returns the duration of the shorter silence

Dialogic		long ATDX_SHORTLOW( int ddd );

SVServer	ATDX_SHORTLOW

ATDX_SIZEHI:		returns the duration of the initial non-silence

Dialogic		long ATDX_SIZEHI( int chdev );

SVServer	ATDX_SIZEHI

ATDX_STATE:		returns the current state of the channel

Dialogic		long ATDX_STATE( int chdev );

SVServer	ATDX_STATE

ATDX_TERMMSK:	returns the channel's last termination bitmap

Dialogic		long ATDX_TERMMSK( int chdev );

SVServer	ATDX_TERMMSK

ATDX_TONEID:	returns the user-defined tone id

Dialogic		long ATDX_TONEID( int chdev);

SVServer	ATDX_TONEID

ATDX_TRCOUNT:	returns the number of the last play/record transfer count

Dialogic		long ATDX_TRCOUNT( int chdev );

SVServer	ATDX_TRCOUNT

ATMS_DNLDVER:	returns the MSI/SC firmware version that was downloaded to the device

Dialogic		long ATMS_DNLDVER(int devh)

SVServer	ATMS_DNLDVER

ATMS_FEMINFO:	returns the firmware information

Dialogic		char *ATMS_FEMINFO(int devh)

SVServer	ATMS_FEMINFO

ATMS_NUMCHAN:	no documentation

Dialogic		int ATMS_NUMCHAN(int devh)

SVServer	ATMS_NUMCHAN

ATMS_STATINFO:	returns information about the MSI/SC board.

Dialogic		int ATMS_STATINFO(int devh, char *statinfop)

SVServer	ATMS_STATINFO

ATMS_TSSGBIT:	retrieves the current station hook status

Dialogic		long ATMS_TSSGBIT(int devh)

SVSerever	ATMS_TSSGBIT

Dialogic Voice API Function Definitions

Please consult the Dialogic Help files for information on the input parameters and return values for these functions.  Each line, or channel, has it's own set of structures with pointers set automatically but the user must supply parameters to fill the structures via the utility functions provided above or Dialogic functions below.  The user must also, allocate space for each channel's DV_TPT and DX_IOTT structures when called for.  They may be allocated via the ut_set_dv_tpt_size and ut_set_dx_iott_size functions respectfully. Channel device handles need not be included unless indicated by the SVServer command shown in bold typeface.

dx_addspddig:		sets a DTMF digit to adjust speed

Dialogic		int dx_addspddig( int chdev, char digit, short adjval );

SVServer	dx_addspddig, digit, adjval

dx_addtone:		adds the tone

Dialogic		int dx_addtone( int chdev, unsigned char digit, unsigned char digtype );

SVServer	dx_addtone, digit, digtype

dx_addvoldig:		sets a DTMF digit to immediately adjust volume

Dialogic		int dx_addvoldig( int chdev, char digit, short adjval );

SVServer	dx_addvoldig, digit, adjval

dx_adjsv:		adjusts the speed or the volume

Dialogic		int dx_adjsv( int chdev, unsigned short tabletype, unsigned short action, unsigned short 			adjsize );

SVServer	dx_adjsv, tabletype, action, adjsize

dx_BargeIn:		no documentation

Dialogic		int dx_BargeIn( int chDev, DX_IOTT *lpPlayIott, DX_IOTT *lpRecordIott, const DV_TPT 			*lpPlayTerminations, const DV_TPT *lpRecordTerminations, DX_XPB *lpPlayParams, USHORT 			Mode);

SVSever	function not implemented, documentation is not available

dx_blddt:		defines a simple dual frequency tone

Dialogic		int dx_blddt( unsigned int id,   unsigned freq1, unsigned int fq1dev, unsigned int freq2, 			unsigned int fq2dev, unsigned int mode );

SVServer	dx_blddt, id, freq1, fq1dev, freq2, fq2dev, mode

dx_blddtcad:		defines a simple dual frequency cadence tone

Dialogic		int dx_blddtcad( unsigned int ToneID, unsigned int freq1,   unsigned int fq1dev, unsigned 			int freq2, unsigned int fq2dev, unsigned int OnTime,  unsigned int OnTimeDev, unsigned int 			OffTime, unsigned int OffTimeDev, unsigned int repcnt );

SVServer	dx_blddtcad, ToneID, freq1, fq1dev, feq2, fq2dev, OnTime, OnTimeDev, OffTime, 			OffTimeDev, repcnt

dx_bldst:		defines a simple single frequency tone

Dialogic		int dx_bldst( unsigned int ToneID, unsigned int freq, unsigned int fqdev, unsigned int 			mode );

SVServer	dx_bldst, ToneID, freq, fqdev, mode

dx_bldstcad:		defines a simple single frequency cadence tone

Dialogic		int dx_bldstcad( unsigned int ToneID, unsigned int freq, unsigned int fqdev, unsigned int 			OnTime,  unsigned int OnTimeDev, unsigned int OffTime, unsigned int OffTimeDev, unsigned 			int repcnt );

SVServer	dx_bldstcad, ToneId, freq, fqdev, OnTime, OnTimeDev, OffTime, OffTimeDev, 			repcnt

dx_bldtngen:		sets up the tone generation template

Dialogic		void dx_bldtngen( TN_GEN *tngenp, unsigned short freq1, unsigned short freq2,short 			amplitude1, short amplitude2, short duration );

SVServer	dx_bldtngen, freq1, freq2, amplitude1, amplitude2, duration

dx_chgdur:		alters the standard definition of the duration component

Dialogic		int dx_chgdur( int typetype, int on, int ondv, int off, int offdv );

SVServer	dx_chgdur, typetype, on, ondv, off, offdv

dx_chgfreq:		changes the standard definition for one of the PerfectCall Call Analysis tones

Dialogic		int dx_chgfreq( int tonetype, int fq1, int dv1, int fq2, int dv2 );

SVServer	dx_chgfreq, tonetype, fq1, dv1, fq2, dv2

dx_chgqualid:		no documentation

Dialogic		int __cdecl dx_chgqualid( int qualid );

SVServer	dx_chgqualid, qualid

where	qualid = not documented

dx_chgrepcnt:		changes the standard definition for one of the PerfectCall Call Analysis tones

Dialogic		int dx_chgrepcnt( int tonetype, int repcount );

SVServer	dx_chgrepcnt, tonetype, repcount

dx_close:		closes Dialogic devices

Dialogic		int dx_close( int dev );

SVServer	dx_close

dx_clrcap:		clears all the fields in a DX_CAP structure

Dialogic		void dx_clrcap( DX_CAP *capp );

SVServer	dx_clrcap

or	dx_clrcap, NULL

where	NULL sets the capp pointer to NULL

dx_clrdigbuf:		clears the digits present in the firmware digit buffer

Dialogic		int dx_clrdigbuf( int chdev );

SVServer	dx_clrdigbuf

dx_clrsvcond:		clears any speed or volume adjustment conditions

Dialogic		int dx_clrsvcond( int chdev );

SVServer	dx_clrsvcond

dx_clrtpt:		allocates and clears the DV_TPT structure

Dialogic		int dx_clrtpt( DV_TPT *tptp, int size );

SVServer	dx_clrtpt, size

dx_comtest:		no documentation

Dialogic		int __cdecl dx_comtest( int chdev );

SVServer	dx_comtest

dx_deltones:		removes all user-defined tones

Dialogic		int dx_deltones( int chdev );

SVServer	dx_deltones

dx_dial:		dials an ASCIIZ string

Dialogic		int dx_dial( int chdev, const char *dialstrp, const DX_CAP *capp, unsigned short mode );

SVServer	dx_dial, dialstr, mode

Note: You must run dx_clrcap prior to this function. dx_clrcap sets the DX_CAP pointer used by 			dx_dial.

dx_distone:		disables the detection of TONE ON

Dialogic		int dx_distone( int chdev, int toneID, int evtMask );

SVServer	dx_distone, toneID, evtMask

dx_enbtone:		enables the detection of TONE ON

Dialogic		int dx_enbtone( int chdev, int ToneID, int evtMask );

SVServer	dx_enbtone, ToneID, evtMask

dx_filechsize:		no documentation

Dialogic		int dx_filechsize ( int handle, long size );

SVServer	dx_filechsize, handle, size

dx_filecommit:		no documentation

Dialogic		int dx_filecommit ( int handle );

SVServer	dx_filecommit, handle

dx_fileclose:		closes the file associated with the handle

Dialogic		int dx_fileclose ( int handle );

SVServer	dx_fileclose, handle

dx_filedup:		no documentation

Dialogic		int dx_filedup ( int handle );

SVServer	dx_filedup, handle

dx_filedup2:		no documentation

Dialogic		int dx_filedup2 ( int handle1, int handle2 );

SVServer	dx_filedup2, handle1, handle2

dx_fileerrno:		no documentation

Dialogic		int dx_fileerrno( void );

SVServer	dx_fileerrno

dx_filefilelength:	no documentation

Dialogic		long dx_filefilelength( int handle );

SVServer	dx_filefilelength, handle

dx_filefstat:		no documentation

Dialogic		int dx_filefstat ( int handle, struct _stat * buffer );

SVServer	function not implemented

dx_fileisatty:		no documentation

Dialogic		int dx_fileisatty( int handle );

SVServer	dx_fileisatty, handle

dx_filelocking:		no documentation

Dialogic		int dx_filelocking ( int handle, int mode, long bytes );

SVServer	dx_filelocking, handle, mode, bytes

dx_fileopen:		opens the file specified by filep

Dialogic		int dx_fileopen ( const char * filep, int flags, int pmode );

SVServer	dx_fileopen, filepath, flags, pmode

dx_fileopen_osfhandle:	no documentation

Dialogic		int dx_fileopen_osfhandle( long osfhandle, int flags );

SVServer	dx_fileopen_osfhandle, osfhandle, flags

dx_fileread:		returns the number of bytes read by the application

Dialogic		int dx_fileread ( int handle, void * buffer, unsigned int count );

SVServer	dx_fileread, handle, bufptr_index, count

where	handle is the file handle returned by dx_fileopen

bufptr_index is the index of a buffer allocated by ut_allocate

count is the number of bytes to read

dx_fileseek:		moves the file pointer associated with handle

Dialogic		long dx_fileseek ( int handle, long offset, int origin );

SVServer	dx_fileseek, handle, offset, origin

dx_filesetmode:	no documentation

Dialogic		int dx_filesetmode( int handle, int mode );

SVServer	dx_filesetmode, handle, mode

dx_filesopen:		no documentation

Dialogic		int dx_filesopen ( const char * filep, int oflag, int shflag, int pmode );

SVServer	dx_filesopen, filepath, oflag, shflag, pmode

dx_filestat:		no documentation

Dialogic		int dx_filestat ( const char * path, struct _stat * buffer );

SVServer	function not implemented

dx_filetell:		no documentation

Dialogic		long dx_filetell ( int handle );

SVServer	dx_filetell, handle

dx_fileumask:		no documentation

Dialogic		int dx_fileumask ( int pmode );

SVServer	dx_fileumask, pmode

dx_filewrite:		writes count byes from buffer into the file associated with handle

Dialogic		int dx_filewrite ( int handle, void * buffer, unsigned int count );

SVServer	dx_filewrite, handle, bufptr_index, count

where	handle is the file handle returned by dx_fileopen

bufptr_index is the index of a buffer allocated by ut_allocate

count is the number of bytes to write

dx_getctinfo:		returns information about voice device

Dialogic		int dx_getctinfo( int chDev, CT_DEVINFO *ct_devinfop );

SVServer	dx_getctinfo

returns  All of the CT_DEVINFO fields are returned in the SVServer result field (RSL=) delimited by a 			comma in the order they appear in the CT_DEVINFO structure.

dx_getcursv:		returns the specified channel's current speed

Dialogic		int dx_getcursv( int chdev, int *curvolp, int *curspeedp );

SVServer	dx_getcursv

dx_getdig:		initiates the collection of digits

Dialogic		int dx_getdig( int chdev, const DV_TPT *tptp, DV_DIGIT *digitp, unsigned short mode );

SVServer	dx_getdig, mode

dx_getevt:		used to synchronously monitor channels

Dialogic		int dx_getevt( int chdev, DX_EBLK *eblkp, int timeout );

SVServer	dx_getevt, timeout

returns	if successful, returns dt_eblk.ev_event in ( RTN= ) and dt_eblk.ev_data in ( RSL= )

dx_getparm:		obtains the current parameter settings

Dialogic		int dx_getparm( int dev, unsigned long parm, void * valuep );

SVServer	dx_getparm, parm

or	dx_getparm, dev, parm

dx_getr2mf:		no documentation

Dialogic		int dx_getr2mf( int Chdev, const char *src1_ptr, unsigned short src1_len, const char 			*src2_ptr, unsigned short src2_len,char *dest_ptr, unsigned short dest_len, unsigned short mode 			);

SVServer	dx_getr2mf, src1index, src1len, src2index, src2len, dstindex, dstlen, mode

dx_getsvmt:		returns the contents of the Speed or the Volume Modification Table

Dialogic		int dx_getsvmt( int ChDev, unsigned short tableType, DX_SVMT *svmtp );

SVServer	dx_getsvmt, tableType

dx_getxmitslot:	Function provides SCbus time slot number of voice transmit channel.

Dialogic		int dx_getxmitslot( int chDev, SC_TSINFO *sc_tsinfop);

SVServer	dx_getxmitslot

dx_gtcallid:		returns the calling line Directory Number (DN) sent by the Central Office

Dialogic		int dx_gtcallid( int Chdev, unsigned char *bufp );

SVServer	dx_gtcallid

returns	the caller's phone number in the (RSL=) field

dx_gtextcallid:		 returns the requested Caller ID message by specifying the Message Type ID

Dialogic		int dx_gtextcallid( int Chdev, int infotype, unsigned char *bufp );

SVServer	dx_gtextcallid, infotype

where	infotype = Message Type ID

returns 	the requested Caller ID message in the (RSL=) field

dx_gtsernum:		 returns the board serial number, either the standard serial number or the silicon serial number

Dialogic		int dx_gtsernum ( int devd, int subfcn, void *pBuff );

SVServer	dx_gtsernum, subfcn

returns 	the board serial number in the (RSL=) field

dx_initcallp:		initializes and activates PerfectCall Call Analysis

Dialogic		int dx_initcallp( int chdev );

SVServer	dx_initcallp

Note: dx_deltones must be sent before dx_initcallp or Dialogic will return -1 failure

dx_InitializeBargeIn:	no documentation

Dialogic		int __cdecl dx_InitializeBargeIn( int Chdev, const DV_TPT * tptp, DX_XPB *xpbp, USHORT 			RecordMode);

SVServer	function not implemented, no documentation

dx_listen:		Connects voice receive channel to SCbus time slot.

Dialogic		int dx_listen( int chDev, SC_TSINFO *sc_tsinfop );

SVServer	dx_listen, timeslot

where	timeslot = The transmit side of the timeslot connection.

dx_listenecr:		enables ECR mode echo cancellation on a specified voice channel and connects the voice channel 			to the echo-referenced signal on the specified SCbus time slot.

Dialogic		int __cdecl dx_listenecr( int chDev, SC_TSINFO *sc_tsinfop );

SVServer	dx_listenecr, timeslot

where	timeslot = valid time-slot number which was obtained by issuing a call to xx_getxmitslot

dx_listenecrex:		performs identically to dx_listenecr and also modifies the characteristics of the echo canceller.

Dialogic		int __cdecl dx_listenecrex( int chDev, SC_TSINFO *sc_tsinfop, void *dx_ecrctp );

SVServer	dx_listenecr, timeslot, flag

where	timeslot = valid time-slot number which was obtained by issuing a call to xx_getxmitslot

flag = NLPflag enable, disable flag

dx_mreciottdata:	records voice data from two SCbus time slots.  The data may be recorded to a combination of data 			files, memory or custom devices.

Dialogic		int __cdecl dx_mreciottdata( int ChDev,DX_IOTT * iottp, const DV_TPT *tptp, DX_XPB * 			xpbp,unsigned short mode, SC_TSINFO *lpTimeSlot);

SVServer	dx_mreciottdata, ts1, ts2, mode

where	ts1 = timeslot 1

ts2 = timeslot 2

mode = switch to set audible tone, or DTMF termination

Note: You must setup the DX_IOTT, DV_TPT, and DX_XPB structures before issuing this function

(-See the utility functions ut_set_dx_iott_size, ut_set_dx_iott,

ut_set_dv_tpt_size, ut_set_dv_tpt and ut_set_dx_xpb )

dx_open:		opens a voice channel device

Dialogic		int dx_open( const char *namep, int oflags );

SVServer	dx_open, namep

where	namep = device name e.g. "dxxxB1C1"

returns 	the channel device handle in ( RTN= )

dx_open_board:	opens a voice channel device

Dialogic		int dx_open( const char *namep, int oflags );

SVServer	dx_open_board, namep

where	namep = device name e.g. "dxxxB1C1"

returns 	the board device handle in ( RTN= )

Note: For calls that require an open board device rather than a channel device.

dx_play:		plays recorded voice data

Dialogic		int dx_play( int ChDev, DX_IOTT *iottp, const DV_TPT *tptp, unsigned short mode );

SVServer	dx_play, mode

where	DX_IOTT and DV_TPT must be setup by ut_set_dx_iott and ut_set_dv_tpt prior to running

mode = synchronous

dx_playf:		permits you to synchronously play voice data

Dialogic		int dx_playf(int ChDev, const char *fNamep, const DV_TPT *tptp, unsigned short mode 			);

SVServer	dx_playf, fNamepath, mode

where	DV_TPT must be setup by ut_set_dv_tpt prior to running

fNamepath =  file path

mode = synchronous

dx_playiottdata:

Dialogic		int dx_playiottdata( int ChDev, DX_IOTT * iottp, const DV_TPT * tptp, const DX_XPB * 			xpbp, unsigned short mode );

SVServer	dx_playiottdata, mode

where	DX_IOTT, DV_TPT & DX_XPB must be setup by ut_set_dx_iott, ut_set_dv_tpt & 			ut_set_dx_xpb prior to running

mode = synchronous

dx_playtone:		plays the tone defined by the TN_GEN template

Dialogic		int dx_playtone( int ChDev, const TN_GEN *tngenp, const DV_TPT *tptp, int mode );

SVServer	dx_playtone, mode

dx_playvox:		plays voice data stored in a single VOX file

Dialogic		int dx_playvox( int ChDev, const char * fNamep, const DV_TPT *tptp, const DX_XPB * 			xpbp,unsigned short mode );

SVServer	dx_playvox, fNamepath, mode

or	dx_playvox, fNamepath, NULL, mode

where	DV_TPT and DX_XPB must be setup by ut_set_dv_tpt and ut_set_dx_xpb

fNamepath =  file path

mode = synchronous

dx_playwav:		plays voice data stored in a single WAVE file

Dialogic		int dx_playwav( int ChDev, const char * fNamep, const DV_TPT *tptp, unsigned short 			mode );

SVServer	dx_playwav, fNamepath, mode

where	DV_TPT must be setup by ut_set_dv_tpt prior to running

fNamepath =  file path

mode = synchronous

dx_readsram:		no documentation

Dialogic		int __cdecl dx_readsram(int ChDev, PUCHAR pBuffer, int BufferSize, int Offset, PCHAR 			pFileName );

SVServer	function not implemented, no documentation

dx_rec:			records voice data from a single channel

Dialogic		int dx_rec(int ChDev, DX_IOTT *iottp, const DV_TPT *tptp, unsigned short mode );

SVServer	dx_rec, mode

where	DX_IOTT and DV_TPT must be setup by ut_set_dx_iott and ut_set_dv_tpt prior to running

mode = synchronous

dx_recf:		permits voice data to be recorded

Dialogic		int dx_recf( int ChDev, const char *fNamep, const DV_TPT *tptp, unsigned short mode );

SVServer	dx_recf, fNamepath, mode

where	DV_TPT must be setup by ut_set_dv_tpt prior to running

fNamepath =  file path

mode = synchronous

dx_reciottdata:		records voice data to multiple destinations

Dialogic		int dx_reciottdata( int ChDev,DX_IOTT * iottp, const DV_TPT *tptp, DX_XPB * 			xpbp,unsigned short mode );

SVServer	dx_reciottdata, mode

where	DX_IOTT, DV_TPT & DX_XPB must be setup by ut_set_dx_iott, ut_set_dv_tpt & 			ut_set_dx_xpb prior to running

mode = synchronous

dx_recvox:		records voice data to a single VOX file

Dialogic		int dx_recvox( int ChDev, const char * fNamep, const DV_TPT *tptp, DX_XPB * 			xpbp,unsigned short mode );

SVServer	dx_recvox, fNamepath, mode

or	dx_recvox, fNamepath, NULL, mode

where	DV_TPT and DX_XPB must be setup by ut_set_dv_tpt and ut_set_dx_xpb 			fNamepath =  file path

mode = synchronous

dx_recwav:		records voice data to  a single WAVE file

Dialogic		int dx_recwav( int ChDev, const char * fNamep, const DV_TPT *tptp, DX_XPB * 			xpbp,unsigned short mode );

SVServer	dx_recwav, fNamepath, mode

or	dx_recwav, fNamepath, NULL, mode

where	DV_TPT and DX_XPB must be setup by ut_set_dv_tpt and ut_set_dx_xpb

fNamepath =  file path

mode = synchronous

dx_route:		not documented

Dialogic		int dx_route( int BdDev, int Chan, int TimeSlot );

SVServer	dx_route, BdDev, Chan, TimeSlot

dx_routerxtx:		not documented

Dialogic		int dx_routerxtx( int BdDev, int Chan, int RxTimeSlot, int TxTimeSlot );

SVServer	dx_routerxtx, BdDev, Chan, RxTimeSlot, TxTimeSlot

dx_RxIottData:		no documentation

Dialogic		int __cdecl dx_RxIottData( int chDev, DX_IOTT *iottp, const DV_TPT *lpTerminations, int wType, 			LPVOID lpParams, USHORT mode);

SVServer	function not implemented, no documentation

dx_sendevt:		to be implemented

Dialogic		int dx_sendevt( int devd, unsigned long evttype, void *evtdatap, short evtlen, unsigned 			short flags );

SVServer	function not yet implemented

dx_SendReadRequest:		no documentation

Dialogic		int __cdecl dx_SendReadRequest( INT SrlDevice, PCHAR Buffer, DWORD DataLength, 			DXRWROUTINE ReadCompletionRoutine );

SVServer	function not implemented, no documentation

dx_SendWriteRequest:	no documentation

Dialogic		int __cdecl dx_SendWriteRequest( INT SrlDevice, PCHAR Buffer, DWORD DataLength, DWORD 			BulkFlags, DXRWROUTINE WriteCompletionRoutine   );

SVServer	function not implemented, no documentation

dx_set_lasterr:		not documented

Dialogic		int dx_set_lasterr( int ddd, long error );

SVServer	dx_set_lasterr, error

dx_setchxfercnt:	sets the bulk queue buffer size for the channel. This function can change the size of the buffer 			used to transfer voice data between a user application and the Dialogic hardware.

Dialogic		int __cdecl dx_setchxfercnt(int chDev, int bufnum);

SVServer	dx_setchxfercnt, xx_chdev, bufnum

where	xx_chdev = specifies the valid device handle obtained when the device was opened using 			xx_open(), where "xx" is the prefix identifying the device to be opened.

bufnum = specifies the bulk queue buffer size for the channel. Use one of the following values:

1	Sets the buffer size to 8K (default).

0	Sets the buffer size to 4K.

4	Sets the buffer size to 2K.

6	Sets the buffer size to 1.5K.

dx_setdevuio:		enables the application to install and retrieve user-defined I/O functions on a per Dialogic channel 			device basis.

Dialogic		int __cdecl dx_setdevuio(int chDev, DX_UIO *devuiop, DX_UIO **retuiop);

SVServer	function cannot be implemented in a general way

dx_setdigbuf:		sets the digit buffering mode that will be used by the Voice Driver. Once the digit buffer is full (31 			digits), the application may select whether subsequent digits will be ignored or will overwrite the 			oldest digits in the queue.

Dialogic		int __cdecl dx_setdigbuf( int ChDev, int Mode );

SVServer	dx_setdigbuf, mode

where	mode = specifies the type of digit buffering that will be used.

dx_setdigtyp:		controls the types of digits

Dialogic		int dx_setdigtyp( int ChDev, unsigned short DigitMask );

SVServer	dx_setdigtyp, DigitMask

dx_setevtmsk:		enables detection of the Call Status Transition (CST) event

Dialogic		int dx_setevtmsk( int ChDev, unsigned int mask );

SVServer	dx_setevtmsk, mask

dx_setgtdamp:		sets up the amplitudes

Dialogic		void dx_setgtdamp( short Min1, short Max1, short Min2, short Max2 );

SVServer	dx_setgtdamp, Min1, Max1, Min2, Max2

dx_sethook:		provides control of the hookswitch state

Dialogic		int dx_sethook( int ChDev, int HookState, unsigned short mode );

SVServer	dx_sethook, HookState, mode

dx_setparm:		allows you to set the physical parameters

Dialogic		int dx_setparm( int Dev, unsigned long parm, const void * valuep );

SVServer	dx_setparm, parm, value

or	dx_setparm, dev, parm, value

dx_setrings:		not documented

Dialogic		int dx_setrings( int devd, unsigned int nrings );

SVServer	dx_setrings, nrings

dx_setsvcond:		sets adjustments and adjustment conditions

Dialogic		int dx_setsvcond( int ChDev, unsigned short numBlks, const DX_SVCB *svcbp );

SVServer	dx_setsvcond, numBlks

dx_setsvmt:		updates the speed or volume

Dialogic		int dx_setsvmt( int ChDev, unsigned short TableType, const DX_SVMT *svmtp, 			unsigned short flag );

SVServer	dx_setsvmt, TableType, flag

dx_settonelen:		changes the duration of the built-in beep tone (sometimes referred to as a pre-record beep), which 			some application programs make use of to indicate the start of a recording or playback.

Dialogic		int __cdecl dx_settonelen( int tonelength );

SVServer	dx_settonelen, tonelength

where	tonelength = specifies the duration of the tone in 200 ms units. Default: 1 (200 ms).

dx_setuio:		allows an application to install user-defined read(), write(), and lseek() functions

Dialogic		int dx_setuio( DX_UIO uio );

SVServer	function cannot be implemented in a general way

dx_stopch:		forces termination of currently active I/O functions

Dialogic		int dx_stopch( int ChDev, unsigned short mode );

SVServer	dx_stopch, chDev, mode

where	chDev = the channel to be stopped.

Note: This function must be run on a channel other than the one being stopped.

Please See stopch convienience function above.

dx_TSFStatus:		not documented

Dialogic		int dx_TSFStatus ( void );

SVServer	dx_TSFStatus

dx_TxIottData:		not documented

Dialogic		int __cdecl dx_TxIottData( int chDev, DX_IOTT *iottp, const DV_TPT *lpTerminations, int wType, 			LPVOID lpParams, USHORT mode);

SVServer	function not implemented, no documentation

dx_RxIottData:		not documented

Dialogic		int __cdecl dx_RxIottData( int chDev, DX_IOTT *iottp, const DV_TPT *lpTerminations, int wType, 			LPVOID lpParams, USHORT mode);

SVServer	function not implemented, no documentation

dx_unlisten:		not documented

Dialogic		int dx_unlisten( int chDev );

SVServer	dx_unlisten

dx_wink:		not documented

Dialogic		int dx_wink( int ChDev, unsigned short mode );

SVServer	dx_wink, mode

dx_wtcallid:		waits for rings and reports Caller ID, if available. Using this function is equivalent to using the voice 			functions dx_setevtmsk() and dx_getevt(), and the Caller ID function dx_gtcallid() to return the 			caller's Directory Number (DN).

Dialogic		int dx_wtcallid( int Chdev, int nrings, int timeout,unsigned char *bufp );

SVServer	dx_wtcallid, nrings, timeout

returns 	the caller's Directory Number (DN).

dx_wtring:		waits for a specified number of rings

Dialogic		int dx_wtring( int ChDev, int numRings, int HookState, int timeout );

SVServer	dx_wtring, numRings, HookState, timeout

case vs_PBXpertInit:	no documentation

Dialogic		int __cdecl PBXpertInit ( void );

SVServer	function not implemented, documentation is not available

r2_creatfsig:		defines and enables leading edge detection of an R2MF forward signal on a channel.

Dialogic		int __cdecl r2_creatfsig(int chDev, int toneid);

SVServer	r2_creatfsig, toneid

where	toneid = group I/II forward signal

r2_playbsig:		plays a tone and controls the timing sequence required by the R2MF compelled signaling 			procedure.

Dialogic		int __cdecl r2_playbsig(int chDev, int backwardsig, int forwardsig, int mode);

SVServer	r2_playbsig, backwardsig, forwardsig, mode

where	backwardsig = group A/B backward signal

forwardsig = group I/II forward signal

mode = set to synchronous mode

Dialogic SCBus (SCTools.C) API Routing Function Definitions

nr_scroute:		Create a half or full duplex connection between two SCBus devices.

Dialogic		int nr_scroute( 	int devh1, unsigned short devtype1,

int devh2, unsigned short devtype2, unsigned char mode )

SVServer	nr_scroute, devh1, devtype1, devh2, devtype2, mode

Or	nx_scroute, chan1, devtype1, chan2, devtype2, mode

where	devh1 = First SCBus device handle.

devtype1 = Specifies the type for devh1.

devh2 = Second SCBus device handle.

devtype2 = Specifies the type for devh2.

chan1, chan2 are the logical line numbers 0, 1 .. N, the device handles are stored in SVServer

mode = SC_FULLDUP = 0 or SC_HALFDUP = 1

(-See Dialogic SCTools.c)

Device Types	SC_VOX       	0x01   		Voice channel device

SC_LSI       	0x02     		Analog Timeslot device

SC_DTI       	0x03  		Digital Timeslot device

SC_FAX      	0x04		Fax channel device

SC_MSI       	0x05 		MSI channel device

nr_scunroute:		Break the half or full duplex connection between two SCBus devices.

Dialogic		int nr_scroute( 	int devh1, unsigned short devtype1,

int devh2, unsigned short devtype2, unsigned char mode )

SVServer	nr_scunroute, devh1, devtype1, devh2, devtype2, mode

nx_scunroute, chan1, devtype1, chan2, devtype2, mode

where	devh1 = First SCBus device handle.

devtype1 = Specifies the type for devh1.

devh2 = Second SCBus device handle.

devtype2 = Specifies the type for devh2.

chan1, chan2 are the logical line numbers 0, 1 .. N, the device handles are stored in SVServer

mode = SC_FULLDUP = 0 or SC_HALFDUP = 1

(-See Dialogic SCTools.c)

Dialogic DTI API Function Definitions

dt_close:		closes a dti device channel

Dialogic		int  dt_close( int tsdev )

SVServer	dt_close

dt_dial:			pulse dials via a dti device channel

Dialogic		int  dt_dial( int tsdev, char * dialstr, unsigned int tmo )

SVServer	dt_dial, dialstring, tmo

where	dialstring = The ASCIIZ string of digits to dial. The maximum length of the string is 32 digits.

tmo = Specifies the maximum number of seconds that the function will block while awaiting a dial 				status response from the D/240SC-T1 or D/300SC-E1 .

dt_getctinfo:		Gets SCbus digital interface information. This function returns information about the digital network 			interface device associated with the specified digital channel (time slot) (dtiBxTx).

Dialogic		int  dt_getctinfo( int tsdev, CT_DEVINFO *devinfop )

SVServer	dt_getctinfo

returns  All of the CT_DEVINFO fields are returned in the SVServer result field (RSL=) delimited by a 			comma in the order they appear in the CT_DEVINFO structure.

dt_getevt:		The dt_getevt( ) function blocks and returns control to the program after one of the events set by 			dt_setevtmsk( ) occurs on the channel specified in the devh parameter, or a timeout occurs.

Dialogic		int  dt_getevt(int tsdev, EV_EBLK * eblkp, int tmo)

SVServer	dt_getevt, tmo

where	tmo = Specifies the maximum amount of time in seconds to wait for an event to occur.

if successful, returns dt_eblk.ev_event in ( RTN= ) and dt_eblk.ev_data in ( RSL= )

dt_getevtmsk:		The dt_getevtmsk( ) function retrieves the current event bitmask(s) for the specified event type and 			Digital Network Interface logical board or time slot device.  The function can be used to find which 			bitmask was set by the dt_setevtmsk() function.

Dialogic		int  dt_getevtmsk( int tsdev, int event, unsigned short * bitmaskp )

SVServer	dt_getevtmsk, event

where	event specifies which event's bitmask will be retrieved.

Note: The bitmask is returned in the (RSL=) field;

dt_getparm:		Gets the current value of the selected Digital Network Interface device parameter.

Dialogic		int  dt_getparm(int tsdev, unsigned long param, void * valuep)

SVServer	dt_getparm, param

where	param = Specifies the parameter to be examined.

Note: The value of the parameter is returned in the (RSL=) field;

dt_getxmitslot:		Returns the SCbus time slot connected to the transmit of a digital network interface device time slot 			on a D/240SC-T1 or D/300SC-E1 board.

Dialogic		int  dt_getxmitslot(int tsdev, SC_TSINFO *tsinfop)

SVServer	dt_getxmitslot

Note: sc_tsinfo.sc_numts is set to 1

The timeslot is returned in the (RTN=) field

(-See also: nr_scroute, nr_scunroute above)

dt_hookflash:		go on-hook for time delay in milliseconds then back off-hook

Dialogic	none	uses dt_settssigsim

SVServer	ez_hookflash, timeonhook, setonbitmask, setoffbitmask

where	timeonhook = time between hangup and pickup, suggested timeonhook=500 ms

setonbitmask and setoffbitmask depend on the A and B bits for the line type

dt_listen:		Connects the receive (listen) of a digital network interface device time slot on a D/240SC-T1 or 			D/300SC-E1 board to an SCbus time slot.

Dialogic		int  dt_listen(int tsdev, SC_TSINFO *tsinfop)

SVServer	dt_listen, timeslot

where	timeslot = The transmit side of the timeslot connection.

(-See also: nr_scroute, nr_scunroute above)

dt_mtfcn:		Initiates or stops the multitasking (asynchronous) function specified by the function ID (fncid) 			parameter for a DTI/212 time slot.

Dialogic		int  dt_mtfcn(int tsdev, unsigned char fncid, unsigned int tmo)

SVServer	dt_mtfcn, fncid, tmo

where	fncid = parameter specifies the function to initiate or the multitasking function to abort.

tmo = Specifies the maximum amount of time in seconds that the function will block while awaiting a 			response from the DTI/212.

dt_open:		Opens a Digital Network Interface device and returns a unique Dialogic handle to identify the 			device.

Dialogic		int  dt_open(char *name, int oflags)

SVServer	dt_open, name, flags

where	name = ASCIIZ string that contains the name of a valid Digital Network Interface time slot device

returns 	the time slot device handle in ( RTN= )

dt_open_board:	Opens a Digital Network Interface board and returns a unique Dialogic handle.

Dialogic		int  dt_open(char *name, int oflags)

SVServer	dt_open_board, name, flags

where	name = ASCIIZ string that contains the name of a valid Digital Network Interface logical board

returns 	the board device handle in ( RTN= )

Note: For calls that require an open board device rather than a time slot device.

dt_route:

Dialogic		int  dt_route( int devh, int chan, int tms, int mode )

SVServer	dt_route, chan, tms, mode

where	chan = ASCIIZ string that contains the name of a valid Digital Network Interface logical board or 			time slot device

dt_rundiag:		Runs diagnostics on the Network firmware.

Dialogic		int  dt_rundiag( int devh, unsigned int tmo, char * diagbufp )

SVServer	dt_rundiag, tmo

where	tmo = Specifies the length of time in seconds the function will block while waiting for a response 			from the device.

Note: The diagnostic code is returned in the (RSL=) field;

dt_set_lasterr:

Dialogic		void  dt_set_lasterr(int devh, int error)

SVServer	dt_set_lasterr, error

dt_setalrm:		Sets the Digital Network Interface device to one of three alarm handling modes.

Dialogic		int  dt_setalrm( int devh, unsigned int mode )

SVServer	dt_setalrm, mode

dt_setevtmsk:		Enables and disables notification for events that occur on a Digital Network Interface logical board 			or time slot device.

Dialogic		int  dt_setevtmsk( int devh, int event, unsigned short bitmask, int action )

SVServer	dt_setevtmsk, event, bitmask, action

where	event = Specifies the type of event to be enabled or disabled

bitmask = Specifies the event to be enabled or disabled by setting the bitmask for that event

action = Specifies how the signaling bit transition event mask is changed.

dt_setidle:		Enables or disables transmission of silence to the network for the audio portion of the specified 			time slot.

Dialogic		int  dt_setidle( int devh, unsigned int state )

SVServer	dt_setidle, state

where	state = Specifies whether to enable or disable the transmission of silence

dt_setiostatus:

Dialogic		int  dt_setiostatus( int devh, int status )

SVServer	dt_setiostatus, status

dt_setparm:		Changes the value of a device parameter

Dialogic		int  dt_setparm( int devh, unsigned long param, void * valuep )

SVServer	dt_setparm, param, value

where	param = Specifies the parameter value to alter.

value = Value to be assigned to the parameter

dt_setsigmod:		Sets the type of signaling that will be performed on the transmitted time slot

Dialogic		int  dt_setsigmod( int devh, unsigned int mode )

SVServer	dt_setsigmod, mode

where	mode = Specifies the transmit mode.

dt_settssig:		Sets or clears the transmit  for the time slot requested

Dialogic		int  dt_settssig ( int devh, unsigned short bitmask, int action )

SVServer	dt_settssig, bitmask, action

where	bitmask = Specifies which signaling bits to change.

action = Specifies whether the signaling bits in the mask should be set or cleared,

dt_settssigsim:	Allows simultaneous setting or clearing of the transmit signaling bits on a Digital Network Interface 			time slot.

Dialogic		int  dt_settssigsim( int devh, unsigned short bitmask )

SVServer	dt_settssigsim, bitmask

where	bitmask = Specifies which signaling bits to change.

dt_tstcom:		Tests the ability of a Digital Network Interface device to communicate with the host PC.

Dialogic		int  dt_tstcom( int devh, unsigned int tmo )

Svserver	dt_tstcom, tmo

where 	tmo = Specifies the maximum amount of time in seconds that the function will block while waiting 			for a response from the Digital Network Interface.

dt_tstdat:		Performs a test that verifies the integrity of the Digital Network Interface I/O interface to the PC.

Dialogic		int  dt_tstdat( int devh, unsigned int tmo )

where 	tmo = Specifies the maximum amount of time in seconds that the function will block while waiting 			for a response from the Digital Network Interface.

dt_unlisten:		Disconnects the receive of a digital network interface device time slot on a D/240SC-T1 or 			D/300SC-E1 board from the SCbus time slot.

Dialogic	int  dt_unlisten( int devh )

SVServer	dt_unlisten

dt_wt_clear:		Terminates an event thread created by dt_wt_event with stop = mask

Dialogic	 	none

SVServer	dt_wt_clear

dt_wt_event:		Waits for an event on digital line. Combines dt_setevtmsk and dt_getevt

Dialogic	 none	Uses dt_setevtmsk, dt_getevt, ATDV_LASTERR, ATDV_ERRMSGP

SVServer	dt_wt_event, event, bitmask, action, tmo, stop

(-See Dialogic documentation for dt_setevtmsk for event, bitmask, & action )

where	event = Specifies the type of event to be enabled or disabled

bitmask = Specifies the event to be enabled or disabled by setting the bitmask for that event

action = Specifies how the signaling bit transition event mask is changed.

tmo = Specifies the maximum amount of time in seconds to wait for an event to occur,

-1 = forever, (-See Dialogic documentation for dt_getevt() for timeout )

if the stop value = 0, no extra thread is created and the channel is blocked by dt_getevt.

returns RSL =  value as follows:

( bit 3 = 1 for BON), ( bit 2 = 1 for BOFF ), ( bit 1 = 1 for AON), ( bit 0 = 1 for AOFF )

if error or timeout ERR = ATDV_LASTERR()

stop should be set to 0 for ring detection

stop = mask, a non-zero value, a thread is created to catch the event and a dx_stopch is issued to 			the channel when the event occurs according to the AB bits in the stop mask value as follows:

( bit 3 = 1 for BON), ( bit 2 = 1 for BOFF ), ( bit 1 = 1 for AON), ( bit 0 = 1 for AOFF )

if the event with the stop mask occurs then the thread is cleared on the next command to SVServer,

otherwise a dt_wt_clear must be issued to terminate the thread.

Note: The stop mask is for detecting a hang up transition so that the channel i\o may be ended.

Set the stop dt_wt_event after you pickup the call and issue a dt_wt_clear after call completion.

After each i\o function check for TM_USRSTOP to determine if dx_stopch was issued

dt_xmitalrm:		Starts and stops transmission of an alarm to a network span.

Dialogic		int  dt_xmitalrm( int devh, unsigned char alarmtype, unsigned int state )

SVServer	dt_xmitalrm, alarmtype, state

where	alarmtype = Specifies the T-1 or E-1 alarm type to be transmitted.

state = Specifies whether to enable or disable transmission of the specified alarm:

dt_xmittone:		Enables or disables transmission of a test tone to the PEB on the specified DTI/211 or D/240SC-T1 			time slot.

Dialogic		int  dt_xmittone( int devh, unsigned int state )

SVServer	dt_xmittone, state

where	state = Specifies tone on or off

dt_xmitwink:		Transmits wink signaling to the T-1 or E-1 network span on any of the available signaling bits.

Dialogic		int  dt_xmitwink(int devh, unsigned int tmo)

SVServer	dt_xmitwink, tmo

where 	tmo = Specifies the maximum amount of time in seconds that the function will block while waiting 			for a response from the Digital Network Interface.

Dialogic SCBus API Function Definitions

scx_close:		Closes an SCBus device handle

Dialogic		scx_close

SVServer	scx_close

scx_clrevtmsk:		Stops the generation of the alarm events specified in the mask parameter.

Dialogic		int scx_clrevtmsk(short int devh, unsigned short int mask)

SVServer	scx_clrevtmsk, mask

scx_getbrdparm:	Retrieves board parameters.  Parameters may be informative, defaults or values previously set by 			the scx_setbrdparm() function.

Dialogic		int scx_getbrdparm(short int devh, unsigned char param, void *valuep)

SVServer	scx_getbrdparm, param

where	param = an unsigned char which specifies the name of the parameter

scx_getctinfo:		Gets Adapter device information.

Dialogic		int scx_getctinfo(short int devh, CT_DEVINFO *devinfop)

SVServer	scx_getctinfo

returns  All of the CT_DEVINFO fields are returned in the SVServer result field (RSL=) delimited by a 			comma in the order they appear in the CT_DEVINFO structure.

scx_getevtmsk:	 Retrieves alarm event mask.  This function identifies the events enabled for a specified station.

Dialogic		int scx_getevtmsk(short int devh, unsigned short *bitmaskp)

SVServer	scx_getevtmsk

scx_getver:		Not documented

Dialogic		int scx_getver(short int devh)

SVServer	scx_getver

scx_gtbdattr:		Gets board attributes of hardware and firmware information for a specific SCX160 SCxbus Adapter

Dialogic		int scx_gtbdattr(short int devh, tSCX_BRDST *datbufp)

SVServer	scx_gtbdattr

scx_open:		Opens an SCX160 device and returns a unique Dialogic handle to identify the device.

Dialogic		int scx_open(char *name, short int oflags)

SVServer	scx_open, scxBus

where	scxBus = the name (scxB1) of the SCX160 SCxbus Adapter.

scx_setbrdparm:	Changes board level parameters.

Dialogic		int scx_setbrdparm(short int devh, unsigned char param, void *valuep)

SVServer	scx_setbrdparm, param, value

scx_setevtmsk:	Enables alarm events specified in the event mask

Dialogic		int scx_setevtmsk(short int devh, unsigned short int mask)

SVServer	scx_setevtmsk, mask

where	mask = Specifies the event mask.

scx_tstcom:		Runs a communication test to verify that the SCX160 SCxbus Adapter can communicate with the 			host computer.

Dialogic		int scx_tstcom(short int devh, short int tmo)

SVServer	scx_tstcom, tmo

where 	tmo = Specifies the maximum amount of time in seconds that the function will block while waiting 			for a response from the SCX160 SCxbus Adapter.  If a response is not received within tmo 			seconds, an error is returned.  A suggested setting is 5 seconds.

scx_tstdat:		Runs a data test to verify the integrity of the host computer-to-SCX160 SCxbus Adapter Shared 			RAM interface for the requested SCX160 SCxbus Adapter board.

Dialogic		int scx_tstdat(short int devh, short int tmo)

SVServer	scx_tstdat, tmo

where 	tmo = Specifies the maximum amount of time in seconds that the function will block while waiting 			for a response from the SCX160 SCxbus Adapter.  If a response is not received within tmo 			seconds, an error is returned.  A suggested setting is 5 seconds.

Dialogic SRL API Function Definitions

Please consult the Dialogic Help files for information on the input parameters and return values for these functions.

sr_dishdlr:		disables the handler	( Reserved For Asynchronous Applications Only )

Dialogic		long sr_dishdlr( long ddd, unsigned long event_type, long (*hdlr)( void *parm ) );

SVServer	function not implemented for synchronous applications

See dx_getevt, dt_getevt, or dt_wt_event

sr_enbhdlr:		enables the handler function	( Reserved For Asynchronous Applications Only )

Dialogic		long sr_enbhdlr( long ddd, unsigned long event_type, long (*hdlr)( void *parm ) );

SVServer	function not implemented for synchronous applications

Use dx_getevt, dt_getevt, or dt_wt_event

sr_getboardcnt:	retrieve the count of boards of a particular type

Dialogic		long sr_getboardcnt( char * brdname, int * brdcnt );

SVServer	sr_getboardcnt, brdname, brdcnt

sr_getevtdatap:	returns the address of the variable data block

Dialogic		void *sr_getevtdatap( unsigned long eventhandle );

SVServer	sr_getevtdatap, eventhandle

where	eventhandle is returned when sr_waitevtEx completes

returns	in RSL: "EVT=%d, DTA=%d"

sr_getevtdev:		returns the Dialogic device handle

Dialogic		long sr_getevtdev( unsigned long event );

SVServer	sr_getevtdev, event

sr_getevtlen:		returns the length of the variable data

Dialogic		long sr_getevtlen( unsigned long eventhandle );

SVServer	sr_getevtlen, eventhandle

where	eventhandle is returned when sr_waitevtEx completes

sr_getevttype:		returns the event type for the current event

Dialogic		long sr_getevttype( unsigned long eventhandle );

SVServer	sr_getevttype, eventhandle

where	eventhandle is returned when sr_waitevtEx completes

sr_getparm:		returns the value of an SRL parameter

Dialogic		long sr_getparm( long ddd, long parmid, void* valuep );

SVServer	sr_getparm, ddd, parmid

sr_hold:		Not documented

Dialogic		long sr_hold();

SVServer	sr_hold

sr_NotifyEvent:	sends event notification to a window

Dialogic		void sr_NotifyEvent( HWND handle, unsigned int message, unsigned int flag );

SVServer	function not implemented for synchronous operations, use dx_getevt

sr_release:		Not documented

Dialogic		long sr_release();

SVServer	sr_release

sr_setlasterr:

Dialogic		void sr_setlasterr( long ddd, long error );

SVServer	function not implemented

sr_setparm:		allows the application to change the value

Dialogic		long sr_setparm( long ddd, long parmid, void* valuep );

SVServer	sr_setparm, ddd, parmid, value

sr_waitevt:		waits for events

Dialogic		long sr_waitevt( long tmout );

SVServer	function not implemented for synchronous operations, use dx_getevt

sr_waitevtEx:		wait for events on certain devices

Dialogic		long sr_waitevtEx( long *handlep, int count, long tmout, long *evthp );

SVServer	function not implemented for synchronous operations, use dx_getevt

Dialogic D42 API Function Definitions

d42_brdstatus:		Retrieves the current D/42-xx board status and places it in an application buffer.

Dialogic		int d42_brdstatus ( int iBoard, int *piStatus, char *pcBuffer )

SVServer	d42_brdstatus

Note: the status is placed in the (RSL=) field as RSL=status, BUF=future 48 bytes

d42_chnstatus:	Retrieves the current D/42-xx channel status and places it in an application buffer.

Dialogic		int d42_chnstatus ( int SrlDevice, char *bufp, char *buffer )

SVServer	d42_chnstatus

Note: the status is placed in the (RSL=) field as RSL=status, BUF=future 48 bytes

d42_closefeaturesession:	Closes an open feature session on the specified channel, terminating the association 			between the telephone extension and the channel number.

Dialogic		int d42_closefeaturesession ( int SrlDevice )

SVServer	d42_closefeaturesession

d42_display:		Retrieves the current LCD/LED display (alphanumeric) data and places it in an application buffer.

Dialogic		int d42_display ( int iChannel, char *pcBuffer )

SVServer	d42_display

Note: the display data is placed in the (RSL=) field

d42_flags:		Retrieves current D/42D-SX LCD Features Display data and places it in the application buffer.

Dialogic		int d42_flags ( int iChannel, char *pcBuffer )

SVServer	d42_flags

Note: the current D/42D-SX LCD Features Display data is placed in the (RSL=) field

d42_getparm:		Retrieves a D/42-xx channel or board parameter and places it in the application buffer.

Dialogic		int d42_getparm(devh, parmnum, parmvalp)

SVServer	d42_getparm, parmnum

where	parmnum = specifies the define for the parameter type whose value is to be returned

Note: the parameter value is placed in the (RSL=) field

d42_getver:		Retrieves the D/42-xx board firmware or library version and places it in an application buffer.

Dialogic		int d42_getver ( int iChannel, char *pcBuffer, int iFlag )

SVServer	d42_getver, flag

where	flag = Specifies what should be returned (firmware or library version number).

Note: the version is placed in the (RSL=) field

d42_gtcallid:		Retrieves the called/calling number ID of the incoming call and places it in an application buffer.

Dialogic		int d42_gtcallid ( int iChannel, char *pcBuffer )

SVServer	d42_gtcallid

Note: the caller-id number is placed in the (RSL=) field

d42_indicators:	Retrieves the status of LCD/LED indicators and places it in an application buffer.

Dialogic		int d42_indicators ( int iChannel, char *pcBuffer )

SVServer	d42_indicators

Note: the status of LCD/LED indicators is placed in the (RSL=) field

d42_lcdprompt:	Retrieves the current LCD prompt data of the D/42D-SX and places it in an application buffer.

Dialogic		int d42_lcdprompt ( int iChannel, char *pcBuffer )

SVServer	d42_lcdprompt

Note: the current LCD prompt data is placed in the (RSL=) field

d42_longbeep:

Dialogic		int d42_longbeep ( int SrlDevice )

SVServer	d42_longbeep

d42_openfeaturesession:	Opens a phone extension feature session on a specified channel, associating the 			telephone extension with the channel number

Dialogic		int d42_openfeaturesession ( int SrlDevice, char *szDnNumber, int* piTerminalType, int iEvtMask )

SVServer	d42_openfeaturesession, extension, evtmask

where	extension = ASCII character string application buffer where the extension number is placed.

evtmask= specifies in a bit mask the events to enable for the feature session.

d42_putpbxmsg:

Dialogic		int d42_putpbxmsg ( int SrlDevice, char* bufp, char msglen )

SVServer	d42_putpbxmsg, message

d42_setparm:		Sets a D/42-xx board or channel parameter.

Dialogic		int d42_setparm ( int iChannel, int iParm, void *pValue )

SVServer	d42_setparm, parmnum, parmval

where	parmnum = Specifies the define for the parameter that is to be updated with the value in the 			variable parmval.

parmval = The parameter value

d42_writetodisplay:	Writes to the phone set display the ASCII string.

Dialogic		d42_writetodisplay ( int SrlDevice, char* bufp )

SVServer	d42_writetodisplay, string

where	string = ASCII data string that is to be displayed on the telephone set display.

Dialogic Audio Conferencing API Function Definitions

dcb_addtoconf:	adds one conferee to an existing conference.

Dialogic		int dcb_addtoconf(int devh,int confID, DCB_CT *cdtp)

SVServer	dcb_addtoconf, confID

where	confID = the conference id returned from dcb_estconf

dcb_close:		closes the dcb device

Dialogic		int dcb_close(int devh)

SVServer	dcb_close

dcb_close_board:	closes the dcb board

Dialogic		int dcb_close_board( int dev );

SVServer	dcb_close_board

dcb_delconf:		deletes a previously established conference

Dialogic		int dcb_delconf(int devh, int confID)

SVServer	dcb_delconf, confID

where	confID = the conference id returned from dcb_estconf

dcb_dsprescount:	returns the available conferencing resource count for a specified DSP

Dialogic		int dcb_dsprescount(int devh, int *cntp)

SVServer	dcb_dsprescount

dcb_estconf:		establishes a conference of up to four parties

Dialogic		int dcb_estconf(int devh, DCB_CT *cdtp,int numpty, int confattr, int *confID)

SVServer	dcb_estconf, numpty, confattr

where	numpty = the number of parties in the conference

confattr = the conference attributes

Note: the conference DCB_CT structure must be setup prior to issuing this function,

Use dcb_set_cdt or dcb_set_cd2 to setup the DCB_CT structure

returns  confId in (RSL=) field

dcb_evtstatus:		gets or sets the status of a process-wide event.

Dialogic		int dcb_evtstatus(int, int, int *);

SVServer	dcb_evtstatus, event, action, status

dcb_get_cdt:		returns chan_num, chan_sel, and chan_attr for the indexed dc_cdt

Dialogic	 None	utility function normally implemented in C code

SVServer	dcb_get_cdt, index

where	index = 0 or 1, indexes the cdt array ms_cdt[0] or ms_cdt[1]

returns 	chan_num, chan_sel, and chan_attr for the indexed dc_cdt in the (RSL=) field

dcb_get_cdt2:		returns both array elements of chan_num, chan_sel, and chan_attr for the ms_cdt array

Dialogic	 None	utility function normally implemented in C code

SVServer	dcb_get_cdt

returns 	chan_num, chan_sel, and chan_attr, chan_num, chan_sel, and chan_attr in the (RSL=) field

dcb_get_volume:	returns vol_control, vol_up, vol_reset, vol_down

Dialogic	 None	utility function normally implemented in C code

SVServer	dcb_get_volume

returns 	vol_control, vol_up, vol_reset, vol_down in the (RSL=) field

dcb_getatibits:		returns the active talker indicator bits set at the time this function is called

Dialogic		int dcb_getatibits(int, unsigned long *);

SVServer	dcb_getatibits

returns	active talker indicator bits in (RSL=) field

dcb_getbrdparm:	retrieves a DCB/SC board parameter value.

Dialogic		int dcb_getbrdparm(int devh, unsigned char param, void *valuep)

SVSerever	dcb_getbrdparm, param

if param = MSG_ACTID, value is returned in (RSL=) field,

else if param = MSG_VOLDIG

follow this function  with dcb_get_volume to retrieve the volume values

dcb_getcde:		retrieves the attributes of a conferee in an existing conference.

Dialogic		int dcb_getcde(int devh, int confID, DCB_CT *cdtp)

SVServer	dcb_getcde, confID

where	confID = the conference id returned from dcb_estconf

requires that the dc_cdt chan_num and chan_sel be setup first

returns the attribute in the (RSL=) field

dcb_getcnflist:		retrieves a conferee list and the total number of parties within a conference.

Dialogic		int dcb_getcnflist(int devh, int confID, int * numpty, DCB_CT * cdtp)

SVServer	dcb_getcnflist, confID

dcb_getdigitmsk:	returns the digit mask for a specified conference.

Dialogic		int dcb_getdigitmsk(int devh, int confId, unsigned short *bitmask );

SVServer	dcb_getdigitmsk, confId

where	confID = the conference id returned from dcb_estconf

returns the digit mask in the (RSL=) field

dcb_getevt:		waits for a dcb event, no documentation available

Dialogic		int dcb_getevt(int devh, EV_EBLK *eblkp, int tmo)

SVServer	dcb_getevt, tmo

where	tmo = Specifies the maximum amount of time in seconds to wait for an event to occur.

returns the event in the (RTN=) field

returns the event data in the (RSL=) field

dcb_gettalkers:	retrieves information about the conferees actively talking in the specified conference.

Dialogic		dcb_gettalkers(int, int, int *, MS_CDT *);

SVServer	dcb_gettalkers, confId		//	returns up to 32 * MS_CDT structure

where	confID = the conference id returned from dcb_estconf

the number of parties in the currently talking is returned in the (RTN=) field

each cdt entry is returned in the (RSL=)  field as: chan_num, chan_sel, and chan_attr

for each active talker in the conference up to 32 entries total

dcb_monconf:		function adds a monitor to a conference.  A monitor has no input in the conference.

Dialogic		int dcb_monconf(int devh,int confID, long *lts)

SVServer	dcb_monconf, confID

where	confID = the conference id returned from dcb_estconf

returns the monitored time slot in the (RSL=) field

dcb_open:		opens a dcb channel device and returns a unique handle to identify the device

Dialogic		int dcb_open( const char *namep, int oflags );

SVServer	dcb_open, station name

dcb_open_board:	opens a dcb board device and returns a unique handle to identify the device

Dialogic		int dcb_open( const char *namep, int oflags );

SVServer	dcb_open_board, board name

dcb_remfromconf:	removes a conferee from a conference.

Dialogic 	int dcb_remfromconf(int devh,int confID, DCB_CT *cdtp)

SVServer	dcb_remfromconf, confID

where	confID = the conference id returned from dcb_estconf

dcb_set_cdt:		sets the chan_num, chan_sel, chan_attr for the indexd cdt array element

Dialogic	 None	utility function normally implemented in C code

SVServer	dcb_set_cdt, index, chan_num, chan_sel, chan_attr

where	index may be 0 or 1

dcb_set_cdt2:		sets the chan_num, chan_sel, chan_attr for both cdt array elements

Dialogic	 None	utility function normally implemented in C code

SVServer	dcb_set_cdt2, chan_num1, chan_sel1, chan_attr1, chan_num2, chan_sel2, chan_attr2

dcb_set_volume:	sets the volume information in the volume structure

Dialogic	 None	utility function normally implemented in C code

SVServer	dcb_set_volume, vol_control, vol_up, vol_reset, vol_down

(-See the MS_VOL structure in the Dialogic Help files )

dcb_setbrdparm:	function sets DCB/SC board device parameters.

Dialogic		dcb_setbrdparm(int devh, unsigned char param, void *valuep)

SVServer	dcb_setbrdparm, param, value

(-See the Dialogic Help files for param and value fields )

dcb_setcde:		changes the attributes of a conferee in an existing conference.

Dialogic		int dcb_setcde(int devh,int confID,DCB_CT *cdtp )

SVServer	dcb_setcde, confID

where	confID = the conference id returned from dcb_estconf

you must setup the dc_cdt before running this function

dcb_setdigitmsk:	enables specific digit detection for a conference

Dialogic		int dcb_setdigitmsk(int devh, int confId, unsigned short bitmask, int action);

SVServer	dcb_getdigitmsk, confID, bitmask, action

where	confID = the conference id returned from dcb_estconf

bitmask = determines the digits to be detected

action = specifies how the digit mask is changed

(-See the Dialogic Help files for the specifics )

dcb_tstcom:		no documentation available

Dialogic		int dcb_tstcom(int devh, unsigned int tmo)

SVServer	dcb_tstcom, tmo

dcb_tstdat:		no documentation available

Dialogic		int dcb_tstdat(int devh, unsigned int tmo)

SVServer	dcb_tstdat, tmo

dcb_unmonconf:	removes a monitor from a conference.

Dialogic		dcb_unmonconf(int devh, int confID)

SVServer	dcb_unmonconf, confID

where	confID = the conference id returned from dcb_estconf

Modular Station Interface API Function Definitions

ms_addtoconf:	adds one conferee to an existing conference.

Dialogic		int ms_addtoconf(int devh,int confID, MS_CDT *cdtp)

SVServer	ms_addtoconf, confID

where	confID = the conference id returned from ms_estconf

ms_chgxtder:		changes the attribute of the connection extender.

Dialogic		int ms_chgxtder(int devh,int xid, MS_CDT * cdtp)

SVServer	ms_chgxtder, xid

where	xid = the  extended connection identifier returned from ms_estxtdcon

Note: You must setup the chan_num, and chan_sel in the MS_CDT structure

Use ms_set_cdt or ms_set_cdt2 to set values in the MS_CDT structure

ms_close:		closes the ms device

Dialogic		int ms_close(int devh)

SVServer	ms_close

ms_close_board:	closes the ms board

Dialogic		int ms_close_board( int dev );

SVServer	ms_close_board

ms_delconf:		deletes a previously established conference

Dialogic		int ms_delconf(int devh, int confID)

SVServer	ms_delconf, confID

where	confID = the conference id returned from ms_estconf

ms_delxtdcon:		deletes an extended connection.

Dialogic		int ms_delxtdcon(int devh,int xid)

SVServer	ms_delxtdcon, xid

where	xid = the  extended connection identifier returned from ms_estxtdcon

ms_dsprescount:	returns the available conferencing resource count for a specified DSP

Dialogic		int ms_dsprescount(int devh, int *cntp)

SVServer	ms_dsprescount

ms_estconf:		establishes a conference of up to four parties

Dialogic		int ms_estconf(int devh, MS_CDT *cdtp,int numpty, int confattr, int *confID)

SVServer	ms_estconf, numpty, confattr

where	numpty = the number of parties in the conference

confattr = the conference attributes

Note: the conference MS_CDT structure must be setup prior to issuing this function,

Use ms_set_cdt or ms_set_cd2 to setup the MS_CDT structure

returns  confId in (RSL=) field

ms_estxtdcon:		establishes an extended connection, a connection in which there is a third party

Dialogic		int ms_estxtdcon(int devh,MS_CDT *cdtp,int *xid)

SVServer	ms_estxtdcon

returns	the xid, extended connection identifier in the (RSL=) field

the user must setuo the MS_CDT structure prior to issuing this function

Use ms_set_cdt or ms_set_cd2 to setup the MS_CDT structure

ms_genring:		generates ringing to a station. The function will terminate when the phone goes off-hook or the 			specified number of rings has been generated

Dialogic		ms_genring(int devh,unsigned short len, unsigned short mode)

SVServer	ms_genring, len, mode

where	len = the number of cycles to ring a station. A maximum value of 255 is allowed.

mode = operation mode, must be synchronous

ms_genringex:		no documentation available

Dialogic		ms_genringex(int devh,unsigned short len, unsigned short mode, unsigned short cadid)

SVServer	ms_genringex, len, mode, cadid

where	len = the number of cycles to ring a station. A maximum value of 255 is allowed.

mode = operation mode, must be synchronous

cadid = no documentation available

ms_genziptone:	generates a zip tone to the station associated with the device handle

Dialogic		ms_genziptone(int devh)

SVServer	ms_genziptone

ms_get_cdt:		returns chan_num, chan_sel, and chan_attr for the indexed ms_cdt

Dialogic	 None	utility function normally implemented in C code

SVServer	ms_get_cdt, index

where	index = 0 or 1, indexes the cdt array ms_cdt[0] or ms_cdt[1]

returns 	chan_num, chan_sel, and chan_attr for the indexed ms_cdt in the (RSL=) field

ms_get_cdt2:		returns both array elements of chan_num, chan_sel, and chan_attr for the ms_cdt array

Dialogic	 None	utility function normally implemented in C code

SVServer	ms_get_cdt

returns 	chan_num, chan_sel, and chan_attr, chan_num, chan_sel, and chan_attr in the (RSL=) field

ms_get_ncb:		returns the ms_ncb structure elements

Dialogic	 None	utility function normally implemented in C code

SVServer	ms_get_ncb

returns	volume, tone, duration, and pulse in the (RSL=) field

ms_getbrdparm:	retrieves a MSI/SC board parameter value.

Dialogic		int ms_getbrdparm(int devh, unsigned char param, void *valuep)

SVSerever	ms_getbrdparm, param

if param = MSG_ACTID, value is returned in (RSL=) field,

else if param = MSG_VOLDIG

follow this function  with ms_get_volume to retrieve the volume values

ms_getcb:		get control block, not implemented in SCBus

Dialogic		int ms_getcb(int devh,int param, MS_CDT *cdtp, void * valuep)

SVServer	not implemented in SCBus

ms_getcde:		retrieves the attributes of a conferee in an existing conference.

Dialogic		int ms_getcde(int devh, int confID, MS_CDT *cdtp)

SVServer	ms_getcde, confID

where	confID = the conference id returned from ms_estconf

requires that the ms_cdt chan_num and chan_sel be setup first

returns the attribute in the (RSL=) field

ms_getcnflist:		retrieves a conferee list and the total number of parties within a conference.

Dialogic		int ms_getcnflist(int devh, int confID, int * numpty, MS_CDT * cdtp)

SVServer	ms_getcnflist, confID

ms_getctinfo:		returns ct_devinfo structure elements

Dialogic		ms_getctinfo( chtable[chindex].msdevice, &chtable[chindex].ct_devinfo );

SVServer	ms_getctinfo

returns device information structure elements in the (RSL=) field

ms_getevt:		waits for a ms event, no documentation available

Dialogic		int ms_getevt(int devh, EV_EBLK *eblkp, int tmo)

SVServer	ms_getevt, tmo

where	tmo = Specifies the maximum amount of time in seconds to wait for an event to occur.

returns the event in the (RTN=) field

returns the event data in the (RSL=) field

ms_getxmitslot:	returns the SCbus time slot information contained in a SC_TSINFO

Dialogic		int ms_getxmitslot(int devh, SC_TSINFO *tsinfop)

SVServer	ms_getxmitslot

returns	SCBus transmit time slot for the device in the (RSL=) field

ms_listen:		connects the receive of a station to an SCbus time slot.

Dialogic		int ms_listen(int devh, SC_TSINFO *tsinfop)

SVServer	ms_listen, xmitslot

where	xmitslot is the transmit timeslot

ms_monconf:		function adds a monitor to a conference.  A monitor has no input in the conference.

Dialogic		int ms_monconf(int devh,int confID, long *lts)

SVServer	ms_monconf, confID

where	confID = the conference id returned from ms_estconf

returns the monitored time slot in the (RSL=) field

ms_open:		opens a ms channel device and returns a unique handle to identify the device

Dialogic		int ms_open( const char *namep, int oflags );

SVServer	ms_open, station name

ms_open_board:	opens a ms board device and returns a unique handle to identify the device

Dialogic		int ms_open( const char *namep, int oflags );

SVServer	ms_open_board, board name

ms_remfromconf:	removes a conferee from a conference.

Dialogic 	int ms_remfromconf(int devh,int confID, MS_CDT *cdtp)

SVServer	ms_remfromconf, confID

where	confID = the conference id returned from ms_estconf

ms_set_cdt:		sets the chan_num, chan_sel, chan_attr for the indexd cdt array element

Dialogic	 None	utility function normally implemented in C code

SVServer	ms_set_cdt, index, chan_num, chan_sel, chan_attr

where	index may be 0 or 1

ms_set_cdt2:		sets the chan_num, chan_sel, chan_attr for both cdt array elements

Dialogic	 None	utility function normally implemented in C code

SVServer	ms_set_cdt2, chan_num1, chan_sel1, chan_attr1, chan_num2, chan_sel2, chan_attr2

ms_set_ncb:		sets the volume, tone, duration, and pulse in the ms_ncb structure

Dialogic	 None	utility function normally implemented in C code

SVServer	ms_set_ncb, volume, tone, duration, pulse

(-See the MS_NCB structure in the Dialogic Help files for field values)

ms_setbrdparm:	function sets MSI/SC board device parameters.

Dialogic		ms_setbrdparm(int devh, unsigned char param, void *valuep)

SVServer	ms_setbrdparm, param, value

(-See the Dialogic Help files for param and value fields )

ms_setcb:		not implemented in SCBus

Dialogic		int ms_setcb(int devh,int param, MS_CDT *cdtp, void * valuep)

SVServer	not implemented in SCBus

ms_setcde:		changes the attributes of a conferee in an existing conference.

Dialogic		int ms_setcde(int devh,int confID,MS_CDT *cdtp )

SVServer	ms_setcde, confID

where	confID = the conference id returned from ms_estconf

you must setup the MS_CDT before running this function

ms_setevtmsk:		changes transition event masks and enables and disables messages from a station

Dialogic		int ms_setevtmsk(int devh, int event, unsigned short bitmask, int action)

SVServer	ms_setevtmsk, event, bitmask, action

(-See the Dialogic Help files for the specifics )

ms_setstparm:		changes the MSI/SC station level parameters

Dialogic		int ms_setstparm(int devh, unsigned char param, void *valuep)

SVServer	ms_setstparm,  param, value

(-See the Dialogic Help files for the specifics )

ms_setvol:		changes or resets the station volume.

Dialogic		int ms_setvol(int devh,int type,int steps)

SVServer	ms_setvol, type, steps

where	type = specifies whether to adjust or to reset current mode.

steps = the number of steps to increase or decrease volume.

ms_stopfn:		stops a multitasking function in progress for a station.

Dialogic		int ms_stopfn(int devh, int fctid)

SVServer	ms_stopfn, fctid

where	fctid = MTF_RING: Stops ringing on a station, if in progress

ms_tstcom:		no documentation available

Dialogic		int ms_tstcom(int devh, unsigned int tmo)

SVServer	ms_tstcom, tmo

ms_tstdat:		no documentation available

Dialogic		int ms_tstdat(int devh, unsigned int tmo)

SVServer	ms_tstdat, tmo

ms_unlisten:		disconnects the receive of a station from an SCbus time slot.

Dialogic		int ms_unlisten(int devh)

SVServer	ms_unlisten

ms_unmonconf:	removes a monitor from a conference.

Dialogic		ms_unmonconf(int devh, int confID)

SVServer	ms_unmonconf, confID

where	confID = the conference id returned from ms_estconf

The Global Call API Function Definitions

gc_AcceptCall:		an optional response to an inbound call request

Dialogic		int gc_AcceptCall(CRN crn, int rings, unsigned long mode);

SVServer	gc_AcceptCall, crn, rings, mode

where	crn = the call reference number returned by gc_WaitCall

rings = specifies how long (the number of rings) the protocol handler will wait before notifying the 			calling entity

mode = set to EV_SYNC for synchronous execution

gc_AnswerCall:	function is equivalent to conventional "set hook off" function in answering an inbound call and must 			be used to complete the call establishment process.

Dialogic		int gc_AnswerCall(CRN crn, int rings, unsigned long mode);

SVServer	gc_AnswerCall, crn, rings, mode

where	crn = the call reference number returned by gc_WaitCall

rings = specifies how long (the number of rings) the protocol handler will wait before notifying the 			calling entity

mode = set to EV_SYNC for synchronous execution

gc_Attach:		attaches a voice resource to the specified line device

Dialogic		int gc_Attach(LINEDEV linedev, int voiceh, unsigned long mode);

SCServer	gc_Attach, linedev, mode

where	linedev = the voice resource handle

mode = set to EV_SYNC for synchronous execution

gc_CallAck:		provides information about the incoming call to the network or retrieves information from the 			network about the incoming call.

Dialogic		int gc_CallAck(CRN crn, GC_CALLACK_BLK *callack_blkp, unsigned long mode);

SVServer	gc_CallAck, crn, mode

where	crn = the call reference number returned by gc_WaitCall

mode = set to EV_SYNC for synchronous execution

use gc_Get_CallAck_Blk to retrieve the GC_CALLACK_BLK structure

use gc_Set_CallAck_Blk to setup the GC_CALLACK_BLK structure

gc_CallProgress:	notifies the network that the connection request is in progress.

Dialogic		int gc_CallProgress(CRN crn, int indicator);

SVServer	gc_CallProgress, crn, indicator

where	crn = the call reference number returned by gc_WaitCall

indicator = call progress indicator

gc_CCLibIDToName:	converts call control library ID to name of call control library.

Dialogic		int gc_CCLibIDToName(int cclibid, char **lib_name);

SVServer	gc_CCLibIDToName, lib id

where	lib id = identification number of call control library

returns	the name of the call control library associated with the lib id in the (RSL=) field

gc_CCLibNameToID:	converts call control library name to ID code

Dialogic		int gc_CCLibNameToID(char *lib_name, int *cclibidp);

SVServer	gc_CCLibNameToID, lib name

where	lib name = the name of the call control library

returns	the identification number of call control library in the (RSL=) field

gc_CCLibStatus:	retrieves status of call control library specified by the lib name parameter

Dialogic		int gc_CCLibStatus(char *cclib_name, int *cclib_infop);

SVServer	gc_CCLibStatus, lib name

where	lib name = the name of the call control library; valid names include ICAPI and ISDN.

returns	the status of the call control library in the (RSL=) field

gc_CCLibStatusAll:

Dialogic		int gc_CCLibStatusAll(GC_CCLIB_STATUS *cclib_status);

SVServer	gc_CCLibStatusAll

returns	n1, n2, n3, n4, names1, names2, names3, names4 in the (RSL=) field

the number of available libraries as n1

the number of configured libraries as n2

the number of failed libraries as n3

the number of stub libraries as n4

the names of available libraries as names1 = name, name ... name,

the names of configured libraries as names2 = name, name ... name,

the names of failed libraries as names3 = name, name ... name,

the names of stub libraries as names4 = name, name ... name,

gc_Close:		closes a previously opened device

Dialogic		int gc_Close(LINEDEV linedev);

SVServer	gc_Close

gc_CRN2LineDev:	a utility function that matches a CRN to its line device ID

Dialogic		int gc_CRN2LineDev(CRN crn, LINEDEV *linedevp);

SVServer	gc_CRN2LineDev, crn

where	crn = the call reference number returned by gc_WaitCall

returns	the line device handle for the channel

gc_Detach:		logically detaches a voice resource from the line device

Dialogic		int gc_Detach(LINEDEV linedev, int voiceh, unsigned long mode);

SVServer	gc_Detach, linedev, mode

where	linedev = the voice resource handle

mode = set to EV_SYNC for synchronous execution

gc_DropCall:		disconnects a call specified by the CRN and enables inbound calls to be detected internally to 			GlobalCall on the line device

Dialogic		gc_DropCall(CRN crn, int cause, unsigned long mode);

SVServer	gc_DropCall, crn, cause, mode

where	crn = the call reference number returned by gc_WaitCall

cause = indicates reason for disconnecting or rejecting a call

mode = set to EV_SYNC for synchronous execution

gc_ErrorValue:

Dialogic		int gc_ErrorValue(int *gc_errorp, int *cclibidp, long *cclib_errorp);

SVServer	gc_ErrorValue

returns	error code, lib id, and error in the (RSL=) field

gc_ExtensionFunction:

Dialogic		int gc_ExtensionFunction(int cclibid, LINEDEV linedev, CRN crn, void *datap);

SVServer	function not implemented, documentation not available

gc_Get_CallAck_Blk:	retrieve the GC_CALLACK_BLK structure

Dialogic	 None	utility function normally implemented in C code

SVServer	gc_Get_CallAck_Blk, union

where	union = 0 to retieve service.dnis.accept union member

union = 1 to retrieve service.isdn.acceptance and service.isdn.linedev

See	Dialogic doumentation for gc_CallAck

gc_GetANI:		returns ANI information received during call establishment/setup

Dialogic		int gc_GetANI(CRN crn, char * anibuf);

SVSever	gc_GetANI, crn

where	crn = the call reference number returned by gc_WaitCall

returns	the ANI information in the (RSL=) field

gc_GetBilling:		gets the charge information associated with the specified call

Dialogic		int gc_GetBilling(CRN crn, char *billing_buf);

SVServer	gc_GetBilling, crn

where	crn = the call reference number returned by gc_WaitCall

returns	the billing information in the (RSL=) field

gc_GetCallInfo:	gets information for the call

Dialogic		int gc_GetCallInfo(CRN crn,int info_id, char *valuep);

SVServer	gc_GetCallInfo, crn, info id

where	crn = the call reference number returned by gc_WaitCall

info id = parameter identifier

returns	the information in the (RSL=) field

gc_GetCallState:	acquires the state of the call associated with the CRN

Dialogic		int gc_GetCallState(CRN crn, int *state_ptr);

SVServer	gc_GetCallState, crn

where	crn = the call reference number returned by gc_WaitCall

returns	the call state information in the (RSL=) field

gc_GetCRN:		gets the CRN for the event to which the pointer metaeventp is pointing.

Dialogic		int gc_GetCRN(CRN *crn_ptr, METAEVENT *metaeventp);

SVServer	not implemented, asynchronous mode function

gc_GetDeviceNameInfo:

Dialogic		int gc_GetDeviceNameInfo(char *DeviceName, GC_DEVICENAME_INFOP devicename_infop);

SVServer	function not implemented, documentation not available

gc_GetDlgerrValue:	returns the error value for the line device

Dialogic		int gc_GetDlgerrValue( LINEDEV linedev, int *dlgerrp);

SVServer	gc_GetDlgerrValue

returns 	the error value in the (RSL=) field

gc_GetDNIS:		gets the DNIS information (DDI digits) associated with a specific CRN.

Dialogic		int gc_GetDNIS(CRN crn, char *dnis);

SVServer	gc_GetDNIS, crn

where	crn = the call reference number returned by gc_WaitCall

returns	the DNIS information in the (RSL=) field

gc_GetFrame:		retrieves the frame received by the application.

Dialogic		int gc_GetFrame(LINEDEV linedev, GC_L2_BLK *l2_blkp);

SVServer	function not implemented, documentation not available

gc_GetLineDev:		gets a line device associated with the event received from the event queue.

Dialogic		int gc_GetLineDev(LINEDEV *linedevp, METAEVENT *metaeventp);

SVServer	not implemented, asynchronous mode function

gc_GetLinedevState:	retrieves status of the line device

Dialogic		int gc_GetLinedevState(LINEDEV linedev, int type, int *state_buf);

SVServer	gc_GetLinedevState, type

where	type = specifies Bchannel or Dchannel device type

returns	the state information in the (RSL=) field

gc_GetMetaEvent:	maps the current SRL event into a metaevent

Dialogic		int gc_GetMetaEvent(METAEVENT *metaeventp);

SVServer	not implemented, asynchronous mode function

gc_GetMetaEventEx:	maps the current SRL event into a metaevent that passes the SRL event handle to the application

Dialogic		int gc_GetMetaEventEx(METAEVENT *metaeventp, unsigned long evt_handle );

SVServer	not implemented, asynchronous mode function

gc_GetNetworkH:	returns the network device handle associated with the specified line device

Dialogic		int gc_GetNetworkH(LINEDEV linedev, int *networkhp);

SVServer	gc_GetNetworkH

returns	the network device handle

gc_GetParm:		retrieves the parameter value specified by the parm_id parameter for a line device.

Dialogic		int gc_GetParm(LINEDEV linedev, int parm_id, GC_PARM *valuep);

SVServer	gc_GetParm, param type, param id

where	param type = "short", or "long", or "int", or "char", or "string" depending on the parameter

param id = identifies the parameter

returns  the parameter value in the (RSL=) field

gc_GetUsrAttr:

Dialogic		int gc_GetUsrAttr(LINEDEV linedev, void  **usr_attrp);

SVServer	not implemented, asynchronous mode function

gc_GetVer:		gets version number of specified software component

Dialogic		int gc_GetVer(LINEDEV linedev, unsigned int *releasenump, unsigned int *intnump, long 			component);

SVServer	gc_GetVer, component

where	component = specifies the software component to which the version number applies.

returns	the major and minor version components in the (RSL=) field

gc_GetVoiceH:		returns the voice device handle

Dialogic		int gc_GetVoiceH(LINEDEV linedev, int * voicehp);

SVServer	gc_GetVoiceH

gc_HoldCall:		function allows the application to place an active call on hold.

Dialogic		int gc_HoldCall(CRN crn, unsigned long mode);

SVServer	gc_HoldCall, crn, mode

where	crn = the call reference number returned by gc_WaitCall

mode = set to EV_SYNC for synchronous execution

gc_HoldCallACK:	allows the application to accept a hold request from remote equipment.

Dialogic		int gc_HoldCallACK(CRN crn);

SVServer	gc_HoldCallACK, crn

where	crn = the call reference number returned by gc_WaitCall

gc_HoldCallRej:	function allows the application to reject a hold request from remote equipment.

Dialogic		int gc_HoldCallRej(CRN crn, int cause);

SVServer	gc_HoldCallRej, crn, cause

where	crn = the call reference number returned by gc_WaitCall

cause = standard Cause Value

gc_LibInit:		no documentation

Dialogic		int gc_LibInit(void);

SVServer	gc_LibInit

gc_LoadDxParm:	sets voice parameters associated with a line device from a dxchan.vcp file

Dialogic		int gc_LoadDxParm(LINEDEV linedev, char *pathp, char *errmsgp, int err_length);

SVServer	gc_LoadDxParm, path

where	path = the path of the dxchan.vcp file

gc_MakeCall:		enables the application to make an outgoing call on the specified line device

Dialogic		int gc_MakeCall(LINEDEV linedev, CRN *crnp, char *numberstr, GC_MAKECALL_BLK 			*makecallp, int timeout, unsigned long mode);

SVServer	gc_MakeCall, numberstr, default, timeout, mode

where	numberstr:	called party's telephone number

default = NULL or 0, NULL makes the gc_makecall_blk_ptr = NULL

timeout = time interval (in seconds) during which the call must be established

mode = set to EV_SYNC for synchronous execution

gc_Open:		opens a GlobalCall device and returns a unique line device handle

Dialogic		int gc_Open(LINEDEV *linedevp, char *devicename, int rfu);

SVServer	gc_Open, devicename

where	devicename = an ASCII string that defines the associated device(s)

returns 	the device handle in the (RSL=) field

gc_OpenEx:

Dialogic		int gc_OpenEx(LINEDEV *linedevp, char *devicename, int rfu, void *usrattr);

SVServer	not implemented

gc_ReleaseCall:	clear a specified CRN and wait for next call

Dialogic		int gc_ReleaseCall(CRN crn);

SVServer	gc_ReleaseCall, crn

where	crn = the call reference number returned by gc_WaitCall

gc_ReqANI:		retrieve caller ID and ANI-on-demand requests

Dialogic		int gc_ReqANI(CRN crn, char *anibuf, int req_type, unsigned long mode);

SVServer	gc_ReqANI, crn, req type, mode

where	crn = the call reference number returned by gc_WaitCall

req type =  ANI request types

mode = set to EV_SYNC for synchronous execution

gc_ResetLineDev:	reset a line device to the null state and set the line to blocking

Dialogic		int gc_ResetLineDev(LINEDEV linedev, unsigned long mode);

SVServer	gc_ResetLineDev, mode

where	mode = set to EV_SYNC for synchronous execution

gc_ResultMsg:		return the text message associated with an event

Dialogic		int gc_ResultMsg(int cclibid, long result_code, char **msg);

SVServer	gc_ResultMsg, cclibid, result code

where	cclibid = call control library identification from which the result_code was generated

result code = result value of the event or error code from the library

returns	the text message

gc_ResultValue:	return the cause of the specified event

Dialogic		int gc_ResultValue(METAEVENT *metaeventp, int *gc_result, int *cclibidp, long *cclib_resultp);

SVServer	not implemented, asynchronous mode function

gc_RetrieveCall:	function allows the application to retrieve a call from HOLD state.

Dialogic		int gc_RetrieveCall(CRN crn, unsigned long mode);

SVServer	gc_RetrieveCall, crn, mode

where	crn = the call reference number returned by gc_WaitCall

mode = set to EV_SYNC for synchronous execution

gc_RetrieveCallAck:	allows the application to accept a retrieve from HOLD request from remote equipment.

Dialogic		int gc_RetrieveCallAck(CRN crn);

SVServer	gc_RetrieveCallAck, crn

where	crn = the call reference number returned by gc_WaitCall

gc_RetrieveCallRej:	allows the application to reject a retrieve from HOLD request from remote equipment.

Dialogic		int gc_RetrieveCallRej(CRN crn, int cause);

SVServer	gc_RetrieveCallRej, crn, cause

where	crn = the call reference number returned by gc_WaitCall

cause = reason for the rejection

gc_Set_CallAck_Blk:	setup the GC_CALLACK_BLK structure

Dialogic	 None	utility function normally implemented in C code

SVServer	gc_Set_CallAck_Blk, type, param1

where	type = type of a structure inside following union

and	param1 = service.dnis.accept union member

Or	gc_Set_CallAck_Blk, type, param1, param2

type = type of a structure inside following union

param1 = service.isdn.acceptance union member

param2 = service.isdn.linedev union member

See	Dialogic doumentation for gc_CallAck

gc_SetBilling:		sets billing information for the call associated with the specified CRN.

Dialogic		int gc_SetBilling(CRN crn,int rate_type, GC_RATE_U *ratep, unsigned long mode);

SVServer	gc_SetBilling, crn, rate type, cents, mode

where	crn = the call reference number returned by gc_WaitCall

rate type = type of billing data

cents = rate amount in cents

mode = set to EV_SYNC for synchronous execution

gc_SetCallingNum:	set the calling party number for outbound calls

Dialogic		int gc_SetCallingNum(LINEDEV linedev, char *calling_num);

SVServer	gc_SetCallingNum, calling num

where	calling num = phone number of the calling party (ASCII string format)

gc_SetChanState:	changes the maintenance state of the indicated channel

Dialogic		int gc_SetChanState(LINEDEV linedev, int chanstate, unsigned long mode);

SVServer	gc_SetChanState, chanstate, mode

where	chanstate = service state of line

mode = set to EV_SYNC for synchronous execution

gc_SetDlgerrValue:	not documented

Dialogic		int gc_SetDlgerrValue(LINEDEV linedev, int dlgerr);

SVServer	gc_SetDlgerrValue, dlgerr

where	dlgerr = not documented

gc_SetEvtMsk:		sets the event mask associated with the specified line device

Dialogic		int gc_SetEvtMsk(LINEDEV linedev, unsigned long mask, int action);

SVServer	gc_SetEvtMsk, bitmask, action

where	bitmask = specifies the events to be enabled or disabled

action =  either set or reset the mask bit(s) as specified in bitmask.

gc_SetInfoElem:	allows applications to set an additional information element in the next outbound ISDN message on 			a specific D channel.

Dialogic		int gc_SetInfoElem(LINEDEV linedev, GC_IE_BLK *iep);

SVServer	gc_SetInfoElem, message

where	message = -see GC_IE_BLK, IE_BLK and Dialogic documentation

gc_SetParm:		sets the default parameters and all channel information associated with the specific line device.

Dialogic		int gc_SetParm(LINEDEV linedev, int parm_id, GC_PARM value);

SVServer	gc_SetParm, param type, param id, value

where	param type = "short", or "long", or "int", or "char", or "string" depending on the parameter

param id = identifies the parameter

value = the value of the parameter

gc_SetUsrAttr:		set a user-defined attribute

Dialogic		int gc_SetUsrAttr(LINEDEV linedev, void *usr_attr);

SVServer	not implemented, asynchronous mode function

gc_SndFrame:		function sends a frame to the data link layer.

Dialogic		int gc_SndFrame(LINEDEV linedev, GC_L2_BLK* l2_blkp);

SVServer	function not implemented, documentation not available

gc_SndMsg:		sends non-call state related ISDN message to the network over the Dchannel while a call exists

Dialogic		int gc_SndMsg(LINEDEV linedev, CRN crn, int msg_type, GC_IE_BLK *sndmsgptr);

SVServer	gc_SndMsg, crn, msg type, message

where	crn = the call reference number returned by gc_WaitCall

msg type = specifies the type of message to be sent

message = -see GC_IE_BLK, IE_BLK and Dialogic documentation

gc_Start:		starts all configured call control libraries,

Dialogic		int gc_Start(GC_START_STRUCT *startp);

SVServer	implemented in SVServer, "Global Call" must be in the SVServer startup command string

this function must be started from the main thread before any other gc_ functions.  it opens the call 			control libraries that interface directly to the network interface so that these libraries can be used 			by the GlobalCall library

gc_StartTrace:		instructs the firmware to trace and place results in shared RAM.  This function opens a file under 			the filename parameter and saves the results to this file.

Dialogic		int gc_StartTrace(LINEDEV linedev, char *tracefilename);

SVServer	gc_StartTrace, dev handle, path

where	dev handle = GlobalCall line device handle of Dchannel board.

path = specifies file name for the trace.

NOTE:	The linedev parameter must use the line device number for the D channel board.

gc_Stop:		stops all configured call control libraries started and cleans-up the GlobalCall database

Dialogic		int gc_Stop(void);

SVServer	implemented in SVServer, "Global Call" must be in the SVServer startup command string

this function must be started from main thread after all other gc_ functions have completed

gc_StopTrace:		stops the trace that was started using the gc_StartTrace function.

Dialogic		int gc_StopTrace(LINEDEV linedev);

SVServer	gc_StopTrace, dev handle

where	dev handle = GlobalCall line device handle of Dchannel board.

NOTE:	The linedev parameter must use the line device number for the D channel board.

gc_WaitCall:		sets up conditions for processing inbound calls.  The function unblocks the time slot (if the 			technology and the line conditions permit unblocking the line) and enables notification of inbound 			calls:

Dialogic		int gc_WaitCall(LINEDEV linedev, CRN *crnp, GC_WAITCALL_BLK *waitcallp, int timeout, 			unsigned long mode);

SVServer	gc_WaitCall, timeout, mode

timeout = pecifies the interval (in seconds) to wait for the call.

mode = set to EV_SYNC for synchronous execution

returns 	crn in chresult

Dialogic Text-To-Speech API Function Definitions

tts_close:		Closes a tts device

Dialogic		int tts_close( int );

SVServer	tts_close

tts_geterror:		Returns the error string

Dialogic		char* tts_geterror( int );

SVServer	tts_geterror, error

where	error = Error number returned by a TextTalk call

tts_getspeechrate:	Retrieves the speech rate being used by the TTS engine.

Dialogic		int tts_getspeechrate( int, int* );

SVServer	tts_getspeechrate

tts_getpitch:		Retrieves the pitch being used by the TTS engine.

Dialogic		int tts_getpitch( int, int* );

SVServer	tts_getpitch

tts_getvoice:		Retrieves the voice being used by the TTS engine

Dialogic		int tts_getvoice( int, int* );

SVServer	tts_getvoice

tts_getvolume:		Retrieves the output volume of the TTS engine.

Dialogic		int tts_getvolume( int, int* );

SVServer	tts_getvolume

tts_init:		Initializes the TTS engine.

Dialogic		int tts_init ( void );

SVServer	tts_init

tts_jump:		Jumps forward or backward in a text file by a specified number of units

Dialogic		int tts_jump(int, int, int);

SVServer	tts_jump, jumpParm, jumpSize

where	jumpParam = Type of unit

jumpSize = the number of units to jump

tts_open:		Opens a TTS resource and returns a TTS resource handle for the specified process.

Dialogic		int tts_open( int );

SVServer	tts_open

tts_pause:		Pauses speech output until a resume event occurs.

Dialogic		int tts_pause(int);

SVServer	tts_pause

tts_phoneme:		Processes a text string and returns its phoneme string

Dialogic		int tts_phoneme(int , LPSTR , LPSTR , long , long *);

SVServer	tts_phoneme, inString, OutStringIndex, OutStringSize

where	InString = Input string

OutString = buffer index of the buffer to hold the returned phonemes.

OutStringSize = the size of OutString buffer.

returns: In (RSL=) the actual buffer size needed if the function fails

Note: a buffer must be allocated using ut_allocate which returns an index to

pass in OutStringIndex

tts_pitch:		Sets the pitch at which the text is spoken.

Dialogic		int tts_pitch(int, int);

SVServer	tts_pitch, pitch

where	pitch = The speech pitch, any number between 50 and 400 Hertz.

tts_resume:		Resumes speech output after a pause.

Dialogic		int tts_resume(int);

SVServer	tts_resume

tts_reset:		Resets all TTS parameters to their default values The default values are applied to the next speak 			operation.

Dialogic		int tts_reset( int );

SVServer	tts_reset

tts_setcontext:		Changes the TTS context setting

Dialogic		int tts_setcontext(int, int);

SVServer	tts_setcontext, context

tts_speakbuffer:

Dialogic		int tts_speakbuffer ( int, LPSTR, int, DV_TPT* tptPtr);

SVServer	-See ez_playtext

tts_speakemail:

Dialogic		int tts_speakemail( int, LPSTR, int, DV_TPT* tptPtr);

SVServer	-See ez_playtext

tts_speakfile:

Dialogic		int tts_speakfile( int, char*, int, DV_TPT* tptPtr);

SVServer	-See ez_playtext

tts_speechrate:	Sets the speed at which the text is spoken.

Dialogic		int tts_speechrate(int, int);

SVServer	tts_speechrate, rate

where	rate = The speaking rate, any number between 50 and 250 words per minute.

tts_stop:		Forces termination of a currently active TTS operation.

Dialogic		int tts_stop( int );

SVServer	tts_stop

tts_tracelevel:		Not documented

Dialogic		int tts_tracelevel(int, int );

SVServer	tts_tracelevel, level

tts_volume:		sets the output volume at which the text is spoken.

Dialogic		int tts_volume(int, int);

SVServer	tts_volume, volume

where	volume = The speech volume, any number between 0 and 16.

Dialogic		int tts_voice(int, int);

SpeechWorks 		API Function Definitions

This interface handles up to 64 Dialog Modules per channel.  Use of these basic Speechworks functions alone requires that the operation be setup within the Dialog Modules themselves without intervention by the application at runtime.  The extended Speechwork API functions provide a means of runtime modification to the services.  In order to utilize the SpeechWorks functions: the keyword "SpeechWorks" must be included in the SvServer startup command line.

ALTServiceCreate:	Creates a Dialog Module

SpeechWorks	ALTdmFuncResult ALTAPI ALTdmCreateModule(ALTdmDialogFactory *, const char *, const char 			*, ALTdmDialogModule **);

SVServer	ALTServiceCreate, service, module

where	service is the name of a service found in the service.cfg file

module is the name of a Dialog Module found in the service.cfg file

returns	dmNo an index handle to the Dialog Module just created, used to reference the Dialog Module in 			the ALTServiceExecute & ALTServiceDestroy

ALTServiceDestroy:	Destroy a Dialog Module and free the memory

SpeechWorks	ALTdmFuncResult ALTAPI ALTdmDestroyModule(ALTdmDialogModule **);

SVServer	ALTServiceDestroy, dmNo

where	dmNo refers to a specific Dialog Module index returned by ALTServiceCreate

ALTServiceExecute:	Executes a specific Dialog Module

SpeechWorks	ALTdmExecuteResult ALTAPI ALTdmExecute(ALTdmDialogModule *, ALTdmDialogSession *);

SVServer	ALTServiceExecute, dmNo

where	dmNo refers to a specific Dialog Module index returned by ALTServiceCreate

ALTServiceInitialize:	Initializes a service, creates the promt, record, call control factory and session

SpeechWorks	none

SVServer	ALTServiceInitialize, version, baseline path, system path

where	version is the SpeechWorks Version, ie. 4.0.1, 5.0.1, 6.0.0

baseline path is the path for the baseline.cfg file

system path is the path for the system.cfg file

Note: Send this command before all other ALT commands, after open

On some versions the paths for baseline.cfg and system.cfg may be obtained from the system 			environment variables Start>Settings>Control Panel>System>Environment

On version 6, ALTServiceInitialize, version, NULL, NULL may be used

ALTServiceTerminate:	Terminates a service, destroys all Dialog Modules, the promt, record, factory and session

SpeechWorks	none

SVServer	ALTServiceTerminate

Note: Send this command after all other ALT commands, just before close and terminate

ALTServiceWaitForPrompt:	Waits for the last prompt to finish then optionally issues dx_stopch

SpeechWorks	ALTdmExecuteResult ALTAPI ALTdmWaitForPrompt(ALTdmDialogSession *);

SVServer	ALTServiceWaitForPrompt, stop

where	stop if non-zero causes a dx_stopch on the channel.

ALTSessionEndCall:	Ends a call session

SpeechWorks	ALTdmFuncResult ALTAPI ALTdmSessionEndCall(ALTdmDialogSession *);

SVServer	ALTSessionEndCall

Note: Send this command right after you hang up the line

ALTSessionNewCall:	Begins a call session

SpeechWorks	ALTdmFuncResult ALTAPI ALTdmSessionNewCall(ALTdmDialogSession *);

SVServer	ALTSessionNewCall

Note: Send this command right after you pick up the line

SpeechWorks 	Extended API Function Definitions

The extended function API documentation for SpeechWorks is extensive and in process.