SK-NET FDDI Driver for Windows NT

Installation

October 1994

Sofware version 2.10

SKFINT.SYS is the driver for the SK-NET FDDI ISA adapter

under the first release of Windows NT on Intel processors.

The driver corresponds to the NDIS 3.0 Specification as delivered with

the first release of the Windows NT DDK.

Files needed

In the root of the installation diskette

DISK3		Indicator.

Preferrably in the root

!SKDF000.cfg	New EISA config file for SK-NET FDDI ISA adapter

In a single directory

OEMNADSK.HLP	Help file for SK-NET drivers.

OEMNADSK.DLL	Dialogs for SK-NET drivers.

OEMSETUP.INF	Setup script for the driver.

SKFINT.SYS	The driver.

How to install

On an EISA or PCI machine the SK-NET FDDI ISA adapter MUST be configured

using the appropriate EISA configuration file.

After having configured your adapter, please start Windows NT. There,

start NETWORK from the CONTROL PANEL, choose ADD ADAPTER and then

<OTHER> and type in the path to OEMSETUP.INF.  Choose the driver

presented and select the appropriate configuration.  Then choose OK

and restart your system.  (Protocol software should be installed.)

Troubleshooting

If you encounter problems using the SK-NET FDDI adapter under

Windows NT, please re-read carefully the instructions above and the

documentation supplied with the system. Additionally, you should look

into your system log file (using the event viewer) and check it for

entries from SKFINT. The last 5 DWORDs of the logging data are:

Slot number (on ISA computers: -1),

Interrupt number,

Dma channel,

Base I/O port and

Vendor code (3 bytes) / adapter id (1 byte).

of the adapter that caused the error.

Following is a list of some events, possible reasons (may be

incomplete) and suggestions to fix your problem.

Event ID:	5001 -	Could not allocate the resources necessary

for operation.

Reason(s):	Not enough RAM or pagefile too small.

Help:		...

---------------

Event ID:	5003 -	Could not find an adapter.

Reason(s):	Wrong slot or IO Port.

Adapter physically not installed.

System configuration information disappeared.

Help:		Check slot/port settings.

Look if adapter is physically present.

On an EISA machine: Look if adapter is EISA configured.

---------------

Event ID:	5004 -	Could not connect to the interrupt number

supplied.

Reason(s):	The selected interrupt is already in use.

Help:		Select free interrupt.

---------------

Event ID:	5007 -	Timed out during an operation.

Reason(s):	Wrong interrupt, dma channel or base address.

Cable not connected.

Help:		Run the diagnostic program supplied with the

adapter or start a driver under DOS (with the same

resource usage) and look if it works.

Check cabling.

---------------

Event ID:	5009 -	Does not support the configuration supplied.

Reason(s):	Invalid IRQ given.

Invalid DMA given.

Help:		Check configuration.

---------------

Event ID:	5012 -	The I/O base address supplied does not match

the jumpers on the adapter.

Reason(s):	Jumper settings and software configuration conflict.

Help:		Check jumper settings against software configuration.

If your special problem is not listed or the error persists, please

send us your system log file or at least the event id number and the

logging data (as dwords) of the entries written by SKFINT.