HUG MENU SYSTEM (MS-DOS 2)

P/N 885-3020-37

IMPORTANT  NOTICE:   THE  SALE OF THE PROGRAMS ON THIS DISK IS  A

SOURCE  OF  REVENUE FOR THE NATIONAL HEATH/ZENITH  USERS'  GROUP.

PLEASE DO NOT PLACE THEM ON COMPUTER BULLETIN BOARDS OR  DISTRIB-

UTE THEM AT LOCAL COMPUTER CLUB MEETINGS.

Introduction:   The  HUG  Menu System is a set of  programs  that

allow  you to perform all normal computer operations from menues.

The  menues themselves are generally operated by using the  arrow

keys  to  move an indicator to the  desired  function,  and  then

pressing  the  RETURN or ENTER key.   Using the HUG Menu  System,

an experienced user can set up disks for inexperienced users that

can be used without knowledge of the operating system,  etc.  The

HUG   Menu  System  consists  of  the  menu  system   itself,   a

sophisticated  menu  driven  file manager,  and a  console  setup

utility.

Requirements:  The HUG Menu System requires MS-DOS version 2.0 or

higher,  and will run on either Z-100 or Z-100-PC (Z-150)  series

computers.   The  programs are useable on systems having 128k  of

memory, but 192k or more is recommended.

** DEMONSTRATION **

If  you would like to see the HUG Menu System in operation before

you read the documentation, follow these instructions:

1.   Copy  this  disk (for protection).   Use the  copy  for  the

following instructions.

2.   Copy COMMAND.COM and CHKDSK.COM from your system disk to the

HUG Menu System disk.

3.   Place the HUG Menu System disk in drive A:  and log on to it

if you are not already logged onto drive A:

4.   Type MENU at the system prompt,  and press RETURN.   If your

computer is a Z-100-PC series (Z-150, Z-160, etc.), and your

moniter cannot distinguish between white and intense  white,

type MENU1 instead.

5.   When  the  menu appears on the screen,  use the up and  down

arrow  keys  to see what happens.   If you press  RETURN  or

ENTER, the menu option that is highlighted will be executed.

Press  HELP  (on  a  Z-100) or F1 (on  a  Z-100-PC)  to  see

information on the highlighted item.

6.   If  you  move the highlighted indicator to the last item  on

the menu and press return,  a second menu will be displayed.

From  this  menu  you can run the HUG File  Manager  or  the

Console Setup utility.

7.   When  you  are through experimenting with the  Menu  System,

press SHIFT-F10 to exit back to the operating system.

The following programs are included on the HUG Menu System disk.

MENU.COM  -- This is the menu program itself.   It processes sep-

arately prepared menu files that contain the screen displays  and

commands.

MENU1.COM  -- The  MENU program normally uses intense  white  for

highlighting  when it is run on a Z-100-PC series  computer.   If

you  ran MENU during the demonstration session described  earlier

in  this  document,  and  one of the menu items  did  not  appear

brighter  than  the others,  you should use MENU1.COM instead  of

MENU.COM.   MENU1 uses reverse video instead of intense white for

highlighting.   When  you  set up your own menu  disks,  you  can

rename MENU1.COM to MENU.COM.

Note:   Both  versions use reverse video to highlight on a  Z-100

computer.

MENU.DOC  -- This file contains instructions for setting up  your

own menues.

MENU.MNU -- This is a sample menu file.

MENU.RAW  -- This  is a menu definition file,  that was  used  to

create  MENU.MNU.   This file was made using WordStar  (TM),  and

then printed to disk using the WordStar PRINT command.

MENU.WS -- This is the above file in WordStar document file form.

You may want to use it as a starting point for creating your  own

menues, if you use WordStar.

UTILITY.MNU  -- This is a sample sub menu file.   A selection  on

the main menu (MENU.MNU) takes you into this menu.

UTILITY.RAW -- The definition file used to create UTILITY.MNU.

MAKMENU.COM -- This program turns menu definition files into menu

files  usable  by  MENU.COM.   Instructions for using it  are  in

MENU.DOC.

HFM.COM -- The HUG File Manager is a menu driven file maintenance

utility  that  allows you to copy  files,  delete  files,  rename

files,  create  and  remove directories,  create or  change  disk

labels,  type  (on  the screen) and print files,  list  files  in

hexadecimal, and sort or unsort the directory.

HFM1.COM -- This version is for Z-100-PC computers with  monitors

that do not show intense white.

HFM.DOC -- Instructions for the HUG File Manager

HEXLIST.COM  -- This  program  is  used by HFM  for  one  of  its

commands.   It  displays  a file in hexadecimal and ASCII  (text)

similarly to the D command in DEBUG.  You can use it by itself by

entering

HEXLIST d:FILENAME.EXT

where d: is the drive containing the file to view, represented by

FILENAME.EXT.  The display will pause every 16 lines, and you can

press  any key to continue,  or type Control-C to exit.   If  the

file you are viewing is a .COM file, HEXLIST will use 100H as its

starting address for the listing.   Otherwise,  0 is used as  the

address.

SETCON.COM  -- This utility allows you to set up certain  console

parameters on your computer.   On a Z-100, you can set the cursor

type  (line or block,  blink or steady),  screen wrap or no wrap,

key click (on/off), and auto repeat (on/off).  On a Z-100-PC, you

can  set the video mode,  the scroll mode,  and the  cursor  type

(normal, single line, block).

When  you run SETCON,  a menu of options will be displayed on the

screen  with one of them highlighted.   Use the up or down  arrow

keys  to move the highlighted indicator to the item you  wish  to

select,  and then press RETURN.  The selected option may not take

effect  until  you  exit the SETCON program.   If  you  run  this

program from the Menu System on a Z-100-PC,  the scroll mode will

be reset to sotware scrolling regardless of where you set it when

the  menu returns.   However,  the other options you set will  be

retained  in  memory by the MENU program,  and restored when  you

leave the menu.

SETCON1.COM  -- This  version  is  for  Z-100-PC  computers  with

monitors that do not show intense white.

MDISK.DVD -- The MENU and HFM programs require the MS-DOS command

processor, COMMAND.COM, for some functions.  These functions will

be  performed  faster if COMMAND.COM is copied to a memory  disk.

This memory disk is a small one (20k),  with just enough room  to

hold COMMAND.COM, without using up too much program memory space.

Instructions  for  using it and patching it to  other  sizes  are

included in HFM.DOC.

CONFIG.SYS  -- This  file can be copied to your system  disks  to

cause  MDISK to be installed,  or use it as a guide for modifying

your own CONFIG.SYS.

AUTOEXEC.100, AUTOEXEC.150 -- One of these files can be copied to

your system disks to cause COMMAND.COM to be copied to the memory

disk  and  the  COMSPEC paremeter set up to  indicate  that  COM-

MAND.COM  is on the memory disk.   They also set the path so that

the  system will always search the root directory on  the  system

drive.   AUTOEXEC.100  is for use on Z-100 series computers  that

boot from a floppy on drive A:, and AUTOEXEC.150 is for use on Z-

100-PC  series  computers that boot from a floppy on  drive   A:.

Copy  the  one  you need to your system disk,  and rename  it  to

AUTOEXEC.BAT.   Use these files as a guide to modifying your  own

AUTOEXEC file if you use one, or to create one if you do not boot

from drive A:.

MENU.ASM, MAKMENU.ASM, HFM.ASM, HEXLIST.ASM, SETCON.ASM -- Source

code for the HUG Menu System programs.

MACROS.ASM -- Macros required to assemble the above source codes.

Program Authors:

MENU -- Robert A.  Metz,  Whirlpool Corp.,  2000 U.S.  33  North,

Benton Harbor, MI 49022.  Modified by P. Swayne, HUG.

MAKMENU -- Robert A.  Metz,  modified by P.  Swayne.   Originally

titled MNUCREAT.

HFM  -- Robert  A.  Metz  and  Patrick  Swayne.   Developed  from

FILEMGMT by Robert A. Metz.

SETCON -- Robert A. Metz and Patrick Swayne.  Z-100-PC section by

Patrick Swayne.

*****************************************************************

* The  Heath  Users'  Group is provided by Heath Company  as  a *

* service  to  its  members for the purpose  of  fostering  the *

* exchange of ideas to enhance their usage of Heath  Equipment. *

* As  such,  little or no evaluation of the program(s) is  per- *

* formed.   The  prospective user is hereby put on notice  that *

* the  program  may  contain faults the consequences  of  which *

* Heath Company in general and the Heath Users' Group (HUG)  in *

* particular cannot be held responsible.   The prospective user *

* is, by virtue of obtaining and using these programs, assuming *

* full risk for all consequences.                               *

*****************************************************************

* These  (this) program(s) were developed and tested on  MS-DOS *

* version 2.13 and are believed to be free of  program  errors. *

* The  Heath  Users' Group cannot predict that  the  program(s) *

* will  perform  on future versions of MS-DOS.   However,  if a *

* version dependent program surfaces,  REMark will publish  all *

* known fixes for all products.   Individual attention to prob- *

* lems  of this nature is not possible with a group as large as *

* ours.                                                         *

*****************************************************************

MS-DOS is a trademark of Microsoft.

####