I. INTRODUCTION :

Terminology

Conventional memory -

It refers to the memory DOS recognizes. This memory is contained in

the computer system board and the 384K RAM card or Multifunction card

(if any). The maximum size of the conventional memory is 640 Kbytes

Expanded memory -

It is a special kind of memory that locates beyond DOS 640K-byte limit.

Application programs can make use of the expanded memory through the

Expanded Memory Manager (EMM).

RAM disk -

A RAM disk is equivalent to an additional disk drive but the storage

media is some of your computer random access memory (RAM). Unlike

an floppy disk drive, a RAM disk is all made of electronic parts, so

it works faster.

How to Get Started?

Before you can use the expanded memory in the Expanded Memory

Specification (EMS) board, you must load the Expaned Memory Manager

(EMM) driver program. MSDOS supports loading device drivers at boot

time by creating a CONFIG.SYS file. A CONFIG.SYS file is an ASCII

code (or text) file which has the following format.

DEVICE=dddddd.SYS  para

dddddd.SYS is a device driver program name and para is a parameter

for the device driver. A CONFIG.SYS file may declare several device

drivers in the above format each separated by a carriage return.

When the DOS is booted up, it searches for a CONFIG.SYS file. If a

CONFIG.SYS file is found, the device drivers declared by the CONFIG.SYS

are loaded one by one.

Supplied in this diskette are three device drivers, namely EMM.SYS,

ERAMDISK.SYS and CRAMDISK.SYS. The function of each device driver

is explained in Section II.

What is the use of a RAM disk?

A RAM disk can speed up a program which has frequent access to a

disk drive. A RAM disk can be implemented by declaring a RAM disk

device driver in CONFIG.SYS. Details of implementing a RAM disk are

described in Section III and IV. After a RAM disk is installed, it

has no file. You have to copy application programs or data files

to the RAM disk. This can be done by the COPY command (refer to DOS

manual for details). After you have finished working with your

program, any useful data file which has been modified in the RAM disk

should be copied back to your floppy diskette or hard disk drive,

since the data in RAM will be destroyed if you turn off the power or

reboot the system.

II. DEVICE DRIVERS INSIDE THE DISKETTE :

EMM.SYS	- The expanded memory manager driver program

This is the driver program for the Expanded Memory Specification

(EMS) board. It must be installed before the expanded memory  in the

EMS board can be used.

ERAMDISK.SYS - The RAM disk driver program for the EMS board

This is the driver program for implementing a RAM disk in the EMS

board. The RAM disk has a drive ID which is the letter next to

the drive ID letters used by the floppy disk drives and hard disk

drives in your system. For example, if your system has two floppy

drives, the drive ID of the RAM disk will be C:. If your system has

two floppy drives and one hard disk drive, the drive ID of the RAM

disk will be D: ( A: and B: are drive IDs for the floppy drives,

and C: is the drive ID for the hard disk drive).

CRAMDISK.SYS - The RAM disk driver program for the conventional

memory

This RAM disk driver program is similar to ERAMDISK.SYS but the

RAM disk occupies conventional memory. The naming of the RAM disk

drive ID is the same as that in ERAMDISK.SYS. If both ERAMDISK.SYS

and CRAMDISK.SYS are installed, they will have separate drive IDs

If this RAM disk is installed, some application programs (e.g.

SYMPHONY) which requires large conventional memory size cannot be

run.

III. INSTALLING THE DEVICE DRIVER :

To prepare a system diskette with the above device drivers, copy

the three device drivers program to your system diskette. Then

create a CONFIG.SYS file in your system diskette to invoke the

device drivers. The function of the CONFIG.SYS is to load the

device drivers at boot time. To create a CONFIG.SYS file, you can

put your system diskette in drive A: and enter the following

command.

COPY CON: A:CONFIG.SYS <CR>

DEVICE=EMM.SYS M3 I0 <CR>

DEVICE=ERAMDISK.SYS 512 <CR>

DEVICE=CRAMDISK.SYS 128 <CR>

<F6><CR>

Remark:	<CR> is the ENTER key and <F6> is the F6 function key.

The above CONFIG.SYS file is only an example. The entries

M3, I0, 512, 128 are parameters for the device drivers.

They may be varied for different system configurations or

applications. If a RAM disk for conventional memory is not

required, the command line DEVICE=CRAMDISK.SYS can be

ommitted. The command line DEVICE=EMM.SYS must be entered

before DEVICE=ERAMDISK.SYS.

IV. PARAMETERS IN THE DEVICE DRIVERS :

EMM.SYS

Format : DEVICE=EMM.SYS  Ma Ib Ib

M is the parameter heading defining the starting frame address of

the memory in the EMS board. the 'a' after M represents a number

which can be 0 to 7.

M parameter	Starting frame address (in HEX)

-----------------------------------------------

M0		C4000

M1		C8000

M2		CC000

M3		D0000

M4		D4000

M5		D8000

M6		DC000

M7		E0000

The M parameter can be defined in any of the above values but you

must make sure that the address space of the EMS memory does not

conflict with the interface card with ROM memory. The EMS board

occupies 64K address space starting from the frame address (i.e.

if M0 is defined, EMS memory occupies address C4000-D3FFF). If

your system contains a hard disk controller card which has a

interface ROM with address C8000-CFFFF, parameter M0 or M1 should

not be used. It is recommended to use parameter M3, since the

address space does not have conflict with most common interface

cards.

I is the parameter heading defining the I/O port address of the

EMS board installed. the 'b' after I represents a number which can

be 0 to 6.

I parameter	I/O port address of 	DIP switch setting in EMS

EMS board (in HEX)	1	2	3	4

------------------------------------------------------------------

I0		208			OFF	ON	ON	X

I1		218			ON	OFF	ON	X

I2		258			OFF	OFF	ON	X

I3		268			ON	ON	OFF	X

I4		2A8			OFF	ON	OFF	X

I5		2B8			ON	OFF	OFF	X

I6		2E8			OFF	OFF	OFF	X

Remark : Pole 4 of the DIP switch may be ON or OFF

If your system is a COMPACT XT with built-in EMS memory,

the factory set I/O port address is 208 and parameter I0

should be used.

A computer system can install up to four EMS board. Each EMS

board must has its own I/O port address. There is a four pole DIP

(dual in line) switch which can be set to configure different I/O

port address. The DIP switch setting on each EMS board must be

different in order to have different I/O port address. You should

define one I parameter for one EMS board, two I parameters for

two EMS boards installed in the computer system and so forth.

For example, if you have four EMS boards in the system, you can

define the parameters as follows.

DEVICE=EMM.SYS M3 I0 I1 I2 I3

ERAMDISK.SYS

Format : DEVICE=ERAMDISK.SYS nnnn

nnnn represents a number which defines the RAM disk size in Kbyte

of memory. The minimum number is 16 and the maximum number depends

on the EMS memory size in your system. Each EMS board contains

2048 Kbytes (2 Mbytes) of memory, if the EMS board is fully filled

with RAM chips. If four EMS boards are installed in your system,

the EMS memory size is 8192 Kbytes (8 Mbytes).

(Note: If your system diskette is MSDOS version 2.0 or 2.1 the

maximum number for RAM disk is 2048. If your system diskette is

MSDOS version 3.0, 3.1, 3.2 or above the maximum number is 8192.)

CRAMDISK.SYS

Format : DEVICE=CRAMDISK.SYS nnn

nnn represents a number which defines the RAM disk size in Kbyte

of memory. The minium number is 16 and the maximum number depends

on the available conventional memory size. If you application

program requires large memory size, it is not recommended to

install this device driver.

#####################################################################################################