%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%OP%DEThe Serial Interface

%CO:A,12,80%%C%%H1%%H2%T h e   S e r i a l   I n t e r f a c e%H2%%H1%

The Z88 has a single serial port which has to be shared by all applications, so

it is important that applications use the system to access the port to avoid

interfering with the correct operation of other applications running in the

machine. The system runs the serial port using interrupts and buffering, so

most of its operation is transparent, but there are certain operations, such as

directly controlling the RTS line (required for auto-dialing by some modems),

which the system calls do not support and in these cases the hardware is

accessed more directly - see the "Hardware" section of these notes. The serial

port specification is reproduced here for convenience:

Baud rates

Parity

Flow

Timeout

Buffer

%H2%Using the Serial Port%H2%

The serial port is accessed by opening the device :COM.0, using GN_OPF, and

then using the standard file I/O routines: OS_GB, OS_GBT, OS_PB, OS_PBT and

OS_MV. The device should be closed after the operation is complete. For most

application this level of access should be sufficient, however, it may be

necessary to change the serial port parameters from within an application.

This is done by writing new values to the Panel, using the OS_SP call, and then

issuing a soft reset command to the serial driver. Changing the Panel setting

may affect other software in the machine, so should not be undertaken lightly.

It would be appropriate to reset the Panel to its original setting after serial

port operations are complete. This can be achieved by first reading the Panel

settings with OS_NQ. The "EasyLink" (part of the Z88 Assembler Workbench) and

"PCLINK II" popdowns use this technique. OS_SP and OS_NQ are covered in detail

in the "Miscellaneous useful routines" section.

The serial soft reset operation is carried out by opening the OS_SI call. The

OS_SI call provides a low level access to the serial port. The call can perform

nine different operations depending on the reason code passed to it:

%P0%

%H4%Reason code%H4%

SI_HRD ($00)

SI_SFT ($03)

SI_INT ($06)

SI_GBT ($09)

SI_PBT ($0C)

SI_ENQ ($0F)

SI_FTX ($12)

SI_FRX ($15)

SI_TMO ($18)

This operating system call used for low level serial port communication uses

the following interface:

OS_SI, low level serial interface

RST 20H, DEFB $8D

IN:

OUT:

The serial various reason code parameters are descibed in detail in the "System

calls Reference" section.

%H2%Flow Control%H2%

Flow control can be controlled either be software or hardware. The hardware

handshaking is always active, so if you want to use software exclusively you

will need to wire a cable to set the handshaking lines high at all times - ie.

pins 5, 8 and 9 together on the Z88 D-connector.

An external device can ask the Z88 to stop sending by either sending an XOFF

character or by de-asserting (bringing low) the CTS line. With software flow

control, transmission can only stop when the XOFF character has been processed,

so there is potentially a slight delay in response, while previously sent

characters are read. With hardware control, transmission stops on the next

character boundary. Transmission is resumed on the receipt of an XON character,

in the case of software control, or be re-asserting CTS, ie. bringing it high.

The output buffer is around 95 bytes long.

If software control is used, the Z88 will send an XOFF to an external device

once the receive buffer is more than half full. Characters will continue to be

received until there are only 15 character spaces left in the buffer. At this

point an XOFF will be sent for every character subsequently sent by the

external device. If the receive buffer overflows then data is lost forever. The

Z88 will send an XON when the receive buffer has been cleared to less than a

quarter full. Under hardware control, the Z88 will de-assert (bring low) RTS

when the receive buffer is less than half full and the re-assert (bring high)

when it has become less than a quarter full. The input buffer is around 127

bytes long.

NOTE: When working with software handshaking the performance of the serial port

is seriously reduced. When working at speeds of 9600 baud or above, the

software handshaking will not work unless there is an intercharacter gap, ie.

some time is left between the sending of each character. The gap allows the Z88

to catch up with serial operations, and without it the machine may loose

characters.

%H2%Serial Port Lines%H2%

The following table describes the 9-pin D-connector of the Z88:

%H4%Pin%H4%

1 -

2 TxD

3 RxD

4 RTS

5 CTS

6 -

7 GND

8 DCD

9 DTR

NOTE:

DTR is high when the machine is awake. The machine is always awake when the the

screen is active, but even if asleep, the machine will wake every minute or so

to carry out various housekeeping tasks, such as checking for alarms, and at

these times DTR will go high. Pin 1 will show a signal if there is power

available to the machine.

%P0%

%CO:B,12,68%

75, 300, 600, 1200, 2400, 9600, 119200, 38400.

Send and receive rates are independent.

None, Odd, Even, Mark, Space. Note the Z88 generates the

appropriate parity, and will ignore, but not strip, the parity of

incoming data.

Hardware handshaking with or without XON/XOFF.

Output: 10 minutes is the default value

Input: infinite is the default value

Output: 95 bytes

Input: 127 bytes

IX = handle returned from GN_OPF of ":COM.0"

L = reason code

Other register usage depends on L

Depends on reason code

IX will always be unchanged

%H4%Signal type%H4%

unswitched +5V at 10mA

transmit data

receive data

ready to send

clear to send

not used

Ground

data carrier detect

switched +5V at 10mA

%CO:C,12,56%

%H4%Action%H4%

Hard reset the serial port

Soft reset the serial port

Interrupt entry point (System use only).

Get byte from serial port

Put byte to serial port

Status enquiry

Flush Tx (transmit) buffer

Flush Rx (receive) buffer

Set timeout

%CO:D,12,44%

%H4%direction%H4%

output

output

input

output

input

input

output

%CO:E,12,32%%CO:F,12,20%%CO:G,12,8%