MSF.COM - A Resident MSF Clock

MSF.COM is a resident utility (TSR) which decodes data received from the MSF

(60  kHz)  transmission  from Rugby and sets the PC's  date  and  time  very

precisely.   The time reference used by MSF is derived from an atomic  clock

at the National Physical Laboratory.  MSF.COM uses up only 1Kbyte of memory.

MSF.COM  requires a feed of data from a suitable MSF receiver (such as  sold

by  Maplin or Galleon).  The data must be supplied to the PC via  a  one-bit

input port.  If your PC has an unused games port this is ideal, but a  spare

'handshaking' bit on a parallel or serial port is also suitable (see later).

MSF.COM defaults to input from bit 7 of the games port (address 201H), which

is  the  fourth  'fire button'.  In this case the signal  ground  should  be

connected  to pin 5  and the data to pin 14 of the joy-stick connector  (15-

pin  D-type).  The signal must be at TTL levels (low from 0 to  +0.4  volts,

high from +2.4 to +5 volts) but the data polarity is unimportant as  MSF.COM

automatically adapts to this.

To install MSF.COM the following command should be entered:

MSF [/p:port] [/b:bit] [/s]

where the square brackets indicate optional parameters, as follows:

/p:port   Sets  the (hexadecimal) port address from which the  MSF

data is to be read.  For example, /p:379 causes data  to

be  input from the LPT1 printer port  handshaking  pins.

If omitted, defaults to /p:201 (the games port).

/b:bit    Sets the bit number (0-7) from which the MSF data is  to

be read.  For example, /b:3 causes data to be input from

bit 3 on the selected port.  If omitted, defaults to 7.

/s        Causes  a sound to be generated whenever the PC's  clock

is set by the MSF data.  Defaults to no sound.

Once  MSF.COM  has  been  installed, the current MSF  status  and  the  most

recently  received MSF data can be displayed by entering the command MSF  on

its  own.   The MSF status is indicated as 'good', 'poor'  or  'fail'.   The

'fail' message indicates either that no data at all has been received for 15

seconds  or  that no valid (i.e. error checked) data has been  received  for

four  minutes.  The 'poor' message indicates that the MSF data is  distorted

or noisy; in this case you may get an improvement by re-siting the  receiver

(MSF  receivers  are especially sensitive to  interference  from  electronic

equipment).   Note  that  the displayed MSF date and  time  are  not  error-

checked;  only  when  error-free and consistent data are  received  for  two

consecutive minutes is the PC's clock set.

If you are unsure whether the MSF transmission is on air (it is closed  down

for  maintenance  on the first Tuesday of each month,  and  occasionally  at

other times) you can telephone for a recorded message on 020 8943 6493.

MSF.COM  can be removed from memory by issuing the command MSF /r.  This  is

necessary before any parameters (e.g. port address) can be changed, as these

are set only on initial installation.

The command MSF /? or MSF /h causes a brief description to be displayed.

If  you  wish to have MSF.COM normally installed,  the  appropriate  command

should  be  placed  in  your AUTOEXEC.BAT file.  If you  run  Windows  3  in

enhanced mode then MSF.COM should be started from the WINSTART.BAT file  (no

special measures are necessary if you run Windows in real or standard mode).

In enhanced mode, you may find that MSF.COM works better if you add the line

TimerCriticalSection=1 to your Windows SYSTEM.INI file.

If  you receive the message  "Hardware timer in use: cannot install MSF TSR"

this  means that another utility has re-programmed the hardware timer  'tick

rate'  to a non-standard value.  In this event it may be possible  to  solve

the problem by installing MSF.COM before the other utility.

If you are running Windows 95 or Windows 98 simply place the appropriate MSF

command in your  AUTOEXEC.BAT file.  Depending on  your system configuration

and the input port you choose to use,  you may find that Windows incorrectly

allocates the port and prevents MSF from working.   To check this, enter the

Control Panel (START - Settings - Control Panel) and double-click the System

icon. Select Device Manager and highlight the device into which the MSF data

is being fed (for example, if you are using the games port click on the plus

sign against "Sound, video and game controllers" then highlight the Joystick

device).  Click Properties then select Resources; in the 'Resource settings'

section ensure that the port you are using (e.g. address 0201 in the case of

the games port) is within the range given.   If it is not,  uncheck the "Use

automatic settings" box and select "Basic configuration 0000" from the drop-

down menu.   Click on OK;  Windows will warn you that you have made a manual

adjustment to the resource settings.  Click Yes to confirm.

Some alternative input ports which are suitable for MSF.COM are as follows:

Port          Signal          Address        Bit           Pin(s)

Games     Fire button A         201           4              2

Games     Fire button B         201           5              7

Games     Fire button C         201           6             10

Games     Fire button D         201           7             14

Games     Ground                                             4, 5

LPT1      Acknowledge           379           6             10

LPT1      Busy                  379           7             11

LPT1      Paper end             379           5             12

LPT1      Select                379           4             13

LPT1      Error                 379           3             15

LPT1      Ground                                            18-25

LPT2      As LPT1 but use port address 279

COM1      Clear to send         3FE           4              5

COM1      Data set ready        3FE           5              6

COM1      Ring indicator        3FE           6             22

COM1      Carrier detect        3FE           7              8

COM1      Ground                                             7

COM2      As COM1 but use port address 2FE

COM3      As COM1 but use port address 3EE

COM4      As COM1 but use port address 2EE

Note  that  the inputs to the Games and LPT ports must be at  TTL  (5  volt)

levels whereas the inputs to the COM ports must be at RS-232/V24 levels.

MSF.COM is Freeware.  You may freely copy and use the program so long as  it

is  not  altered  in any way and is distributed along  with  the  associated

files.  MSF.COM is the Copyright (C) 1992 of R. T. Russell; any comments  or

reports of problems would be appreciated.  I can be contacted as follows:

Email:          info@rtrussell.co.uk

Web:            http://www.rtrussell.co.uk

Facsimile:      01375 892439

Richard Russell, 3rd March 1992. Amended 23rd October 1999, 27th October 2001